Adra (Tai) Cyf

Volume 3 – Specification

in relation to a procurement for the provision of Drain Clearance, Investigation and Associated Services for Adra (Tai) Cyfyngedig

SPECIFICATION

Introduction

Unless otherwise specified, all defined terms in this Specification have the same meaning as set out in Adra’s Standard Terms & Conditions to which this Specification shall be annexed (the Agreement).

The Services

Adra are seeking to appoint an appropriately qualified and experienced Contractor who is able to provide professional services to individual properties upon request in order to unblock and identify reasons for drain blockages including the use of camera surveying services. These properties will be located across the north Wales region.

The Services shall comprise a full range of drain clearance, investigation and associated services, whether directly or via the use of appropriately qualified sub-contractors holding the relevant licenses, which shall include (but shall not be limited to):

    - Rodding;

    - Jetting;

    - CCTV Surveys;

    - Minor repair works (as required depending upon the circumstance);

    - Sustainable drainage maintenance

- as further specified within this specification (and as may be amended from time to time) to the standards stated, and which can be reasonably inferred from the requirements included in this document.

The Contractor shall provide a range of comprehensive drain clearance, investigation and associated services to sites, which ensures a clean, healthy and safe environment and presents a positive image to all stakeholders and enhances Adra’s reputation for the maintenance of its properties and communal areas.

The Contractor will:

Provide the services as set out in this Specification (the Services) and as set out in the Agreement; and

Provide the Services in accordance with all service levels and key performance indicators as set out in this Specification and in the Agreement.

The list below is not exhaustive, nor is it a guarantee of actual workload, and this Specification may be updated and amended by the written agreement of the parties from time to time during the term of the Agreement.

Contractors should note that where it is necessary to terminate any part of the provision of these services, such termination shall be in accordance with the provisions of clause 31 and Schedule 6 of the Volume 2: Terms & Conditions.

Full details of the Services required for each individual Appointment will be contained within each Appointment.

- Other Contractors

  - On occasion, other works may be undertaken by other contractors at the same time as the contractor is delivering services under this contract. In such circumstances, the contractor must work in conjunction with, and liaise with, Adra and any other contractor engaged in any other works.

- Call Out process

  - The contractor is expected to have a robust call out service for normal working hours and out-of-hours work. This call out process will be required 24 hours a day, 365 days of the year including all bank holidays, it is expected that the level of service remains the same at evenings, weekends and bank holidays.

  - For the purposes of this contract, normal working hours are between 08:00 and 18:00 Monday to Friday.

  - All reactive call outs will be placed via Adra’s Assets & Infrastructure Directorate, Contact Centre or out of hours management centre. For the avoidance of doubt, other individuals within Adra are not authorised to instruct the contractor directly and you are not authorised to take instructions from anyone other than those mentioned above.

  - Should an instruction be received from any other party, the contractor is required to contact an authorised representative of Adra’s Asset Management Team (as identified in Schedule 5 of Adra’s Standard Terms and Conditions) for authorisation.

  - Adra will require the Contractor to be able to attend a property suffering from a blockage or drainage problem within a maximum of 24 hours notification being made by Adra to include evening weekends and public holidays.

  - The above timescale may be adjusted by or on behalf of Adra before the issue of any work order to the contractor because of (but not limited to) such factors as:

    - Issues such as the vulnerability of the tenant;

    - The nature of the works;

    - The volume of work orders being issued at the time;

    - Any other circumstances Adra deem necessary.

  - Works are deemed to have an emergency priority where they are required to resolve a situation:

    - That is actually or potentially dangerous.

    - That poses a serious risk to the health and safety of any persons or to the structural stability of any property;

    - Where immediate action will prevent serious damage to property.

  - Where works are deemed to have an emergency priority, the Contractor will attend site to undertake initial investigation within 4 hours of the request being made and attempt to unblock the system and remedy the issue.

  - Requests for out of hours emergency work will be received from Adra’s call handling facility. The tenant will be advised that contact will be made by the contractor prior to a crew being dispatched.

  - Within 1 hour of receiving an order the Contractor must make a telephone call to the Tenant to:

    - advise of the expected time of arrival

    - ascertain the exact nature of the works, and

    - advise the Tenant on any H&S implications.

If the Tenant is non-contactable by telephone. The Contractor must leave a voicemail (if such facilities are available) and must inform Adra’s call handling facility that they have been unable to contact the tenant. If tenant is not contactable the contractor should not attend, and should advise Adra

  - The contractor must complete work orders in line with the timescales set out. If it becomes apparent that the contractor is unable to respond within the relevant time period, they must inform Adra immediately, giving reasons for this.

  - If the Contractor is unavailable to respond within these timescales, then Adra may procure the services outside this contract.

  - In the event that the services are not provided within the timeframe specified in this contract, Adra reserves the right to charge the Contractor for the difference between the contracted rates of the contractor and the price paid for the services provided by another contractor procured in line with 5.13 above.

  - All call outs placed with the contractor must be supported by either a Adra purchase order number or Adra Service Request number. This purchase order or service request will contain critical job information such as address and job description. Should a call out occur out of hours and Adra’s representative does not have access to generate a Purchase Order number or Service Request number the contractor is to record the callers’ name.

  - No job should be accepted without a purchase order number or service request number being provided as this could delay in payment when completing invoices etc. Should the call be place out of hours and the member of staff is unable to provide a purchase order number the contractor is to record the caller’s name and chase Adra’s Asset Management Team the following day for a purchase order number or service request number.

  - All completed remedial works are to be covered by a signed engineers job sheet, which must contain the below information as a minimum but not be restricted to the below. Failure to have the sheet sign may delay in payment:

- Purchase Order Number

- Address

- Callers ID

- Time on and off site

- Type of asset affected

- Blockage description i.e. User error or miscellaneous items found down the drain

- Equipment used

- Staffing / resources

  - The Contractor’s staff must do their upmost to promote and enhance the image and reputation of Adra through providing a Tenant-centred, safe and efficient service.

  - The Contractor shall employ, provide and supervise sufficient suitably trained and/or experienced management and operatives, including sub-contractors (other than apprentices and trainees) to provide the drain clearance, investigation and associated services as outlined within the contract documents. The staff shall be made aware of and comply with the requirements of each site including, inter alia, the operations taking place, security and similar requirements.

  - The Contractor will be expected to maintain a structure that will support the successful delivery of this service. The Contractor will be expected to provide Adra with details of their structure at the outset and will be expected to keep Adra informed of any staff / structure changes that may affect the delivery of the service.

  - Staff are required to be smartly presented and identifiable as employees of the Contractor. Each site operative shall carry a personal identification card which shall include a photograph of the individual and should wear protective clothing as required whilst working on Adra premises. Such badges or clothing shall be kept clean and the identification kept legible and shall be replaced when worn, torn, damaged or become un-presentable for whatever reason.

  - Training in relation to the management and delivery of this contract is essential and brief details on Contractor’s training policy should be provided in the tender submission. New employees must be given instruction in health and safety practices and procedures before commencing active duties and full training within one month of the commencement of their employment.

  - The Contractor shall be required to nominate a Contract or Regional Manager who shall be responsible for managing all aspects of the Agreement. These individuals are named at Schedule 5 of the Agreement and shall not be changed or removed by the Contractor without prior consultation with Adra. Supervisors will be responsible for day-to-day supervision and operational issues, and the Regional Manager shall be responsible for representing the Contractor, and managing the Adra’s account at a more strategic level and at regular review meetings.

  - All operational service teams shall include an appointed supervisor responsible for the following elements:

    - Undertaking regular work inspections to ensure ongoing compliance with all relevant procedures, work schedules and performance standards.

    - Administering all relevant quality control systems.

    - Demonstrating attendance and time on site.

    - Identifying and recording general site-related defects or maintenance problems that require attendance (e.g. uneven paving slabs, damaged fencing, retaining walls, down pipes or air-vents), within, or to the perimeter or the specified sites.

    - Reporting back progress and issues to Adra’s Contract Manager.

    - Ensuring that delays in delivering services are communicated effectively to all key staff, Adra and its tenants.

  - Contractors will be expected to maintain and upgrade their information technology systems to exploit the potential of new technological developments where reasonable.

- Working in occupied properties

  - Contractor’s responsibilities in relation to occupied Properties

    - The Contractor must give reasonable notice to the Tenant and, where appropriate, adjacent occupiers of their intention to commence Works in or adjacent to Occupied Properties.

    - The Contractor must ensure that no Work starts or continues in any Property until all practicable steps have been taken to prevent danger to persons employed or living in the Property at the time. This includes (but is not limited to) danger from any:

      - live electric cable or apparatus;

      - plumbing works;

      - exposed asbestos; or

      - any other hazard which is liable to be a source of danger.

    - Where the Property is occupied by an elderly, frail or ill Tenant or a Tenant with a disability the Contractor must take due care and consideration for such person in the execution of the Works.

    - The Contractor must agree with Adra in advance where because of the nature of the Works and/or the Tenant’s needs, the Tenant cannot remain in occupation during the Works. This is to apply only in exceptional circumstances.

  - Contractor’s obligations to protect and/or remove furniture

    - The Contractor must carefully move all furniture, fittings, apparatus, carpets, and the Tenant’s personal possessions as necessary for the Works. The Contractor must agree the condition of such items with the Tenant before moving them and must notify Adra’s Representative of any failure to agree this.

    - The Contractor must cover all furniture, fittings, apparatus and carpets that are not moved with spot cloths and effectively protect them from dirt and splashes.

    - On completion of the Works, the Contractor must replace and refit all such furniture, fittings, apparatus and carpets in their original positions, to the Tenant’s satisfaction.

    - Under no circumstances must gas appliances or gas pipework be interfered with by Staff not on the Gas Safe Register. Any damage caused to gas appliances or pipes by the Contractor during the execution of the Works must be reported to Adra’s Representative immediately. The Contractor must reimburse Adra for the costs of having such damage rectified.

    - The Contractor must make good any damage which may have been caused to any property to the satisfaction of the occupier. For the avoidance of doubt, this includes both properties owned by Adra and properties that are privately owned.

    - The Contractor must not remove any item from the Property except rubbish or unused Materials accumulated or supplied as part of the carrying out of the Works.

    - The Contractor must settle any claims for damage to any personal property directly with the affected party.

  - Access to adjoining properties

    - The Contractor must ensure that no Staff, Materials or Equipment (including scaffolding) trespass on any properties adjoining the Properties to which Works are undertaken.

    - If the execution of the Works requires Staff to enter onto any adjoining property, the Contractor must first obtain permission from the adjoining landowner.

    - The Contractor shall comply with all reasonable requests of landowners, owners and householders particularly regarding means of access. Any request which the Contractor regards as unreasonable should be referred to Adra’s Representative for decision.

    - The Contractor must inform Adra’s Representative if they are unable to obtain such permission and Adra’s Representative will endeavour to arrange access to the adjoining property (using the Party Wall Act if necessary).

  - Party walls

    - The Contractor must notify Adra’s Representative where access is required to an adjoining property to undertake the Works or where the Works are adjacent to walls, fences or other party boundary situations and require either permission from the adjoining owner or the giving of notices under the Party Wall Act 1996.

    - Following a notification under Paragraph 7.4.1, Adra’s Representative will either:

      - Instruct the Contractor to proceed with the Works as originally ordered; or

      - temporarily cancel the Order (without charge) until Adra has either:

        - obtained permission from the adjoining owner to carry out the Works; or

        - served a party wall notice on the adjoining owner and either obtained their consent to the Works being carried out or obtained a party wall award in relation to them.

    - If instructed to proceed as originally ordered, the Contractor must complete the Works within the response period for the order (as extended by the time the Contractor was waiting for the Instruction from Adra’s Representative). In such circumstances, the Contractor is not to be liable for any resulting breach of the Party Wall Act 1996.

    - Subject to Paragraph 7.4.3, the Contractor must always be mindful of and must ensure it does not place Adra in breach of the Party Wall Act 1996 and the obligations it imposes on Adra towards owners of adjoining property when works are undertaken on, over or adjacent to walls, fences or similar party boundaries. The Contractor must comply with the terms of any permission given by the adjoining owner of any party wall or the terms of any party wall award when carrying out the Works.

  - Environmental sustainability

    - The Contractor must use all reasonable endeavours to use working methods, Equipment, Materials and consumables that minimise environmental damage in the Works.

    - The Contractor must:

      - promote the objectives of Adra’s Environmental Policy; and

      - support and assist Adra in achieving those objectives.

    - The Contractor must provide Adra with a copy of their Environmental Policy or Statement of Intent towards environmental issues. These must be compatible with Adra’s Environmental Policy objectives. The Contractor must be able to demonstrate, if required, that they operate in harmony and compliance with Adra’s Policy.

  - Protection of existing and adjoining buildings and gardens

    - The Contractor must rectify any damage caused to the satisfaction of Adra’s Representative and, where applicable, the Tenant.

  - Services and security at the end of each Working Day

    - The Contractor must ensure that at the end of every Working Day the Tenant and/or other occupants of the Property being repaired have full facilities available to them for lighting, heating, power, drinking water and sanitation, together with washing and cooking facilities.

    - Where a Property is unoccupied on completion of the Works or at the end of each Working Day, the Contractor must secure that Property, ensuring all doors and windows are locked and any temporary door and window coverings are reinstated as necessary.

- No access procedure

  - If on arrival and after confirming attendance, the Contractor is unable to gain access to carry out any required Works, the Contractor must notify Adra’s Representative immediately.

  - If the Contractor’s Staff cannot gain access to the Property on the initial visit, the Staff member must leave a card requesting the Tenant to contact the Contractor to arrange an alternative time for the Works.

  - If the Tenant does not respond to the Contractor within 24 hours, the Contractor must notify Adra’s Representative and must then try and contact the Tenant by telephone call. If the Tenant is non-contactable by telephone the Contractor must leave a voicemail (if such facilities are available) requesting the Tenant to contact the Contractor to arrange an alternative time for the Works and must inform Adra’s Representative.

  - If the Tenant still does not respond to the Contractor within a further 48 hours The Contractor must notify Adra’s Representative. Following notification from the Contractor, and subject to the Contractor having complied with Paragraphs 8.1 to 8.4 (inclusive), either:

    - Adra will make the access arrangements directly with the Tenant; or

    - Adra’s Representative will cancel the Order.

  - As well as following the “no access card system”, the Contractor must provide details of all access problems and abortive calls to Adra’s Representative.

- Staff Safety Register and ‘Do not visit alone’ policy

  - Adra will give advance notice to the Contractor where Adra’s HSp01 Staff Safety Register and / or QPg13 Warning Notes applies in relation to a particular Property.

  - Where a notice has been given under Paragraph 9.1, the Contractor should follow the instructions contained within the document (for example, ‘Visit in pairs’, ‘use lone working device’, and so on).

- Welsh Water Assets

  - on arrival if the issue is located on an asset that is the responsibility of Dwr Cymru/ Welsh Water (DCWW), the contractor is to restore flow and then inform Adra’s’ representative. Adra will then report the fault to DCWW.

- Traffic Safety Measures

  - The Contractor shall carry out the works in such a manner as to cause as little interference as possible to the flow of traffic and pedestrians and shall make all necessary arrangements with the appropriate Authorities except where detailed otherwise in the Contract.

  - The Contractor must arrange that all possible means of existing pedestrian and vehicular access and rights of way are maintained for residents, visitors and the public during the execution of the works. The whole of the works must be carried out so as to cause the minimum inconvenience to Adra’s tenants and the general public, and the Contractor must provide safe passage to pedestrians and vehicular traffic at all times.

  - All operations should, be carried out so as not to interfere unnecessarily or improperly with the public convenience or improperly restrict use of public or private roads and footpaths to or of properties and the Contractor shall save harmless and indemnify Adra in respect of all claims, demands, proceedings, damages, costs, charges and expenses whatsoever arising out of or in relation to any such matters.

  - The Contractor shall not commence any work until all traffic safety measures and controls necessitated by the works are fully operational and for the duration of the works ensure that all signs, barriers, cones, signals etc. are maintained and correctly positioned in a clean and serviceable condition at all times.

  - All work undertaken on the public highway shall be in accordance with Traffic Management “best practice”, to conform to Chapter 8 of the Traffic Signs Manual published by HMSO or any amendment thereof for the time being in force and the Contractor shall provide, erect, reposition and remove on completion all necessary signs, barriers, cones and signals etc.

  - All traffic signs and signals shall be in accordance with the requirements of the Traffic Signs Regulations and general directions (Statutory Instrument No. 1519 -1994) and subsequent amending regulations.

  - Access to some of locations may be restricted in terms of the size of the vehicle / machine that can be used. It is the responsibility of the Contractor to acquaint themselves with the access to each location, and any other problems affecting access relating to all locations. The Contractor shall avoid, where possible, vehicle encroachment onto grass and other areas, except where vehicle encroachment is absolutely necessary for the effective performance of the contract. Any damage caused due to encroachment onto grass and other areas by the Contractor’s plant or vehicles shall be made good at the Contractor’s expense, and within time limits stipulated by Adra.

  - The contractor must ensure that that its staff do not illegally park any motor vehicle on any public or estate road or footpath, fire-path, in any pre-allocated car parking space or on any grassed areas.

  - Where the volume of traffic or parked vehicles makes it impracticable or hazardous to carry out the works during normal working hours, the Contractor shall apply to Adra’s Representative for permission to work outside of normal working hours.

- Plant and equipment

  - The Contractor should note that they will be responsible for the provision of all equipment, machinery, tools, plant, vehicles, implements, supplies and consumables necessary for the proper execution of the service, and any such costings should be included as part of the contractors submitted price framework.

  - The Contractor will be responsible for making his own arrangements for the supply of water and/or electricity and pay all charges for the supply including the provision of mobile generators where necessary.

  - The use of any standpipe which has not been approved by the water undertaker is expressly forbidden.

  - All tools and machinery, plant, equipment and vehicles employed on the works must be of a suitable type and construction, safe in use, properly maintained and must conform to all relevant legislation and British Standards.

    - All plant and tools shall be provided with the correct guarding and interlocking devices. Any available safety devices shall always be used when operating any tools and/or plant.

    - All powered items of equipment or machinery must have an in date Portable Appliance Test (PAT) sticker on its plug prior to attending site, the plug, lead and casing must be visually inspected prior to every use. Any defects identified during this visual inspection will mean that the item of plat or equipment cannot be used in the delivery of these services.

    - Any plant and tools shall be satisfactorily maintained, and records of maintenance shall be available to Adra upon request.

    - Machinery must not be used by the Contractor in close proximity to children, and if in the Contractor’s opinion a potentially dangerous situation exists, the Contractor will remove his machinery from the premises, and notify Adra.

    - All guards, safety flaps and safety devices must be used in their correct position at all times.

    - All chemicals must be kept safe under lock and key when not in use. Empty containers must be removed from site, and disposed of in a manner approved in the relevant regulations.

    - All equipment shall be suitable for the relevant operation to be carried out and shall only be used in compliance with the manufacturer’s recommendations and current Health and Safety legislation.

  - The Contractor shall be responsible for all costs relating to the recovery of or damage to any plant and equipment used in the delivery of these services.

  - If excavation is necessary this shall be carried out only after the express consent of Adra.

- Dealing with Flows and Blockages

  - The Contractor shall allow in his rates and prices and be responsible for dealing with flows as far as is reasonably possible. Such arrangements shall be subject to Adra approval.

  - If the Contractor anticipates requiring over-pumping, night or weekend working, he shall request authorisation from Adra’s Representative. In the case of emergencies where flood water is building up then pumping will be required to remove the risk of properties flooding, pumps to be maintained 24hrs until surface / storm water drainage infrastructure can be repaired.

  - In the case of a blockage, the Contractor shall inform Adra on the nature of the blockage. The contractor should suggest options for remedial action and Adra’s Representative shall instruct the Contractor on how the blockage should be dealt with.

  - Where there is sewage leakage into a property or it’s curtilage then immediate action will be required to investigate and clear the blockage.

  - In the event of a sewer collapse within the property curtilage, confirmation of ownership must be completed. Then if owned by Adra excavate and repair (in the event of complex repairs carryout a make safe and refer to Adra). Where the blockage emanates on infrastructure not owned by Adra then the contractor is to report the defect to Welsh Water (and inform Adra).

  - In emergency situations, the contractor is permitted to complete repairs totalling a maximum cost of £250.  On completion, the invoice is to reference the work carried out, the materials used and their quantities. The costs are to replicate the costs indicated on the agreed schedule of rates. Adra will only pay for work that has been carried out on assets owned and managed by Adra, if there is any doubt on the ownership of the assets, the contractor must contact Adra’s representative to confirm ownership prior to carrying out the works.

  - Works required to resolve an emergency include:

    - Situations that are actually or potentially dangerous.

    - Situations that pose a serious risk to the health and safety of any persons or to the structural stability of any property.

    - Where immediate action will prevent serious damage to property.

  - The contractor will be required to clear up any area which has been contaminated with wastewater from blockages. This is to include the use of wet vacuums and other devices which may assist in reducing the impact of the blockage. This is to be covered using the tendered schedule of rates issued as part of this specification.

  - Further details in relation to remedial work can be found at Paragraph 18 below.

- Cleaning existing Drainage Systems

  - The Contractor shall take measures when clearing blocked drains to ensure that adjacent water courses or groundwater via soakaways, will not be contaminated. Contamination includes mud or soil being washed or flushed into streams as well as other more obvious contaminants including diesel fuel, oil and chemicals.

  - Initial attempts to clear blocked drains prior to jetting, shall be undertaken by hand rodding and any debris and silt removed by the operation shall be removed off Site. The Contractor shall report any localised blockages that cannot be cleared by rodding to Adra.

  - Cleaning of Gullies, Manholes, Catchpits, Soakaways and Oil Separators

    - At each chamber, all mud and vegetation in the vicinity of the chamber likely to impede the flow of water shall be removed and within 1.00 metre of each approach, back to the surrounding kerb line or a line 75 mm behind the back face of the cover or grating. After lifting the cover or grating the chamber shall be cleansed of all water, detritus, debris, and silt, refilled with clean water to the outlet level, and all covers and gratings replaced and evenly bedded.

    - Cleaning of chambers shall be by mechanical means. The vehicle used to clean existing chambers shall be equipped with a 125 mm dia gulley arm with boom jets, an exhauster with a minimum output of 5.95 cum/min and minimum 5455 litres capacity. Sediment, detritus, and liquor from the chamber shall not be permitted to discharge into the outlet. This may be achieved either by plugging the outlet during cleaning, or by simultaneous jetting and abstraction of liquor from the chamber using a tanker fitted with low pressure, high volume water jets around the boom.

    - Gullies and chambers not cleaned for whatever reason, blocked connections and broken or cracked covers, gratings or frames shall be marked to aid subsequent identification.

    - The Contractor shall dispose of all surplus water, debris and arisings from the works off Site at a licensed tip.

    - All covers which have been removed for cleaning operations shall be replaced.

    - The Contractor shall report any damage and defects to the drainage system or components to Adra each day, or immediately if considered a safety hazard.

  - Low Pressure High Volume Jetting of Drainage Systems

    - The pipe material of the drainage systems to be cleaned shall be identified prior to commencement of cleaning. Where the material is unknown and cannot be verified by either visual inspection, without recourse to man-entry, or from drainage records, or damage to the pipeline is the suspected cause of a blockage, cleaning shall proceed on the basis of use of the lowest pump pressure stated in Table 1.

  - Jetting of Piped Drainage Systems

    - Cleaning shall take place from downstream of the blockage in an upstream direction. The pipe shall be plugged below the jetting point to prevent contaminants flowing to the watercourse. The cleaning shall take place from a chamber, although it may be necessary to make a temporary excavation into the pipeline in some circumstances.

    - Should the Contractor find that it is not possible to jet in an upstream direction, then the pipe shall be plugged below the blockage and the jetter then used to “back jet” to a suction hose.

Table 1: Maximum Recommended Pump Pressures

| Material | Maximum pump pressure (bar/psi) |
| --- | --- |
| Unknown/structurally damaged | 80 / 1200 |
| Brick/masonry | 100 / 1500 |
| Plastics - Structural Wall | 127 / 1900 |
| Plastics - Solid Wall | 127 / 1900 |
| Concrete | 340 / 5000 |
| Clay 340/5000 | 340 / 5000 |

    - The Contractor shall select a jet head that is appropriate for delivering the maximum jetting pressure for the pipe material and also the minimum volume of water for the appropriate pipe diameter stated in Table 2.

    - The jet head shall be of a configuration such that the number and direction of jets are adequate to be directed to the sides and soffit of the pipeline to prevent debris passing over the top of the jetting head.

Table 2: Minimum Jetting Flow Rates

| Pipe Diameter | Minimum jetting flows |
| --- | --- |
| <230 mm | 156 l/m |
| 450 mm | 270 l/m |
| 900 mm | 300 l/m |
| 1600 mm | 342 l/m |

    - The Contractor shall provide suitable jetting equipment that shall include the provision of the following:

      - operation at a maximum rate of 850 l/min at 150 bar/2250 psi;

      - liquid ring vacuum pump operating with air flows of 4000 m3 per hour, with a working vacuum of not less than 90%;

      - a minimum of 20 m of 100 mm diameter suction hose for cleaning pipes up to 300 mm diameter and 150 mm diameter suction hose for cleaning pipes of diameter greater than 300 mm;

      - minimum of 150 m of 25 mm or 38 mm diameter jetting hose and a capability of jetting up to 200 m;

      - automatic and continuous water recycling;

      - silt, sand and rubble to be de-watered prior to discharge at licensed tip.

    - The Contractor shall ensure that no damage occurs to the manhole chamber or pipeline during insertion of the jetting equipment.

    - The jetting head shall be propelled through or over the blockage and then the hose pulled backwards enabling the force of the jet to break up the blockage material. The minimum force necessary to penetrate the blockage shall be used but the pump pressures should not exceed those stated in Table 1.

    - The hose shall be rewound at a rate of between 100 mm and 200 mm per second.

    - There is a possibility that damage may occur to certain pipe materials should the jetting head remain stationary thus the Contractor must ensure that the jetting head never remains stationary for more than 60 seconds.

  - Silt Removal

    - Suitable measures such as stanks or stoppers shall be positioned downstream of the drainage system to be cleaned to minimise the risk of sediment causing contamination of watercourses or soakaways.

    - Wherever practical the Contractor shall use equipment to carry debris over a greater depth than one atmosphere and with a capability to suck liquid.

    - All arisings from the cleaning process shall be disposed of in an environmentally sensitive manner in accordance with current legislation.

- CCTV Operation and Equipment

  - The Contractor shall carry out and record an internal inspection with CCTV equipment when instructed to do so.

  - The Contractor shall provide all necessary equipment, including suitable covered accommodation for viewing the monitor screen, together with personnel experienced in the operation of the Equipment and interpretation of the results.

  - The Contractor shall survey the pipework / culverts listed on the Work Order so as to record all salient features and to discover evidence of their structural and service condition, all in accordance with the format laid down in Section 15.

  - Person entry shall generally be avoided if other appropriate means can be employed. With pan and tilt camera, correct lighting and calibration, pipes up to 1800mm diameter can be inspected using CCTV without access.

  - The Contractor shall provide sufficient vehicles and equipment, including stand-by units, to complete the survey, cleanse, jetting within the contract period stated in the Work Order.

  - The CCTV team shall be equipped with a mobile phone to enable them to contact or to be contacted by Adra’s Representative for the duration of the survey, cleansing and jetting and repair work.

  - Each CCTV survey vehicle shall contain a means of transporting the CCTV camera in a stable condition through the drain under inspection.

  - Where the CCTV camera is towed by winch and bond, all winches shall be stable with either lockable or ratcheted drums. All bonds shall be steel or of an equally non-elastic material to ensure the smooth and steady progress of the CCTV camera. All winches shall be inherently stable under loaded conditions.

  - Each unit shall carry sufficient numbers of guides and rollers such that, when surveying, all bonds are supported away from pipe and manhole structures and all CCTV cables and/or lines used to measure the camera's location within the drain are maintained in a taut manner and set at right angles, where possible, to run through or over the measuring equipment.

  - The CCTV survey operation should be undertaken to minimise the risk of picture distortion.

  - The Contractor shall use a suitable measuring device which enables cable length and camera position to be accurately recorded. At the start of each drain/ culvert length being surveyed, the length of drain or culvert from zero chainage up to the cable calibration point shall be recorded and reported in order to obtain a full record of the drain length.

  - The meter reading entered on to the data display at the cable calibration point must allow for the distance from the start of the survey to the cable calibration point such that the meterage at the start of the survey is zero.

  - In the case of a survey through a manhole where a new header sheet is required, the meterage shall be set at zero with the camera focused on the outgoing pipe entrance. The meterage counter shall start to register immediately the camera moves.

  - At the start of each manhole length a data generator shall electronically generate and clearly display on the viewing monitor and video recording a record of data in alpha-numeric form containing the following minimum information:

    - Automatic update of the camera's meterage position from adjusted zero.

    - Pipe dimensions.

    - Upstream and downstream Manhole/pipe length reference numbers.

    - Date of survey, cleanse, jet and line.

    - Road name/location.

    - Direction of Survey

    - Time of start of Survey

    - Reason for any termination of survey and location of termination

  - The size and position of the data display shall be such as not to interfere with the main subject of the picture.

  - Once the survey is under way, the following minimum information shall be continually displayed:

    - Automatic update of the camera's meterage position adjusted zero.

    - Pipe dimensions.

    - Manhole/pipe length reference number.

    - Direction of Survey

  - All survey footage and reports shall be conveyed through instantaneous means within the timescales laid out within this specification.

  - The Contractor shall have the ability to undertake Sondes surveys.

  - Any further requirement will be detailed in the Work Order.

  - The CCTV equipment must be intrinsically safe so as not to ignite possible flammable sewer gasses

- CCTV Survey Reporting

  - On completion of the survey the Contractor will be required to submit written reports within 3 working days suggesting repair or relining works to pipework as necessary once identification of the reason for the blockage has been identified. The report should be supported by CCTV footage and other evidence to support.

  - The Contractor shall use industry standard software, and shall ensure that any reports, plans, CCTV footage are provided in a format that can be supported by CCG’s IT system.

  - The report shall be validated and presented in accordance with the format laid down in this specification and comprise (as a minimum):

    - A written report and marked-up drawing

    - CCTV footage

    - A list of identified defects or points of interest, with references to CCTV footage and still photographs as required

    - Recommendations on required solutions or interventions

    - Recommendations as to which intervention or suite of interventions would deliver the greatest benefit in terms of conveyance of flow without detrimental impact downstream.

  - The contractor shall provide Adra with web portal access to enable Adra’s representative to view any report, plans, CCTV footage, and so on.

- Manhole Covers / Gullies / Inspection chambers

  - The Contractor shall, except where otherwise specified, be responsible for lifting and refitting manhole covers, preliminary line threading, clearance of all equipment, etc. from site on completion of the works.

  - The Contractor shall follow best practice and employ safe working when undertaking work in confined spaces, and shall ensure that only “Confined Spaces Trained” personnel with appropriate equipment are permitted to enter confined spaces e.g. manholes.  The Contractor will be required to provide evidence that any such personnel are trained and that their training certification is up to date.

  - If, either prior to the commencement of works or during the course of the works, the Contractor observes any damage to manholes which are a danger to a third party he shall report the defect to Adra immediately.

  - If a manhole cover is damaged whilst being lifted the Contractor shall provide temporary protection for pedestrians and for vehicular traffic and shall immediately inform Adra so that arrangements can be made for replacement. If the contractor damages the manhole cover, then they would be responsible for supply and installation of a replacement which shall be of equal or superior standard to the damaged manhole cover, at their own cost.

  - If a Buried chamber or rodding point is encountered, the contractor shall inform Adra’s representative immediately. Adra’s representative shall then make a decision and provide further instruction, if deemed necessary, to raise the chamber cover to the surface.

- Remedial Works

  - As part of this contract the successful contractor will be able to undertake routine remedial repairs / apply preventative measures. Other than in situations detailed in 12.6 – 12.7 above, all remedial works will require authorisation from a Adra representative prior to commencement.

  - The Contractor must carry out and complete the Works under each Order:

      - in accordance with Law.

      - in accordance with Good Industry Practice.

      - in accordance with all applicable Standards.

      - in accordance with Adra’s Policies as set out in the Contract Details.

      - to the standards of workmanship and using Materials as set out in the Specification.

      - in accordance with any other specific requirements for Works carried out under this Contract; and

      - to the reasonable satisfaction of Adra’s Representative.

  - To the extent the standard of any works has not been specified in this Contract, the Contractor must agree the relevant standard for the works with Adra’s Representative prior to their execution.

  - To the extent that it is necessary to design any aspect of the Works, in preparing those designs the Contractor must use the reasonable skill and care to be expected of an experienced maintenance contractor that is skilled in undertaking works similar to the works.

  - All Materials used in the works must:

      - comply with all applicable standards.

      - be to the reasonable satisfaction of Adra’s Representative.

  - The Contractor must not use any Prohibited Materials in carrying out the Works.

  - Other than in situations detailed in 12.6 – 12.7 above, no works should be undertaken by the contractor without written consent or a valid purchase order number. Works completed without the previous mentioned consent may result in a delay of payment.

  - The contractor’s contract manager is expected to send a written quotation to Adra’s contract manager detailing the work order it relates to, the address, and a description of the works to be undertaken along with a quantified agreed schedule of rates.

  - Upon completion of the works, the contractor shall submit to Adra details of the work carried out, which shall include before and after photographs of work delivered.

- Excavations

  - If excavation is necessary, this shall be carried out only after the express consent of Adra’s representative.

  - In exceptional circumstances, if excavation is required to resolve an emergency situation as detailed in 12.7 above, the contractor is permitted to undertake excavations and complete repairs totalling a maximum cost of £500 in line with 12.6.

  - The contractor must:

    - Follow HSE guidance found in HSG47

    - Adequately fence, shore and protect all  excavation

    - Erect suitable warning notices.

    - Provide stop blocks or similar barriers to stop vehicles from manoeuvring too close to the edge of the excavations.

- Damage to Existing features

- The Contractor shall protect existing buildings, fences, gates, walls and other features. Any damage caused to any property (including, but not limited to, dwellings, vehicles, roads, gardens and pavements) by the Contractor’s negligence or default will be fully reinstated to Adra’s satisfaction at the Contractor’s expense, and within the time limits defined by Adra. Without prejudice to the terms of the contract, Adra may instruct another Contractor to remedy such damage at the Contractor’s expense which he failed to reinstate within the agreed time period.

- Sustainable Drainage infrastructure and components (SUDS)

  - Adra’s new development sites conform to Schedule 3 of the Flood and Water Management act 2010 and include the provision for sustainable drainage.

  - Adra reserves the right to add to the contract for the maintenance of these elements on nominated estates. Each estate is unique in the type and number of elements included in the designed infrastructure.

  - Elements are maintained on monthly, quarterly, six monthly or annually.

  - The estates can be located anywhere within Adra’s operating boundary

  - The contractor is required to service and maintain the respective element as described in the specification table provided (Volume 4- Price framework). Each site is bespoke and is based on the SUDS Manual (CIRIA, 2007).

- Quality Control and Monitoring

  - Work standards and contract compliance will be monitored closely by Adra and the Contractor’s representatives to maintain high standards of service delivery. Adra shall have the right to visit the Contractor on site at any time.

  - The Contractor shall operate a quality control system in accordance with best practice, and with the principles of ISO9001.

  - Work should be carried out in a sympathetic, environmentally sound and sustainable manner in accordance with environment best practice, and with the principles of ISO14001.

  - A periodic inspection of works will be carried out at any time by Adra or its tenants. It is important for Adra to involve its tenants and receive feedback from them in relation to the quality of the service.

  - All services performed under this contract shall be subject to independent inspections by Adra. Payments for services will be only be made if the services have been performed to the required standard.

  - Any defects or noted unsatisfactory work arising within one month of each tasks completion shall be rectified at the Contractor’s expense to the satisfaction of Adra and should be responded to within 48 hours.

  - Contract monitoring will include, but not be limited to:

    - Routine and random inspections and checks of all work carried out.

    - Inspections of vehicles, plant and equipment.

    - Inspection of compliance with livery, uniform and staff ID requirements.

    - Inspection of documented records, including Contractors’ work programme, works and schedules, completed worksheets and all other relevant documentation kept and maintained by the contractor and appointed sub-contractors.

    - Tenant satisfaction.

22.8	The Contractor is to provide a monthly report listing all of the callouts to Adra, which will include all agreed and relevant data.

22.9	All instances of sewer abuse encountered by the contractor is to be reported to Adra.

- Complaints handling

  - The Contractor must deal with any complaints received in a prompt, courteous and efficient manner.

  - The Contractor must keep written records of all complaints received and of the action taken in relation to each of them. Such records must be kept available for inspection by Adra’s Representative at any reasonable time.

  - The Contractor must notify Adra’s Representative immediately in writing of all complaints received and of all steps taken to deal with them.

  - The Contractor must promptly provide all information Adra requires to deal with any complaints Adra receives in connection with the Works or the Contractor.

- Health and safety provision

  - The Contractor must familiarise itself with all current Health and Safety legislation, and the Health and Safety at Work Act 1974. Relevant health and safety legislation can be found listed below, though this list is not exhaustive:

    - The Workplace (Health, Safety and Welfare) Regulations 1992.

    - The Control of Substances Hazardous to Health Regulations 2002.

    - Management of Health and Safety at Work Regulations 2006.

    - Personal Protective Equipment at Work Regulations 1992.

    - The Reporting of Injuries, Diseases and Dangerous Occurrences Regulations 2013.

    - The Confined Spaces Regulations 1997

    - Working at Height Regulations 2009.

    - Provision and use of Work Equipment Regulations 1998.

    - Manual Handling Operations Regulations 1992.

    - The Pressure Systems Safety Regulations 2000

    - the Pressure Equipment (Safety) Regulations 1999

  - The Contractor must take all reasonable precautions to prevent injury to any person, including children, the elderly and vulnerable adults.

  - The contractor must provide warning notices to warn the public of dangerous operations, equipment and chemicals.

  - Warning notices and signs must:

    - Conform to the Health and Safety (Safety Signs and Signals) Regulations 1996

    - Be securely fixed and displayed in a prominent position where they can be properly seen.

  - The Contractor shall employ a safe method of working for the dealing with and disposal of hypodermic syringes and needles found during drain cleansing. No gullies, chambers or the like shall be emptied by mechanical means when hypodermic syringes or needles are visible or known to be present.

  - The Contractor must immediately inform Adra, and in all cases not more than 2 hours after the event of, any accidents arising out of, or in connection with the Works which cause death or personal injury to any person, whether connected with the Works or not, or any damage to property, giving full details and statements of witnesses.  In the event of a claim being made against the Contractor, the Contractor shall promptly report the facts in writing to Adra, together with full details of the claim. The contractor must also inform the HSE though the RIDOR procedures.

  - Adra will notify the Contractor of any Health and Safety issues arising during the contract period. Should the Contractor refuse to rectify any unsafe work practice, the matter will be referred to the Health and Safety Executive.

  - The Contractor will be required to provide details of all materials used in this contract to ensure compliance with COSHH regulations. Any hazardous materials should be brought to the attention of Adra.

- Risk Assessments and Method Statements

  - The Contractor shall provide Adra with all relevant Risk Assessments and Method Statements in relation to the delivery of this contract. Any risks that the Contractor identifies that are not listed below should be brought to the attention of Adra and included in the Contractor’s Risk Assessments and Method Statements. It is critical that the Contractor considers all options when working through a plan of works to create the risk assessment. The points below offer a guide only and are not exhaustive.

    - Residential areas occupied at all times

    - Covid Procedures

    - Manual handling procedures

    - Managing substances

    - Use of potentially faulty work equipment

    - Adverse weather conditions

    - Ensuring occupant’s awareness whilst work is in progress

    - Working at heights

    - Confined spaces

    - Working above entrances and exits

    - Working with vulnerable tenants

    - Welfare provision

    - Noise pollution

    - Personal protective equipment

    - COSHH risk assessments.

  - The Contractor shall make all the appropriate arrangements to ensure that noise levels are reasonable. The Contractor’s attention is drawn to the current legislation and in particular to procedures contained in Part III of the Control of Pollution Act 1974 relating to control of noise on construction and demolition sites.  Attention is also drawn to the British Standard Code of Practise on Noise Control on Construction and Open Sites (BS5228: Part 1:2009).

  - The Contractor shall employ the best practicable means to minimise noise and vibration from within the site and shall pay particular attention to the most appropriate available plant in to minimise the neighbourhood noise (as mentioned in BS5228).  All plant and equipment shall be properly maintained to avoid unnecessary noise and the Contractor shall make the use of suppressors and other mechanical means of reducing noise where these are commercially available.

  - The Contractor shall take all necessary measures to prevent dust arising from the Works causing nuisance to traffic and property.

- Asbestos

  - If during the course of any work associated with the delivery of this contract the contractor discovers the presence of any materials suspected or known to contain asbestos and such materials have not been previously notified to the contractor as containing or possibly containing asbestos, the contractor must:

    - Immediately cease the works.

    - Ensure the works are left in a safe and satisfactory condition with no danger and as little inconvenience as possible to any tenant and / or the general public.

    - Seek instruction from Adra;

    - Respond in accordance with those instructions.

  - Any work involving asbestos that falls within the remit of the of the Control of Asbestos Regulations 2012 may be undertaken only by an asbestos licenced contractor.

- Waste Management

  - The Contractor shall handle, transport, treat and/or dispose of all wastes in a manner suitable for their nature and potential to pollute or cause harm.

  - The Contractor shall be responsible for collecting, handling and disposal of all waste in accordance with all relevant legislation and procedures applicable at the time-of-Service provision.

  - Waste from any drain blockages is to be disposed of from site by the Contractor, at a licenced site and at the Contractors cost. As a result, all costs associated with waste Disposal should be considered when submitting a Tender. Copies of the waste transfer note will be required at invoice stage.

  - Where required, The Contractor shall remove all excavated material and other surplus material from the site leaving the road, footway or verge in a clean and tidy condition.

- Abandonment of works

  - Abandonment of any of the activities in any Work Order may be considered in the following circumstances:

    - inability to maintain picture quality for reasons of asset condition.

    - risk to Contractor’s equipment.

    - further progress impossible due to pipe blockage, root ingress, collapsed pipe, or any other reasonable impediment which hinders progress.

    - inability to locate the manhole.

    - inability to gain access to the manhole once located.

    - risk to Contractor’s operations due to unsafe condition of manhole.

    - inability to carry out the works from the manhole due to blockage, silt or high flow.

    - tenant abuse.

  - In case 26.1.1 the Contractor shall, as soon as reasonably possible, on completion of the manhole length, inform Adra of his inability to maintain picture quality and shall then continue with the works pending a decision on the merits of continuing by Adra.

  - In case 26.1.2 to 26.1.7 inclusive, the Contractor shall photograph the problem area, cease the works and, as soon as reasonably possible, inform Adra should any of his opinion that his equipment is at risk. The Contractor shall then proceed to carry out the survey on the same length from the opposite direction or proceed to the next length of pipework or culvert requiring works as appropriate, or as directed by Adra’s Representative.

  - In the case of 26.1.8, the contractor shall immediately and safely leave the site and inform Adra’s representative.

  - Services will be carried out as far as is reasonably practicable, regardless of weather or climatic conditions, providing this is not detrimental to the service. In the event of exceptionally adverse weather conditions, all or part of the works may be suspended and deferred, until such time as weather, climate or ground conditions improve, for services to resume.

- Portal

As part of the agreement, the contractor is required to have a Web based portal that will assist Adra with the management of the contract. The contractor shall provide Adra with web access to the portal. As a minimum, we would expect the portal to have a Dashboard that allows Adra to easily see;

- Reference of the property

- The status of a Job

- Description of works carried out

- Any associated pictures/video of a Job

- Any future appointments booked

- Work order details, including cost

- Performance

  - Key Performance Indicators (KPI’s) are quantifiable measurements that reflect the critical success factors and goals of the Contract. The successful Contractor will be invited to provide detail of KPI’s that they are currently being assessed against in comparable contracts, however, initially the Contractor will be required to follow and be performance monitored against the initial set of KPI’s as outlined within Volume 6 of these tender documents.

- Meetings

  - Nominated representatives from both parties will meet monthly during the delivery of any Appointment to:

    - Review and monitor performance of the Services.

    - Resolve any concerns of each party.

    - Review the Risk Register.

    - Review any complaints and recognise achievements.

    - Identify improvements which can be made in provision of the Services.

    - Review expenditure against budget

- Welsh language provision

  - Over 65% of the population served by Adra speak Welsh as a first language with many wards in the county having 85% Welsh speakers. It is therefore important to Adra that it and its partners communicate with its Tenants in their language of choice and respect the bilingual nature of the communities we serve.

  - The Contractor must adhere to Adra's "Welsh Language Policy". All communication with tenants must be in both Welsh and English; this includes printed material, public consultation meetings, signage (including hazard warning signs), advertisements in the local press and other marketing and publicity material. Welsh should always appear before the English.

  - Adra can provide guidance on this matter from time to time where required. Contractor should refer to Volume 7 of this tender pack for Adra's full language policy. (Please note Adra may revise our policies from time to time and in such circumstances, we will advise you of any amendments where we consider that they will affect your provision of the required services).

- Publicity and media relations

  - The Contractor must liaise with Adra communication officers on any public relations activity and media relations.

  - Adra will have absolute approval on any press releases issued by the Contractor.

- Management of risk

  - The Contractor will work with Adra to minimise risk to the delivery of the services. All risks will be identified in the form of a Risk Register and reviewed at each performance meeting as detailed in Section 31 above, and each party will agree how and by whom such risks will be managed.

  - In line with Section 25 above, the Contractor will provide Adra with suitable and sufficient risk assessments for all work activities that take place under the contract e.g. working with machinery, slip, trips & falls, manual handling.

  - The Contractor will ensure that all personnel receive appropriate training for all tasks that they are expected to complete.

- Information Security

  - Adra manage data carefully through the implementation of an ICT Security and Acceptable Use Policy, and Data Protection Policy. Contractors will be required to give assurance that they have similar policies in place.

- Business Continuity

  - Adra manage the risk of business disruption and system downtime through the regular review and testing of our Business Continuity Plans and Disaster Recovery procedures. Contractors will be required to give assurance that they have similar procedures and testing in place.

- Pricing

  - For avoidance of doubt, any costs associated with meeting the requirements of this specification are deemed to be included in the Contractor’s completed price framework. This includes all equipment, machinery, tools, people, plant, vehicles, implements, supplies and consumables necessary to meet all the requirements of this specification.