Void Property – Cleaning Specification

Affordable Homes / Market Rent Properties

Void Property Cleaning - Specification

SPECIFICATION

Introduction

Adra Tai (Cyf) has a housing stock of just over 7300 properties and currently experiences approx. 450 empty homes per year / 39 per month. Adra introduced a new Void (empty property) standard starting from the beginning of April 2024 which consists of all void properties receiving a full internal clean to the specification below.

Works will be awarded as per the Lot Structure described within the Framework Brief, Volume 4 of this tender pack.

Performance will be monitored by turnaround times, quality and health and safety record. As per the Framework Brief, Volume 4 of this tender pack.

The contractor(s) will be expected to comply with the specification of works at all times, complete the work within given time frame and to an acceptable quality.

Unless otherwise specified, all defined terms in this Specification have the same meaning as set out in Adra’s Standard Terms & Conditions to which this Specification shall be annexed (the Agreement).

- 2. 	Delivery

  - The general standard of cleaning shall be in accordance with the BICSc Cleaning Standards (which can be found at the following website - www.bics.org.uk), as modified by the contents of this specification. These standards shall be regarded as the minimum standards acceptable to Adra throughout the term of the Agreement.

  - The Service Provider should note that they will be responsible for the provision of all equipment and materials necessary to undertake the works, and any such costings should be included as part of the fixed cost of the pricing framework. The Service Provider shall ensure that all equipment used in the delivery of the Services complies with legislation, that required electrical and mechanical safety requirements such as Portable Appliance Testing are met and that, where applicable, a full risk assessment has been carried out prior to undertaking dangerous tasks such as high level cleaning.

    - All electrical equipment used in connection with this contract on site must have been maintained in accordance with the Electricity at Work Regulations 1989-HSR25.  A full inventory of all items is required, together with evidence of maintenance and testing.  The Service Provider must instruct their employees on steps to be taken if electrical equipment is damaged or suspected of being faulty which must immediately be disabled to prevent any further use pending inspection by a competent person.

  - The Service Provider must fully comply with all provisions of the COSHH Regulations 2002.  Service Providers working on behalf of Adra retain the legal duty to ensure that the activities of their own employees meet the requirements of the regulations.  An inventory of substances to be used and their safety data sheets, as well as any related protection measures must be provided prior to the Commencement of Contract.

  - The Service Provider shall be responsible for the storage of all cleaning products in accordance with legislation and manufacturer’s instructions and where reasonably practical, environmentally friendly products and substances shall be used. All such materials shall be properly and clearly labelled in accordance with the Classification, Packaging and Labelling of Dangerous Substances Regulations 1997 and shall not be left unattended during the course of any work except when properly stored. For these purposes hazardous materials shall include any substances generally considered to be hazardous or notified to the Service Provider in writing by the Service Manager.

  - The Service Provider shall comply with all Health and Safety Regulations, Legislation etc.  These include, but are not limited to the following:

    - Health & Safety at Work Act 1974 and the Management of Health and Safety at Work Regulations 1992

    - Control of Substances Hazardous to Health Regulations 2002

    - Personal Protective Equipment at Work Regulations 1992

    - Provision and Use of Work Equipment Regulations 1992

    - Manual Handling Operations Regulations 1992

    - Reporting of Injuries, Diseases and Dangerous Occurrences Regulations 2013

  - This list is not exhaustive and is only a guide to the level of expectation with regard to Health and Safety and Safeguarding.

  - Notice should be taken of the Asbestos Regulations 2012, and with any other applicable statutory requirements in accordance with the recommendations of the Asbestos Research Council and its Environmental Control Committee.  Particular regard shall be paid to the prevention of any health hazard.   Should any asbestos material be discovered, or suspected, during the course of the service provision, Adra Tai (Cyf) Service Manager must be advised immediately.

## The Service Provider shall provide and use at all times appropriate signage indicating wet floors and similar hazards.

## In order to establish the cleaning standards used in this agreement, the tasks to be performed are defined below:

    - Void Clean – Standards to be achieved:

## Cleaning standards to be achieved need to be in line with the following as a minimum:

- Glazed windows will be cleaned internally including glass, PVCu, sill boards, handles rebates, and ventilators. This will include removal of nicotine, non-permanent marks, dirt and dust.

- Ground floor windows, entrance doors and sills will be cleaned internally and externally.

- First floor windows will be cleaned internally only.

- Balconies, where installed, will be brushed and swept clean.

- All doors will be wiped down internally and externally, including handles and all furniture. This will include removal of nicotine, non-permanent marks, dirt marks and dust.

- Skirting boards, door frames including architraves and other glossed items i.e., shelves, will be cleaned and free of nicotine, non-permanent marks, dirt and dust.

- Kitchen cabinets will be cleaned inside and out including shelves, unit tops, drawer doors, worktops and base shelf and will be free of grease, nicotine, non-permanent marks, dirt and dust.

- Sink bowls and drainers will be clean, including taps, plug, chain, overflow, and outlet and be free of grease, non-permanent marks, dirt, dust and any residue from cleaning products.

- Any integrated appliances including ovens, hobs, cooker hoods dishwashers and fridge/freezers are to be thoroughly cleaned and free from any food residue, grease and grime, inside and out. All appliances should be left in a clean, hygienic and fully operational condition.

- Fire surrounds and radiators (including convection fins) will be clean, free of nicotine, non-permanent marks, dirt, dust and paint splashes.

- Plug/light sockets and light fittings to be wiped and dusted.

- Fireplaces to be wiped and dusted.

- All tiling and grout will be wiped down and cleaned.

- Walls and ceilings will be clean, free from non-permanent marks, dirt, grease and cobwebs.

- Stair treads to be swept clean free of dust, strings, handrails and infill posts or panels will be clean, free of nicotine, non-permanent marks, dirt or dust.

- Floor coverings in bathrooms, kitchens and hallways will be swept, free from dirt, grease, dust and water-based paint splashes. Floors will be wet mopped and dried to manufacturers instruction Altro safety flooring illustrated cleaning guide - manual | Altro | Altro.

- Carpets to be thoroughly vacuumed and free from dirt, dust and loose fibres. Stains to be treated and removed, if necessary, use a professional carpet cleaning service to deep clean and refresh carpets – as per Volume 3a Price Framework Void Cleaning rates.

- Timber and concrete floors will be swept free from dirt and dust.

- Baths, shower enclosures including screens, basins and WCs will be clean, free from grime, non-permanent marks and dust.

- Clean and disinfect all toilets.

- Taps, plugs and chain, waste outlets, bath panels and WC seats will be clean, free from grime, non-permanent marks, dirt and dust.

- Tiled areas will be clean, free of grease, grime, mould and dirt.

- Footpaths will be swept and free from dirt, rubble and litter,

- Litter pick and dispose from gardens.

- A solid gel neutralising air freshener to be left in the property.

- Clean any other landlord fittings e.g., boiler casings etc.

- Also, all tacks, nails, tape and other fixings from walls

## In order to establish the cleaning standards used in this agreement, the tasks to be performed are defined below:

    - HARD FLOORS

## General - Collect all large pieces of litter and rubbish from hard floors and other areas prior to vacuuming, sweeping, or any floor cleaning / maintenance operation and place in a suitable receptacle and remove to the specified collection area. Chewing gum, or similar, and resultant marks to be removed from all floors, other surfaces and furniture using appropriate chewing gum remover. Stains and heavy soiling following building work and footfall to be removed by spot mopping or scrubbing as necessary.

Sweep – The Service Provider to avoid sweeping as far as reasonably practical and use a vacuum, on occasions when using a vacuum is not practical the Service Provider is to use a soft or hard brush according to the conditions, to sweep floors and collect all dirt and fluff, paying particular attention to corner areas where dust gathers. All waste materials to be disposed of.

Spot / wet mop - To immerse the mop head in a suitable cleaning solution, remove and wring out until almost dry. Mop floor and remove all marks and stains from the surface. A warning sign, indicating wet or slippery floors, should be placed in the affected area. All cleaning solutions to be subject to a COSHH assessment.

Wet / damp mop - The laying of a suitable cleaning solution using a cotton head mop and wringing bucket, to remove soiling and dirt. A warning sign, indicating wet or slippery floors, should be placed in the affected area. All cleaning solutions to be subject to a COSHH assessment.

    - SOFT FLOORS

General - Collect all large pieces of litter and rubbish from carpet and other areas prior to vacuuming, sweeping, or any carpet cleaning / maintenance operation and place in a suitable receptacle and remove to the specified collection area. Chewing gum, or similar, and resultant marks to be removed from all floors, other surfaces and furniture using appropriate chewing gum remover. Move light furniture, including chairs and small carpets / rugs which are not stuck down as necessary. Return furniture / rugs after task is completed. All carpet areas shall be fully vacuumed no less than once per week.

Vacuum clean - Vacuum all areas, including under furniture (except the areas beneath permanent fixtures) to remove all types of soiling. Where carpeted areas have heavy stains a spot clean operation will be carried out using a suitable cleaning solution to remove any stains.

    - STAIRCASES / LANDINGS

## General - Dust mop, brush or vacuum staircase/ landings. All dust and dirt shall be removed from treads, risers and strings.

## Banisters / Handrails -Banisters/handrails are to be damp dusted and wiped clean using a solution of germicidal detergent and water. Polish/buff handrails using suitable approved polish and a soft polishing cloth.

    - DOOR ENTRANCES

## Doors, Door Frames and Door Furniture - All internal and external doors, door frames and door furniture to be wiped clean with a solution of approved germicidal detergent and water and a soft cloth to remove grease stains, finger marks and all other marks.

## Metal Kick & Finger Plates - clean kick plates and remove all residues using an approved polish stripper. Buff and polish kick plates with metal polish. Finger plates should be wiped clean using a solution of germicidal detergent and water.

    - FIXTURES & FITTINGS

## All fixtures, fittings, radiators (including convection fins), pipe work, door furniture, ledges, sills, toilet cisterns, electrical fittings, skirtings, architraves, doors, door casings and other horizontal surfaces to be damp dusted using a solution of approved germicidal detergent and water and a damp cloth. According to finish apply suitable polish and buff.

    - INTERNAL GLASS

## Internal Window and Door Glass - Internal window and door glass shall be cleaned using an approved glass cleaner and lint free cloth, all frames and surrounds to be left free from runs, finger marks stains and all other marks.

    - TOILETS & WASHROOM Facilities

## Showers/Baths - All exterior fittings of the shower including curtains and safety mats shall be damp wipe dusted using a germicidal detergent. Plug holes shall be cleared of debris. Rinse with clear water and wipe leaving all surfaces free from smears and stains. External surfaces and surrounds shall be damp wiped using germicidal detergent ensuring all marks are removed. Ceramic tiles and PVC cladding shall be thoroughly washed with an approved germicidal detergent to remove all dirt, grease, graffiti and scuff marks. Treat limescale stains with an approved de-scaler.

## Toilets (General) - Toilets shall be kept odour-free, sanitary and clean for use. All urinals, splash plates, WC pans and pedestals, including tops and undersides of WC seats and lids, associated piping and surrounding edges shall be disinfected and free of ingrained and loose dust, dirt, grease, smears, finger marks and other marks, deposits, stains or accumulations. All surfaces shall be dried to enhance appearance and to reduce risk of bacterial loads.

## Toilets - Flush pans and reduce water level with WC brush. Apply approved toilet cleaner to the inside of the pan and under rim and leave for at least 5 minutes. Scrub pan with toilet brush and flush toilet repeat until all stains are removed. Using a solution of germicidal detergent and water damp wipe/dust all surfaces of toilet seat, lid, exterior of pan, chains, handles, pipes, u-bends, cisterns, surrounds and surfaces of all other fittings. Any faults or blockages shall be reported to the Site Representative and logged in the Site Log (Communication) Book.

## Sinks, Basins & Fittings - Sinks and basins shall be cleared of all extraneous deposits e.g., soap etc. Remove debris etc from plug holes, overflow and clean plug chain. Thoroughly clean internal surfaces of sinks and basins using a germicidal detergent ensuring residues from around taps, overflows, and other fittings until all stains are removed. Rinse with clear water and wipe leaving all surfaces free from smears and stains. External surfaces and surrounds, draining boards/drainers and work tops shall be damp wiped using a germicidal detergent ensuring all marks are removed.

## Mirrors - Clean all mirrors using an approved glass cleaner and soft cloth. Dry polish to finish leaving the surface free from runs, splash and finger marks, stains and smears.

## Descale Sinks, Basins and Toilet Pans - Remove lime scale build up using and approved and appropriate descaler for the type of surface.

    - KITCHENS

## Work Surfaces - Damp wipe/dust, clean and dry all worktops, shelves, sills and adjacent surrounds using a suitable cleansing agent and or bacterial cleanser. An appropriate cleaning cream may be used to remove stubborn grease deposits, but scouring powder must not be used. The contents of worktops, tables, shelves and surrounds should be removed prior to cleaning and replaced on the dry surface.

## Equipment Exteriors and Work Units / Cupboards - Damp wipe/dust, clean and dry all exterior surfaces of work units/cupboards and any other kitchen equipment using a suitable cleansing agent and or bacterial cleanser. An appropriate cleaning cream may be used to remove stubborn grease deposits, but scouring powder must not be used.

## Cupboards - Remove contents of cupboards. Damp wipe/dust, clean and dry all interior surfaces of work units/cupboards using a suitable cleansing agent and or bacterial cleanser. An appropriate cleaning cream may be used to remove stubborn grease deposits, but scouring powder must not be used. Replace contents of cupboards.

## 2.89	INTEGRATED APPLIANCES

## Preparation

- Isolate the appliance from the electrical supply by switching off at the switch or unplugging and ensure it is cool before starting the cleaning process.

## 2.  Gather all necessary cleaning supplies, such as mild detergent, microfiber cloths, sponges, and a soft brush.

## Oven - Remove oven racks and soak them in warm, soapy water. Wipe down the interior with a damp cloth to remove loose debris. Apply a suitable oven cleaner to the interior surfaces and let it sit for the recommended time. Scrub the interior with a sponge or brush, paying special attention to stubborn spots. Wipe down the interior with a damp cloth to remove any residue. Clean the oven door, including the glass, with a glass cleaner or a mixture of vinegar and water. Rinse and dry the oven racks before placing them back in the oven.

## Hob - Remove any removable parts, such as burner grates and soak them in warm, soapy water. Wipe down the hob surface with a damp cloth to remove loose debris. Apply a suitable cleaner to the hob surface and let it sit for a few minutes. Scrub the surface with a sponge or brush, paying special attention to stubborn spots. Wipe down the surface with a damp cloth to remove any residue. Rinse and dry the removable parts before placing them back on the hob.

## Cooker Hood - Remove the grease filters and clean with a non-aggressive detergent. Wipe down the exterior of the hood with a damp cloth and mild detergent. Ensure the hood is buffed and dried fully and is free from grease, grime and streaks.

## Dishwasher - Remove any food debris from inside the dishwasher. Clean the filter according to the manufacturer's instructions. Wipe down the interior with a damp cloth to remove loose debris. Run a cleaning cycle with a dishwasher cleaner. Wipe down the exterior with a damp cloth and a mild detergent.

## Fridge/Freezer - Remove all food items and shelves from the fridge/freezer. Wipe down the interior with a damp cloth to remove loose debris. Clean the shelves and drawers with warm, soapy water. Wipe down the interior with a suitable cleaner. Pay special attention to seals and gaskets, cleaning them with a soft brush. Wipe down the exterior with a damp cloth and a mild detergent. Dry the shelves and drawers before placing them back in the fridge/freezer.

## Final Check - Ensure all appliances are thoroughly clean and free from any food residue, grease, and grime. Check that all appliances are in a hygienic and fully operational condition before leaving the property.