HIGHWAY AND STREET LIGHTING MAINTENANCE TERM SERVICE CONTRACT 2027-2033

DOCUMENT 2

SCOPE & SPECIFICATION

1. Description of the Service

2. The Council’s Vision, Aims and Objectives and Welsh Language Policy

3. Pre Tender Health and Safety Plan

4. HCDHW Specification

5. Special Requirements

Huw Percy

Pennaeth Gwasanaeth (Priffyrdd, Gwastraff ac Eiddo)

Head of Service (Highways, Waste and Property)

CYNGOR SIR YNYS MÔN

ISLE OF ANGLESEY COUNTY COUNCIL

Swyddfa’r Sir

LLANGEFNI

Ynys Môn  -  Anglesey

LL77 7TW

1. Description of the Service

The Authority maintains an infrastructure of approximately 1182km of road, 1086km of public rights of way, 9950 streetlamps and 885 illuminated and non-illuminated road traffic signs.  The objective is to improve and maintain the condition of its asset with the service provided.  The annual expected expenditure on the works included in the contract is a minimum value of:

- £2,000,000 for Highway Maintenance; and

- £500,000 for Street Lighting.

The Authority’s highway maintenance section is responsible for ensuring the safety, functionality, and overall condition of the highway and its assets, and other transportation infrastructures, ensuring it is in optimal condition for drivers and pedestrians.

Duties and Responsibilities

Highway maintenance work ensure the safety, functionality, and overall condition of highways and roads. The duties and responsibilities include:

- Road Repairs: Repairing road damage such as potholes, cracks, and surface erosion.

- Snow and Ice Removal: During winter months, duties include snow ploughing and salting of roads to prevent hazardous driving conditions.

- Routine Maintenance: Performing routine maintenance tasks such as cleaning drainage systems, removing debris, and clearing vegetation along the roadside. These activities help prevent water accumulation, reduce erosion, and maintain proper visibility for drivers.

- Signage and Signal Maintenance: Installation of and the repair and replacement of road signs, signals, and markers. Ensuring that these signs are visible and accurate is crucial for guiding drivers safely and effectively.

- Guardrail and VRS installation and Repair: Installation and repair guardrails and barriers along highways. These safety features are vital for preventing accidents and protecting drivers in the event of a collision.

- Emergency Response: Highway maintenance are often the first responders during accidents, natural disasters, or other emergencies. They quickly assess road damage, clear debris, and make necessary repairs to reopen roads promptly and ensure the safety of highway users.

- Public Interaction: Interacting professionally with the public, responding to inquiries, and addressing concerns. Effective communication is crucial, especially when maintenance activities may cause temporary inconveniences to commuters.

- Lighting: The contractor will carry out routine night inspections of the lighting and illuminated sign assets at intervals specified by the Highway Authority and will identify and provide reports on the defects noted and will be responsible for the repair and upkeep of all lighting assets.

- The contractor shall ensure that there is an emergency standby cover available 24 hours a day throughout the year and shall ensure that there are Emergency Crews available to attend to and make safe any emergency incident which may be reported.

- The contractor shall ensure that there is a Winter Service standby cover available during the winter period between the 1st of October and the 1st of April and that crews are available to attend to any winter operations during and outside working hours.

A detailed breakdown of these work activities will be highlighted in the relevant sections as described throughout this document and will be used in conjunction with the:

- Highway Safety Inspection Policy

- Winter Service Plan

- Highway Asset Management Plan

- Emergency Policy, Plan and Procedure

- Weather Event Procedures

- Gully Emptying Policy

- Weed Treatment Policy

The operations undertaken in the contract and the indicative expenditure for a typical year are: -

Street Lighting 								15%

Footway Works 								6%

Community Gangs and Thermal Road Repair 				20%

Fences and Walls 								2%

Vehicle Restraint Systems and Pedestrian Barriers 2%Drainage 		11%

Gully Emptying 								10%

Weed Killing 									7%

Winter Service 								14%

Emergency Works 								5%

Signs 										2%

Siding and Ditching 								1%

Public Footpaths 								1%

Traffic Works 									3%

Structures Work 								3%

Total										100%

Subcontract

The Highway Authority accepts that the Contractor may have to supplement his own resource by sub-contracting certain works and operations of a specialist nature and furthermore during periods of additional funding.

The Contractor is therefore required to submit a list of anticipated sub-contractors for formal approval by the Highway Authority prior to commencing works, including quality assurance checks that have been carried out. A strong emphasis is drawn to small and medium businesses and how they can benefit from the contract and the local supply chains.

Construction (Design and Management) Regulations 2015 (CDM 2015)

The appointed Contractor will be required to undertake the role of Principal Contractor for the purposes of CDM 2015.

The appointed Contractor acknowledges and agrees that it will assume all statutory duties and responsibilities associated with the Principal Contractor role, including (but not limited to) the planning, management, monitoring and coordination of the construction phase, ensuring compliance by all parties on site, and cooperating with the Client and Principal Designer as required under CDM 2015.

Depots and Workshops

(i) 	Tenders’ attention is drawn to Clause Z14 in the Conditions of Contract which indicates that the Authority will make provisions for two depots on Anglesey for the purpose of the Service, namely at:

    - Council Depot, Compound, Menai Bridge; and

    - Gritter Shed and Salt Barn at Gaerwen, as shown on Plan S3.

These facilities will be available to the Contractor on a lease basis.

The Contractor will be free to elect to provide other facilities for the execution of the Service, at their own expense.

Winter Service Vehicles and Thermal Road Repair (TRR) Plant

Seven highway gritting vehicles and a Thermal Road Repair owned by the Highway Authority will be made available on the starting date to the Contractor to provide the Service.

The specification of the vehicles is described in the Table 1 below.

The Contractor will maintain and service the fleet to the same standard for the duration of the contract at the Highway Authority’s cost. The Contractor is to ensure that all checks are carried out to the correct timescales.

A condition survey of the gritters will be carried out jointly by both parties and agreed prior to start of the contract period. The contractor will be responsible for repairs and any damages incurred by themselves.

The contractor will also insure and license the whole vehicle gritting fleet (7 vehicles) plus the snow ploughs at its cost from the starting date. The Contractor is to insure the fleet to a fully comprehensive cover. A copy of the standards of insurance is available in insurance agreement (see Insurance Agreements for conditions and level of cover required).

Table 1: Specification of Winter Service Vehicles

| Route No. | Description | Vehicle Reg. | Capacity | Ploughs |
| --- | --- | --- | --- | --- |
| 1 | Mercedes 1827A 18t 4x4 with 6Cu Spreader | YD67JNL (will be replaced 2026) | 9t | Yes |
| 2 | Mercedes 1827A 18t 4x4 with 6Cu Spreader | YG68DFX (will be replaced 2026) | 9t | Yes |
| 3 | Mercedes 1827A 18t 4x4 with 6Cu Spreader | YK70HRG | 9t | Yes |
| 4 | Mercedes 1827A 18t 4x4 with 6Cu Spreader | YK70HRL | 9t | Yes |
| 5 | Mercedes 1827A 18t 4x4 with 6Cu Spreader | YK70HRF | 9t | Yes |
| 6 | MAN | WU72EUT | 9t | Yes |
| 7 | MAN | WU72EUV | 9t | Yes |

All ploughs are type WSPD PD 28A

2. The Council’s Vision, Aims and Objectives and Welsh Language Policy

The Council’s Plan

The Isle of Anglesey Council Plan (2023 to 2028) is our principal strategy document.

It influences and informs strategies and plans for the provision of services over the lifespan of the Plan. The Plan can be accessed here: Council Plan 2023 to 2028

The council's vision

'Creating an Anglesey that is healthy and prosperous where people can thrive'.

The council’s vision is to create an Anglesey that is healthy and prosperous where people can thrive.

Collaboration is key to realising our vision. We will be a respectful council that is considerate of others, we will collaborate honestly and we are committed to high standards of behaviour and integrity to present the island and its communities in the best light possible.

To realise this vision, we will need to continue modernising and transforming our services and our way of working. An integral part of modernising will be listening to what our residents, service users and businesses think – and being ready to act.

To create this plan, we carried out our most far-reaching consultation work ever as a council. During the engagement and consultation exercises conducted by us in 2022, we asked residents for their views on their most important priorities. The response played a central role in setting our strategic objectives.

Across all age groups and the different engagement and consultation methods that we used, the consistent strategic objectives were:

- Welsh language - increasing opportunities to learn and use the language

- social care and wellbeing – providing the right support at the right time

- education – ensuring an effective provision for today and for future generations

- housing – ensuring that everyone has the right to call somewhere home

- economy – promoting opportunities to develop the island’s economy

- climate change – responding to the crisis, tackling change and working towards becoming a net zero organisation by 2030

The plan focuses on these objectives.

Like all other councils, we will face significant financial pressure, and further savings will have to be identified. We will have to control increased costs when setting budgets and respond to increasing demands due to demographic and social changes.

To work in this challenging and changing period, we are adopting a strategic approach to managing our resources. Pressures have been identified in the mid-term financial forecasts. Based on the forecasts for inflation, the cost of living and growth pressure, together with an annual council tax increase of 5%, we predict that challenging times lie ahead of us.

Consequently, this is an ambitious plan. As delivering services will be challenging, our residents’ experiences and feedback will be an important measure of success. We will produce an annual delivery document and will formally report on progress, success and outcomes.

By 2028, there will be a significant change in the way services are delivered, success will be celebrated and we will have progressed on the journey of creating an Anglesey that is healthy and prosperous where people can thrive.

Our six strategic objectives are:

- Welsh language

Increasing opportunities to learn and use the language.

- Social care and wellbeing

Providing the right support at the right time.

- Education

Ensuring an effective provision for today and for future generations.

- Housing

Ensuring that everyone has the right to call somewhere home.

- Economy

Promoting opportunities to develop the island’s economy.

- Climate change

Responding to the crisis, tackling change and working towards becoming a net zero organisation by 2030.

Base line example of minimum allowable levels (Contractor may wish to exceed these levels, for fair tendering purposes).

| KPI Ref | Performance Indicator | Measure | Minimum Target | Reporting Frequency |
| --- | --- | --- | --- | --- |
| CR01 | Contract Carbon Emissions | Annual reduction in operational CO₂e emissions against established baseline | ≥ 3% reduction year-on-year | Annually |
| CR02 | Fleet Fuel Consumption | Reduction in fuel consumption against Year 1 baseline | ≥ 3% reduction year-on-year | Annually |
| CR03 | Anti-Idling Compliance | Percentage of audited vehicles and plant complying with anti-idling requirements | ≥ 95% compliance | Annually |
| CR04 | HVO Utilisation | Percentage of eligible plant operating hours using HVO or approved low-carbon fuel | ≥ 10% | Annually |
| CR05 | Ultra Low Emission Vehicles | Percentage of newly procured light vehicles that are ULEV compliant | 10% | Annually |
| CR06 | Waste Diversion from Landfill | Percentage of waste diverted from landfill | ≥ 50% | Annually |
| CR07 | Recycled Asphalt Content | Recycled asphalt planings (RAP) incorporated into asphalt production where technically feasible | ≥ 10% | Annually |
| CR08 | Recycled Aggregate Usage | Proportion of recycled aggregate used in maintenance activities | ≥ 10% | Annually |
| CR09 | Material Reuse | Excavated materials reused or recycled | ≥ 10% | Annually |
| CR10 | Renewable Electricity Usage | Electricity supplied from certified renewable sources at contract facilities | 50% | Annually |
| CR11 | Drainage Asset Inspection Compliance | Gully emptying schedules cleansing target | 90% | Annually |
| CR12 | Flood Sensitive Location Inspections | Inspection of identified flood-sensitive locations before winter season – grillages | 90% | Annually |
| CR13 | Climate Risk Assessment | Completion and submission of climate risk and resilience assessment YM and WG data | 100% | Annually |
| CR14 | Climate Change Reporting | Submission of climate performance report to the Authority | Within 10 working days of year end | Annually |
| CR15 | Supply Chain Environmental Compliance | Contract spend with suppliers operating an environmental or carbon management policy | ≥ 20% of annual spend | Annually |

3. PRE TENDER HEALTH AND SAFETY PLAN

A.	PROJECT

Title of Project:

Highway and Street Lighting Maintenance Term Service Contract 2027-2033

Nature of Project:

Highway, civil engineering and street lighting maintenance works on Anglesey.

Client or Agent:

Highways, Waste and Property, Isle of Anglesey County Council – Contact name: Mr Huw Percy

Principal Designer:

Isle of Anglesey County Council - Contact name: Mr. Gethin Gilford

Notification of Project to HSE:

To be undertaken by the Highway Authority plus 2nd notification for large scale works.

B.	SAFETY HEALTH AND WELFARE OBJECTIVES

Statement of policy objectives:

Execute and complete all works within the time period required in accordance with the conditions and to the specification set out in the contract documents with due regard to the welfare of staff and without putting at significant risk their health and safety and that of others who may be affected by the project, particularly the public.

C.	HEALTH AND SAFETY FILE

Health and Safety file exists.

D.	GENERAL INFORMATION

General working restrictions:

Working hours - The normal working hours within any site shall be Monday to Saturday (excluding Bank Holidays) between 07:30 and 17:00 hours although the Highway Authority representative may require/permit works to be carried out outside these working hours.  Special restrictions may apply when works are carried out near a place of worship.  See Section E - stats.

Contract requires out of hours emergency work and winter service operations.

Noise restrictions - The acceptable noise levels are given in contract specification Appendix 1/9.

Welfare:

The Principal Contractor's attention is drawn to the fact that, in general, some works will be situated in remote rural areas, and this should be considered when developing the Health and Safety plan for providing easily accessible facilities.

Washing and Toilet facilities - All operatives shall have access to adequate washing and toilet (water flushing wherever possible) facilities.  The number of toilets and basins etc. for washing will be dependent on the number of operatives employed at any one site.  Where such facilities are shared between men and women, they must be lockable.

Clothing storage and changing rooms - Consideration must be given to the facilities made available to operatives for changing between, and the storage of, "off-site/everyday" clothing and "on site/protective" clothing.  Adequate arrangements will be required when men and women both use such facilities.  The facilities must incorporate somewhere to dry wet site clothing.

Rest facilities - Adequate rest facilities must be provided for taking breaks and meal breaks which should include provisions for seating, a means of boiling water, a means of preparing food, etc.

E	DESIGN/CONSTRUCTION INFORMATION

Significant hazards:

The Principal Contractor will be required to provide proposals for managing the risks to health and safety associated with the following: -

1.	The Principal Contractor must specifically address the hazards presented by construction traffic, management of the existing traffic, and specific regard to management of pedestrian traffic throughout parts of the Works to ensure the safety of pedestrians and road users.

2.	Emergency services must have access to all properties at night as well as in the daytime.

3.	General excavation and avoiding services (HSG47) ‘Avoiding danger from underground services.

4.	Safety of sites outside of working hours.

5.  Falling objects.  Safety helmets and boots should be worn on all sites, together with reflective clothing.  Ensure that all materials are stocked away from the excavation.  Follow HSE guidance notes series GS28 for all craneage operations.

6.  Materials.  A COSHH assessment should be provided for all materials used on site.

7.  Flying particles.  Goggles should be used wherever materials are cut, dressed, etc. see also (6).  Contractor to be aware for the safety of any cyclists/pedestrians.

8.  Inhalation of dust or fumes.  Approved masks and goggles as in (7) should be worn to protect from dust and fumes.  All public shall be protected from dust/fumes.

9.  Inadequate scaffold.  Ensure that any scaffold has been erected by a competent person in accordance with the HSE Regulations, and that it is checked weekly on the Scaffold Inspection forms (F91 Part 1) and signed by a competent person.

10. Risk of drowning.  Some work may be situated near water, the level of which may rise in the event of storm and high tide.  The Contractor should take the necessary precautions to limit the risk during all work including scaffolding.  Guidance is given in the Health and Safety Executive publication Construction Safety Working Places Section 7 No. 23 and No. 24 - Prevention of Drowning Note: any breaches of the above safety measures may involve (a) suspension of the works, (b) removal of the operatives from the site or other action deemed necessary by the Highway Authority.

Design parameters having a bearing on work organisation:

To be assessed per site.

Facilities for which a maintenance manual is required:

To be assessed per site.

Co-ordination of Health and Safety with other parties:

The Principal Contractor is referred to the special requirements in relation to Wales and West Utilities, British Telecommunications Plc, Electricity generating and distribution companies, Environment Agency, Environment Agency Welsh Region Fisheries and Conservation, Network Rail, Water and Sewage companies, Works on the Existing Highways, National Assembly for Wales - Agriculture Department and UK Highways contained in the Conditions of Contract for the tender.  The Principal Contractor must liaise with these statutory bodies when working in the vicinity of their apparatus or works.  The Contractor must also liaise with the rail network operators when working in the vicinity of any rail tracks or associated land.

F.	MATERIALS

Information on materials with significant hazards:

The Principal Contractor shall consider all hazards associated with the materials used in works.  It is not anticipated that the Principal Contractor will encounter materials with significant hazards outside his field of experience.

Transportation, storage and handling of hazardous materials:

See above

Special requirements:

None anticipated

G.	SITE RESTRICTIONS

Access/egress:

The Principal Contractor shall ensure that all access and egresses form the site are such so as not to compromise the health and safety of others and to cause the least nuisance to adjoining residents.  Emergency services must always have access.

Site accommodation:

The location of any site accommodation will be agreed with the Highway Authority.

Storage areas:

The Principal Contractor shall make provision for the disposal of materials gained through processing to an authorised tip or approved recycling locations.  Storage of other materials are to be agreed with the Highway Authority prior to commencement of the works.  Any temporary stockpiling of materials gained through processing and its subsequent removal and disposal shall be carried out so as not to endanger the surrounding environment and shall only be allowed for very limited periods as agreed with the Highway Authority, the Principal Contractor's Health and Safety plan shall indicate how they propose to deal with such matters.

Traffic/pedestrian safety & management:

Traffic routes - All deliveries to, and removal of materials from, the site are to be routed so as not to compromise the health and safety of others and to cause the least nuisance to adjoining residents.

Temporary traffic restrictions - Site specific to be notified.

Traffic management - The works shall be signed in accordance with Chapter 8 of the Traffic Signs Manual 1991.  The operation of temporary traffic signals will require notice to the County Council's Highways Streetworks Section, notification for which must be made at least 7 days in advance for two way and 1 month in advance for 3-way signals.  The Principal Contractor is reminded that some sites may involve working on narrow roads where an adequate safety zone as required by Chapter 8 cannot be achieved.  Alternative traffic management regimes may need to be considered in such situations.

H.	SITE INSTRUCTIONS

Emergency procedures:

The Principal Contractor shall develop in his Health and Safety plan his procedures for dealing with any emergencies or other incidents on the site.  Access into and through the site for the emergency services to attend any such incident must be maintained.

Operative training requirements:

The works must be supervised by a person having a prescribed qualification as a supervisor as required by the New Roads and Street Works Act 1991.  In addition, a person having a prescribed qualification as an operative as required by the said Act must always be present on site when the works are in progress.

Unauthorised access:

The Principal Contractor shall secure the site to prevent any unauthorised access to areas which are hazardous.  All storage areas shall be secured to prevent any unauthorised access (see also Section G).

I.	SITE LIAISON

Meetings:

The Principal Contractor, Highway Authority’s representative, and, if considered necessary, other Contractors shall meet at the intervals specified by the Highway Authority’s representative to discuss safety issues.

Procedures:

The Principal Contractor is to collate all relevant information which is required to be included in the Health and Safety file.  All information gained by the Principal Contractor during the construction phase of the project is to be stored in a suitable format agreed with the Highway Authority’s and presented to him on completion of the project.

Specific liaison requirements:

To be agreed with the Highway Authority’s representative.

Unsafe practices:

The Principal Contractor shall develop in his Health and Safety Plan his procedures for reporting and dealing with unsafe practices together with procedures for resolving such problems.

Co-ordination of information:

The Principal Contractor should outline his procedures for co-ordination and dissemination of information and advice on Health and Safety matters to all employees, other Contractors and other persons on the site.

J.	SPECIAL REQUIREMENTS

None anticipated.

4. HCDHW Specification

Reference to the Manual of Contract Documents for Highway Works shall be to the pre-September 2025 four-volume structure. Volume 4 – Method of Measurement for Highway Works, 2009 shall apply. No later MMHW documents or revisions shall have contractual effect.

| Volume | Title | Last Amendment Before Sept 2025 |
| --- | --- | --- |
| Volume 1 | Specification for Highway Works | Series-based amendments up to 2023–2024 |
| Volume 2 | Notes for Guidance on SHW | Aligned with Volume 1 updates to 2023 |
| Volume 3 | Highway Construction Details | Drawing updates to 2024 |
| Volume 4 | Method of Measurement for Highway Works | May 2009 (final amendment) |

PREAMBLE TO THE SPECIFICATION

1. Other than where references to the Highway Authority are made in the context of the Highway Authority granting statutory or type approvals, the roles and functions of the Highway Authority shall be undertaken by Huw Percy.

Where the Specification requires the provision of documentation to the Highway Authority for statutory or type approval such documentation shall be provided to Huw Percy.

2. If the Specification is used in conjunction with a Contract under which the Contractor is responsible for the design of any part of the Permanent Works, the delegation of the roles and functions of the Highway Authority as stated in paragraph 12 above shall be further amended as follows:

(i)  If any agreement, consent or approval required to be obtained from the Highway Authority impacts on the health and safety of the general public, the environment or any property or equipment not owned or operated by the Contractor or the Design Build Finance and Operate concessionaire, such agreement, consent, approval shall be obtained from Gethin Gilford.

(ii)  Where the Specification provides for the Highway Authority to require a test, waive the requirement for a test or alter testing frequency, the party to whom the Highway Authority’s roles and functions have been ascribed by paragraph 12 above shall exercise such decisions in accordance with the Secretary of State’s requirements stated in the Contract.

14. Where Standards and other documents are incorporated into the Contract by reference the respective edition used shall be that which is current on the Contract Reference Document Date N/A unless otherwise stated in the Specification.

5.2 APPENDIX 0/0

APPENDIX 0/000:

002 Terms and Abbreviations

Delete 2nd paragraph Item 002-1 and substitute with new paragraph

The term Overseeing Organisation means the Isle of Anglesey County Council as the Highway Authority.

APPENDIX 0/1:

CONTRACT-SPECIFIC ADDITIONAL SUBSTITUTE AND CANCELLED CLAUSES, TABLES AND FIGURES INCLUDED IN THE CONTRACT

LIST OF ADDITIONAL CLAUSES, TABLES AND FIGURES

| Clause No. Table or Figure No. | Title | Page Nos. |
| --- | --- | --- |
| 153AR | Publicity |  |
| 154AR | Survey of existing properties |  |
| 155AR | Structural Condition of Existing Properties |  |
| 156AR | Protection of adjoining properties |  |
| 158AR | Supervision of Staff |  |
| 160AR | Computer record and management systems |  |
| 163AR | Contractors Equipment |  |
| 164AR | Association of Professional Bodies |  |
| 174AR | Supply and delivery of samples to the Engineer |  |
| 205AR | Protection from nuisance due to works |  |
| 502AR | Prevention of damage to and replanting of trees and shrubs |  |
| 516AR | Maintenance of sewage and storm water flow |  |
| 517AR | Safety precautions in sewers |  |
| 618AR | Reinstatement of Sports Turf and Grassed Areas |  |
| 619AR | Control of Japanese Knotweed in construction |  |
| 642AR | Excavation and Filling of Cellars or like voids |  |
| 643AR | Breaking up existing pavement |  |
| 644AR | Siding |  |
| 706SR | Highway Crossings |  |
| 770AR | Carriageway and pavement finished surfaces |  |
| 950AR | Geotextiles used to reinforce pavements |  |
| 951AR | Repairs to existing bituminous surfaces - potholes and depressions |  |
| 1222AR | Removal of existing lighting or signing equipment or road markings |  |
| 1427AR | Temporary overhead feed to lighting units |  |
| 1450AR | General |  |
| 1451AR | Random replacement/repair to electrical equipment |  |
| 1452AR | Minor Works |  |
| 1454AR | Cleaning Lubricating and visual inspection |  |
| 1455AR | Electrical Test of Lighting Units |  |
| 1456AR | Structural Inspection of Lighting Units |  |
| 1457AR | Re-alignment of Lighting Units |  |
| 1458AR | No supply |  |
| 1459AR | Identification Numbering and Danger Warning Labels |  |
| 1460AR | Trimming of Trees/Shrubs |  |
| 1461AR | Disposal of Waste |  |
| 1462AR | Special Columns |  |
| 1463AR | Night Inspections |  |
| 1930AR | Painting |  |
| Series 2700 | Community Gang |  |
| Series 2800 | Gully emptying |  |
| Series 2900 | Emergency works |  |
| Series 3100 | Winter maintenance |  |

NOTE:	   There is no contract specific substitute or cancelled Clauses

| 153AR | Publicity 1 The Contractor shall not give any information concerning the works for publication in the press, social media, or on radio, television or screen elsewhere without the written approval of the Overseeing Organisation. 2 No advertisements shall be erected within the Site by the Contractor or by any Sub-Contractor without the written approval of the Overseeing Organisation (as Drawing S5 or similar approved in the contract drawings). |
| --- | --- |
| 154AR | Survey of Existing Properties 1 Prior to the commencement of works in the vicinity of properties the Contractor shall plan and accompanied by the title holder or his appointed representative, shall compile schedules of conditions (together with photographs). The Overseeing Organisation or his appointed representative shall be present at all inspections, and the Contractor shall give 7 days’ notice of his intention to carry out such inspections. |
|  | 2 One copy of the Schedules of Conditions, including photographs, shall be forwarded to the Overseeing Organisation seven days prior to any work being commenced in the vicinity of the properties concerned. 3 The details of the survey shall be as described below: i) Photographs, half plate size to be taken for each property giving a general indication of cracking, structural damage or defects and any other visual faults including damp penetration. |
| 155AR | Structural Conditions of Existing Properties 1 The Contractors working methods are to take account of the existing structural conditions of adjacent properties to avoid further structural deterioration and aggravation. 2 Particular attention is drawn to the properties referred to in the task order and/or scheme drawings. 3 The Contractor is to submit his working proposals to the Overseeing Organisation giving 7 days’ notice, for operations in the vicinity of any properties indicating structural defects which may be aggravated by the Contractor’s normal working methods. (ii) The length and width of cracks to be assessed accurately. Structural cracks to be monitored throughout the currency of the contract with approved ‘tell-tale’ devices. (iii) Both the interior and exterior of the facades of the properties to be surveyed. |
|  | (iv) A further copy of the completed survey (Schedule of Conditions etc.) is to be furnished to the owner or tenant of each property upon request. (v) All photographs to be presented in albums clearly annotated as to the property number, owner’s name, particular aspect of view, etc. 4 All photographs to be in colour. |
| 156AR | Protection of Adjoining Property 1 Sufficient and satisfactory precautions must be taken by the Contractor to preserve and protect any existing properties, buildings or other structures, roads, paths, walls, fences or apparatus of Third Parties in the vicinity of the Works during construction of the Works and during Maintenance Period from all harm, including damage arising from the influence of heavy loads associated with the carrying out of the Works. Any damage which occurs in this respect must be done good to the satisfaction of the owners, occupiers or tenants, and to the approval of the Overseeing Organisation at no extra charge to the Overseeing Organisation. |
|  | 2 The Contractor shall notify the Overseeing Organisation of all claims relating to any damage or injury which may occur to any such property during the progress of the Works, or during the period of maintenance. The Contractor shall notify the Overseeing Organisation of all such claims for which he is responsible. |
|  | 3 Where approved arrangements are made for temporarily dismantling any property, building or structure, wall, fence or apparatus, the Contractor must re-erect, make good and otherwise reinstate it to at least its condition prior to dismantling to the reasonable satisfaction of the owners, occupiers or tenants and the Overseeing Organisation. |
|  | 4 Where existing properties, buildings, structures, roads, paths, walls, fences or other apparatus are on or adjacent to the site and are liable to be affected in any way by the Works, the Contractor shall prior to any works being carried out which may affect such property etc., inspect such property, together with the Overseeing Organisation’s Representative, the Overseeing Organisation’s Estate Surveyor, the owner and any other interested parties in order to obtain an accurate record of any existing defects at the time of inspection. Photographs shall be taken of such property etc., at the Contractor's expense if so, directed by the Overseeing Organisation and one copy of all such photographs shall be provided to the Overseeing Organisation free of charge. |
|  | 5 Before commencing any excavation for new works in the proximity of or under any buildings, walls or any other structures the Contractor shall shore up or otherwise support any part of such buildings, walls, or any other structure which may be affected by the operations of the Contractor. |
|  | 6 All shoring, supports and underpinning shall be carried out so as to cause the minimum amount of damage to the building, walls or any other structure and any such damage shall be made good by the Contractor at the completion of the Works, to the satisfaction of both the Overseeing Organisation and the owners of the property concerned. The cost of all such temporary work and repairs so required shall be deemed to be included by the Contractor in his rates for excavation, unless a specific item for such temporary work is included in the Price List. |
| 158AR | Supervision and Staff 1 The Contractor’s Operational staff shall comply with: - (i) Electrical Association Engineering Recommendation ERG 39. (ii) BS 7671:2018 Requirements for Electrical Installations. |
|  | (iii) The Institution of Lighting Engineers, Code of Practice for Electrical Safety in the Highway Electrical Operations. (iv) The Electricity at Work Regulations 1989. (v) The Health and Safety and Nuclear (Fees) Regulations 2022 & 2023. 2 The Contractor shall submit to the Overseeing Organisation details of the number of supervisors, chargehands, foreman and workman to be employed in the day-to-day performance of the Contract, the information shall include where appropriate the competent persons category of the Highway Sector Scheme and shall be updated as and when required. |
| 160AR | Computer Record and Management Systems 1 General (a) The Overseeing Organisation operates ‘Alloy’ computerised management system for highway maintenance and street lighting work utilised for the following: - (i) Storage of inventory records covered by the Contract. (ii) The transmission of works instruction and fault instruction by the Overseeing Organisation. (iii) The return of completed reports, repairs and final measures by the Contractor. (iv) Budgetary management and audit. (b) The Contractor shall within 28 days upon being awarded the Contract make available, at his own cost, facilities as described in Sub-Clause 2 of the Clause, for the operation of the computerised system between the Contractor and the Overseeing Organisation. (c) The Contractor shall provide in his rates for all necessary liaison with the Overseeing Organisation during development and revision of the arrangements detailed in this Clause as development proceeds towards the establishment of ‘Alloy’ a fully electronic data exchange system between the Overseeing Organisation and the Contractor. The Contractor should be aware that arrangements may be subject to amendment from time to time as circumstances dictate. (d) During the setting up period of the computerised system it may be necessary for information to be passed between the Overseeing Organisation and the Contractor in writing in the format provided by the Overseeing Organisation. Until such time that the system is fully operational the contractor will be required to plan at his own expense to collect/return the required data in 1 (a) (ii) (iii) above daily. |
|  | 2 Computer System for the Contractor - The Contractor shall after being appointed provide and install and maintain, at his own cost ‘Alloy’ a computer system which will be fully compatible with the Overseeing Organisation computer system to enable the transfer of data via a suitable link between the Overseeing Organisation Office and the Contractor’s local office/store. |
| 163AR 164AR 174AR | Contractor Equipment 1 The Contractor shall ensure that items of Contractors Equipment shall be suitable in every respect for the purpose intended and shall comply with all current and relevant Regulations, Specifications and Code of Practice. Association of Professional Bodies The contractor shall be enrolled as an Approved Contractor on the Register of the National Inspection Council for Electrical Installation Contractors, the Association of Street Lighting Electrical Contractors. Should the enrolment be suspended or repealed the Contractor shall forward an early warning notification to the Overseeing Organisation within 24 hours. Supply and Delivery of Samples to the Overseeing Organisation |
|  | 1 The Contractor shall provide all equipment necessary for the testing of materials and workmanship. |
|  | 2 The Contractor shall be responsible for taking all samples and preparing, storing and delivering samples of tests pieces to the Materials Laboratory or other laboratories for testing. Samples shall be taken in accordance with the methods and with the equipment specified in the latest revised British Standards where such exist. The Overseeing Organisation may require additional sampling to be carried out to investigate unusual properties of the material. Samples of materials shall be taken either before or after placing in the work at the direction of the Overseeing Organisation, and if he so desires, in the presence of the Overseeing Organisation’s Representative. Samples and test pieces may be selected and sampled by the Overseeing Organisation or his Representative. |
|  | 3 Where practicable samples shall be taken in sufficient time to allow testing to be completed and the Overseeing Organisation’s approval obtained before the material is used in the works. 4 Each sample shall be accompanied by a delivery note stating the name of the Contractor, the origin of the material, the title of the works, the location of the material represented by the sample or other information as the Overseeing Organisation may direct. The Contractor shall maintain a complete record of all samples taken and such record shall include details of the quantity of materials in the work, the name of the supplier and method of sampling. The record shall be available for the inspection of the Overseeing Organisation. |
|  | Where tests are ordered by the Overseeing Organisation to be made at a commercial test house the Contractor shall place the order for such tests. |
|  | Two copies of the results of tests made at a commercial test house shall be forwarded to the Overseeing Organisation as soon as they are available. |
|  | 5 All testing and commissioning shall be carried out in accordance with the requirements of the latest revised British Standards where such exist or as specified in this Specification. |
| 205AR | Protection From Nuisance Due to the Works 1 Without prejudice to the Conditions of Contract, existing roads, footpaths, accesses to adjacent houses, buildings, any other right of way, and any new road, temporary road diversions, drains, ditches and grips whether part of the Site or not and which are being used by any vehicles or items of plant of the Contractors or his sub-contractor or suppliers in connection with the Works, shall be kept clean and free from all dirt, mud and material dropped from vehicles or tires and tracks. |
|  | 2 Existing roads shall not be used for the storage of materials or plant. |
|  | 3 The Contractor shall take all precautions to prevent the spread of airborne dust and to conform to all relevant Statutory Instruments. 4 Any area of the Works from which dust may arise shall be the subject of close control by the Contractor, who shall provide such water spraying, protective screening or other suitable equipment as shall be required to reduce the dust level to an absolute minimum to the satisfaction of the Overseeing Organisation. |
| 502AR | Prevention of Damage to and Replanting of Trees and Shrubs 1 Working space within wooded areas shall be limited to the minimum requirement agreed with the Overseeing Organisation. All operations affecting trees and shrubs shall be carried out in accordance with BS5837: 2012. 2 No trees or shrubs shall be felled, lopped or topped without prior approval of the Overseeing Organisation following consultations with the Ynys Môn County Council, Planning Department and landowner. 3 Protective fencing (1.2m high chestnut paling BS1722 Pt4) shall be erected on limits of working space in wooded areas, maintained and removed on completion following reinstatement. |
|  | 4 All arboriculturally works shall be carried out by a specialist operator and carried out in accordance with BS5837 requirements. 5 Tree planting, shrub planting and/or landscape work shall be carried out within the first planting season following the works. 6 All excavation for trenches within wooded areas shall be by hand. Excavated material shall be stored on plastic sheets separating surface and subsoil materials. |
|  | 7 The Contractor shall indemnify the Overseeing Organisation against any claim in respect of injury, damage or loss in accordance with Schedule 3, Section 8 of the New Roads and Street Works Act 1991. |
| 516AR | Maintenance of Sewage and Storm Flow 1 During the undertaking of any Works, the Contractor shall arrange for the maintenance of the sewage, surface water, sub-soil and storm water flow throughout the period of the Contract and for turning the flow into the new sewers or culverts at the appropriate times including the maintenance of flow in ditches, river and stream channels and culverts. |
|  | 2 The Contractor shall provide and maintain all temporary works, dams, chutes, tanks, pipes, plant temporary diversions and channels necessary to enable delivery of the Works. Upon completion of the Permanent Works the Temporary Works shall be removed or filled up as required by the Overseeing Organisation and the ground reinstated as is required for the Permanent Works. Where work is to be carried out on existing pipes, manholes, culverts or channels the Contractor shall ensure that no debris falls into the flow. |
|  | When pumps are installed for the maintenance of flow the Contractor shall arrange for continuous attendance and for sufficient stand by pumps, hoses and all other equipment necessary to ensure continuous pumping in the event of a breakdown. 3 Full details of the Contractor's proposals for dealing with the flow, including the proposed timing, shall be submitted to the Overseeing Organisation for his approval which shall not relieve the Contractor of his responsibilities. The Overseeing Organisation is to be notified before any work commences on existing sewers. |
|  | 4 Where any new sewer is laid to supersede an existing sewer which is to be abandoned, the Contractor shall locate and trace all live connections to the existing sewer and shall reconnect these to the new sewer after works downstream have been completed and tested. The existing sewer shall not be removed or grouted up until all reconnections have been completed to the Overseeing Organisation’s satisfaction. The Contractor's proposals for locating to the sewer shall be approved by the Overseeing Organisation before work commences. It is the Contractor's responsibility to ensure that no live connections are left connected to pipes which have been removed, grouted or blocked up. 5 Other existing drains and sewers shall be located and traced where required by the Overseeing Organisation. |
| 517AR | Safety Precautions in Sewers 1 Where the works involve entry into, or the carrying out of works in any sewers, the Contractor shall take all necessary steps to ensure the safety of all persons and shall comply with the “Health and Safety at Works Act 1974” and any regulations and requirements of the Water Authority preventing accidents in sewers and confined spaces and the recommendations set out in “Safety in Sewers and at Sewage Works” published in 1963 (as amended) by the Institution of Civil Engineers and the recommendations of the Health and Safety Executive in relation to entry into confined spaces. The Contractor is to supply, regularly maintain, and have available on the Site, all necessary safety and rescue apparatus. |
|  | 2 Prior notification is to be given to the Overseeing Organisation of the Contractor’s intention to carry out any work in existing sewers and manholes. |
| 618AR | Reinstatement of sports turf and grassed areas 1 Existing turf shall be stripped off the whole working width and stacked to one side for reuse. Turf shall be undercut to not less than 25mm depth, in sections suitable for movement without fragmentation. |
|  | 2 Turf shall be stacked for not more than 14 days, and protected from drying out or frost, by covering and/or watering. |
|  | 3 Distinct layers of topsoil and subsoil shall be excavated and stored separately without mixing. If placed on undisturbed turf or hard surfaces, a protective sheet shall be laid before placing the soil, soil shall not be allowed to become waterlogged. |
|  | 4 The trench shall be backfilled using each material in its natural sequence. The volume of ducts and imported materials shall be deducted from the lowest layer in the reinstatement sequence so that the full depth of topsoil is replaced. |
|  | 5 Each material shall be placed and compacted to the same density as the corresponding undisturbed material adjoining. The sides of the trench shall not be smeared or compacted, so that continuity of water movement is maintained. |
|  | 6 All compaction over the whole working strip shall be removed by cultivation, and the surface brought true to levels. |
|  | 7 An application of fertiliser, 10:15:10NPK formula at 25g/lm2 shall be raked into the reinstated topsoil surface before turf replacement. 8 Turf shall be replaced and firmed into place so that all joints are closely butted and the surface is even and true to surrounding levels. Any shortfall in turf shall be made up with cultivated turf of matching species composition, from a source approved by the Overseeing Organisation. |
|  | 9 The turf shall be irrigated with water applied evenly by sprinkler, immediately after laying. Further irrigation shall be applied during the maintenance period as necessary to prevent drying out of the turf. 10 Remedial seeding/turfing If during the maintenance period the turf should shrink or crack, the joints shall be filled with fine sifted soil and over sown with BSH A8 (or Interseeds Pro 75) outfield turf mixture at 30g/m2. Any areas of failed turf shall be cut out and replaced with cultivated turf of matching species composition, from a source approved by the Overseeing Organisation. |
|  | 11 Mowing Mowing of the working area during the maintenance period will be undertaken by the Overseeing Organisation. |
| 619AR | Control of Japanese Knotweed and Himalayan Balsam in Construction 1 This specification deals with the general requirements for the treatment and control of Japanese Knotweed. |
|  | 2 It sets out amendments and additions, if any, to the standard Specification 600, and shall be read as part of this Specification. |
|  | 3 The Contractor is reminded that the following legislation is applicable to his operations: i) The Wildlife and Countryside Act 1981 (Amendments) |
|  | Schedule 9 Section 14 makes it an OFFENCE for anyone knowingly to introduce Japanese Knotweed into the wild. ii) The Control of Pesticides Regulations 1997 (Amendments) These regulations require all those who handle or use pesticides (including herbicides) to hold approved Certificates of Competence. Only Approved pesticides may be used. |
|  | iii) The Environmental Protection (Duty of Care) Regulations 1991 |
|  | These regulations require all producers, carriers and disposers of waste to follow the Code of Practice and keep records. Any material contaminated with Japanese Knotweed is a "waste" unless treated for re-use. 4 This Specification also contains clauses which require compliance with this legislation. |
|  | Identification 1 Japanese Knotweed is a vigorously growing perennial plant which invades other vegetation and may cause damage to permanent works. 2 The Contractor shall ensure that his staff, and those of all sub-contractors can identify this plant and are aware of the need to control it. He shall display in his site offices and messrooms the illustrated leaflets provided by the Overseeing Organisation. |
|  | Purpose of Control 1 Japanese Knotweed is time consuming and expensive to control once it gains hold. It spreads rapidly beneath the ground surface. Small fragments rapidly re-grow into invasive plants. This specification is intended to avoid further spread of Japanese Knotweed, and to eradicate it wherever possible. |
|  | SITE INSPECTION AND CONTROL OF SPREAD Initial Inspection 1 Before the commencement of site clearance of any earthworks, the Contractor shall inspect the site with the Overseeing Organisation’s representative, to identify and locate the presence of Japanese Knotweed. Attention shall be paid to watercourses, tipped material, site boundaries and adjoining land. All occurrences of Japanese Knotweed are to be recorded on plans by the Contractor with copies provided to the Overseeing Organisation. |
|  | Interim Inspections 1 At all stages of the works, any Japanese Knotweed observed by the Contractor shall be notified to the Overseeing Organisation, who will give instructions for its disposal or control. |
|  | Control of Spread 1 No disturbance of identified knotweed areas shall be permitted until the written approval of the Overseeing Organisation has been given, this will only be: |
|  | (a) after effective eradication of Japanese Knotweed has been demonstrated, by means of the viability test given in this specification; or (b) where material is to be placed in designated disposal areas; or (c) where material is to be placed in designated control area; or (d) under other circumstances at the Overseeing Organisation’s discretion. |
|  | Imported Material 1 The Contractor shall ensure that no Japanese Knotweed is brought on to the site by his works or those of any sub-contractor. He shall take such steps as necessary, which may include: i) inspection of any material insitu; before importing. ii) inspection of each load as delivered. 2 All materials delivered to site and found to contain Japanese Knotweed shall be removed and returned to the place of origin, at the Contractor's own expense. The haulier and originator of the material shall be notified in writing of the presence of Japanese Knotweed, and this notification copied to the Overseeing Organisation. |
|  | Exported Materials 1 Material in which Japanese Knotweed has been identified or is observed, shall, if it is to be exported from the site, be classed as "controlled waste" and subject to the Duty of Care regulations. In addition, it shall only be exported from the site with the written approval of the Overseeing Organisation who will require to be satisfied as to its destination and treatment. |
|  | Disposal areas on site 1 The Overseeing Organisation may designate areas within the site for the disposal of Japanese Knotweed and infested fill material. |
| 642AR | Excavation and Filling of Cellars or Like Voids 1 Any fill material within a cellar or like void shall be excavated. 2 External underground walls shall be demolished to 600mm below earthworks outline or to sub-formation level whichever is the lower, as shown on the drawings. |
|  | 3 Dividing walls to adjacent cellars or like voids shall be demolished to the lowest floor level. 4 The lowest floor is to be perforated to the satisfaction of the Overseeing Organisation, to allow for drainage. 5 The cellar or like void shall be filled with acceptable material to Clause 601 and compacted in accordance with Clause 612 (alternative Class C7.5P concrete may be used as directed by the Overseeing Organisation) to earthworks outline or sub-formation level whichever is applicable. |
| 643AR | Breaking Up Existing Pavements 1 Where the existing pavement (footway and carriageway) is directed to be loosened and broken up this shall be by approved means, including hand excavation in the vicinity of statutory undertakers' mains and other service cables and equipment. The surfaces are to be broken to achieve drainage through the pavement with the approval of Overseeing Organisation. The existing pavement shall be broken into pieces no greater than 0.5 sq. m in areas to obtain permeability and then re-compacted were directed by the Overseeing Organisation. |
|  | 2 Where the Contractor has over broken the existing pavement, then this shall be made good to the satisfaction of the Overseeing Organisation. |
| 644AR | Siding 1 Siding shall include the removal of vegetation, vegetable matter and all other deposits along the edge of carriageway to expose the edge of pavement construction. |
|  | 2 Siding shall be carried out in such a manner to maintain a uniform and tidy edge to the verge. 3 The exposed pavement construction shall be left in a clean state free of debris and detritus. 4 The Contractor shall take all necessary measures to avoid damaging the pavement construction. If, in the opinion of the Overseeing Organisation damage is caused by the negligence of the Contractor, the Contractor shall carry out such repairs as are necessary to reinstate the pavement construction at his own expense. |
| 706AR | Highway Crossings 1 License under Section 50 and Schedule 3 of the New Roads and Street Works Act 1991 as amended. Reinstatements, interim or permanent, shall be undertaken in accordance with Section 71 of the Act and Paragraph 1.3 of the Code of Practice. |
|  | HAUC Specification for the Reinstatement of Openings in Highways June 92 as amended. Highway verge shall be reinstated in accordance with Section 9 of The Code. The Contractor shall observe and enforce the Safety Measures of the Act (A Code of Practice issued under Section 65) and shall execute the works in accordance with the recommendations contained in Chapter 8 of the Traffic Signs Manual (2009) published by HMSO as amended. Regard shall be taken to the needs of people with disability. For excavations where the depth of cover over the buried plant or equipment does not exceed 1.5 metres the maintenance period for permanent reinstatement shall be 2 years from the date of the completion of the permanent reinstatement. Where the depth exceeds 1.5 metres the maintenance period for permanent reinstatement shall be 3 years from the date of the completion of the permanent reinstatement. Should a reinstatement be found to be defective the Contractor will be charged for each inspection carried out by the Highway Authority. |
|  | Training Qualifications The Contractor shall name and provide copies of certificated Units of Competence achieved with details of awarding body and Assessment Centre for the Supervision and Operatives carrying out the work. |
|  | To satisfy the requirements of the Act, the Contractor shall ensure the Works are carried out by named trained Supervisor of Operatives. |
| 770AR | Carriageway and Pavement Finished Surfaces In no circumstances shall finished surfaces be used as working areas or for storage or as operating platforms for plant. |
| 950AR | Geotextiles used to Reinforce Pavements 1 A geogrid is to be used over existing concrete haunch as directed by the Overseeing Organisation and in accordance with the manufacturer's instructions. |
|  | 2 The geogrid shall be a woven polypropylene geotextile with the following characteristics: - |
|  | (a) Breaking load of 40 KN/m (b) Ultimate elongation of 8% (c) Weight at 240g/m2 (d) Laid with tack coat 1-1.3 l/m² (e) Temperature of overlaying material not to exceed 1650C. |
|  | 3 Prior to laying the geogrid the following surface treatment shall be carried out: - (a) All cracks of widths greater than 3mm shall be blown clean and dry with hot compressed air and filled with hot pour polymer modified bitumen. |
|  | (b) The surface is to be dry, swept free of dirt, oil, vegetation and other debris by a mechanical broom and sprayed with tack coat which shall be allowed to break and dry. |
|  | 4 The geotextile to be laid out either by hand or mechanical under sufficient tension to avoid ripples. Transverse joint to be lapped by 500mm minimum in direction of power. Longitudinal joints to be lapped by 100mm. 5 The grid surface to be rolled with a rubber coated drum roller maximum two passes to activate adhesion. 6 Grid to be kept clean of mud, dust and other materials. Construction traffic to avoid turning braking or sharp movement when travelling on grid. 7 Grid to be overlaid by surfacing materials directly. 8 Materials to be stored in accordance with Manufacturer’s instructions. |
| 951AR | Repairs to Existing Bituminous Surfaces - Potholes and Depressions Surface Profile, Regularity and Texture 1 The surface of the completed work shall comply with the requirements of the HAUC Specification for the Reinstatement of openings in Highways as amended. |
|  | If on checking the level of surface course after compaction any area is found to be outside the limits of the above requirements the full depth of the surface course material shall be removed in the areas concerned and replaced by new material, properly compacted. |
|  | Joints 2 Joints for reinstatements, patches and surfacing shall be let into the existing surface by cutting back to vertical edge faces unless otherwise agreed with the Overseeing Organisation. Such joints shall be formed by compressor tool or by planning and removing of material to the requisite depth. Joints in surface courses shall be sawn. The edge faces of joints shall be painted with rapid curing cationic bitumen emulsion to BS434, typically 50 pen grades at 70% bitumen content or hot applied 50 pen grade bitumen to BS3690. |
|  | Setting in for Footway Surface Course Overlays 3 The existing footway shall be scarified, planed or burnt off to allow a minimum 20mm thickness of resurfacing at the setting-in point. |
|  | Temporary Ramps 4 The Contractor shall make provision for making safe in suitable material any raised joint at the end of each day's work and shall allow for this in his rates. |
|  | Works to be carried out within 24 hours of Instruction 5 Inspections and maintenance of County highways are carried out in accordance with the Road Liaison Group - Well Managed Highway Infrastructure Oct 2016 and amendment and the Isle of Anglesey County Councils HIGHWAY SAFETY INSPECTION POLICY AND PROCEDURES November 2023. Maintenance requirements for defects identified within the Policy. |
| 1222AR | Removal of Existing Lighting or Signing Equipment or Road Markings 1 Where an existing system to lighting or signing is in use before roadworks commence the Contractor will be required to make provision for the maintenance of lighting and signing during road construction. The lighting shall be to a standard of not less than that existing before work commenced. Before the electricity supply to existing equipment is cut off the Contractor shall allow for the re-cabling of existing electricity feeds to signs, bollards, lighting columns or other equipment which must be maintained. The Contractor shall then advise the Overseeing Organisation’s Representative to disconnect the service. |
|  | The Contractor shall allow for the supply of all cables, terminations, jointing boxes, jointing material, screws and other items necessary for the completion of this work. He shall also allow for all labour and plant involved in cable trenching, cable laying, laying marker strip, backfilling, consolidation, temporary reinstatement, cable jointing, marking off terminations, testing and connecting. |
|  | 2 When the equipment has been disconnected the Contractor shall allow for the removal of lamps, lanterns, signs, control gear, time switches, fuses, neutral blocks and column doors protecting these from the weather and delivering and off-loading to store as directed in the Price List. |
|  | 3 Only after removal of the items described in paragraph 1 and 2, of the Clause shall the Contractor dig out the columns which are no longer required first exposing all public utilities underground services and other properties before removing the columns by crane. Metal columns shall have the foundation concrete removed and the bracket arms also removed, and the columns and bracket arms shall be delivered to store as directed in the Price List. |
|  | Concrete columns shall be removed from site to tip, and the Contractor will be required to advise the Overseeing Organisation as to the location of his tip. It is emphasized that the arrangements for the tip are the Contractor's responsibility and an allowance should be made for this in his tender. |
|  | 4 The Contractor shall allow for this additional fill material required to complete his temporary reinstatement after removal of columns and this will take the form of selected fill or hardcore approved by the Overseeing Organisation and returned in layers of 150mm and well compacted. |
|  | 5 The Contractor shall give notification of all connection and disconnection dates to the Overseeing Organisation, at the end of each week. 6 Existing Road marking shall be removed by heating the marking and road surface which shall be removed to a depth of approximately 6mm. The affected area shall then be painted with hot bitumen, and 3mm fine graded macadam applied to make up to the original surface level. Other methods for the removal of road markings may be approved by the Overseeing Organisation. i) Department of Transport Publications (Published by HMSO). ii) Department of Transport Publications (Published by the Department). iii) Transport and Road Research Laboratory Publications (Published by TRRL). iv) Miscellaneous |
| 1450AR | General 1 The Contractor shall make the necessary arrangements to ensure that all jobs issued under fault instructions are completed as soon as reasonably practicable, and in any event within five (5) working days of notification by the Overseeing Organisation. The Contractor shall record and submit details of all works undertaken and submit this information electronically to the Overseeing Organisation within six (6) working days from the date of instruction. 2 Jobs issued under works instructions shall be programmed and completed within the specified time. The Contractor shall record the details and measurements of the works undertaken and submit this information electronically to the Overseeing Organisation within six (6) working days of completion. 3 In order to enable the Contractor to carry out the works safely and to fulfil their obligations under the Contract, the Contractor shall ensure the provision and maintenance of adequate quantities of standard plant, materials, and equipment. This shall include suitable access equipment required for routine maintenance of street lighting and illuminated road signs, as well as resources necessary for the replacement of columns, posts, and lighting units damaged because of road traffic collisions. 4 All other materials required to undertake random replacement and bulk changing of lanterns shall be supplied and appropriately stored by the Contractor to ensure that such works can be completed within the specified timeframes. 5 All materials used under the Contract, and the methods of installation employed, shall comply in all respects with the Specification and associated Appendices. |
| 1451AR | Random Replacement/Repair to Electrical Equipment 1 Notification of faulty/defective equipment including Minor Works as Clause 1452AR will be issued to the Contractor via telephone/computer link to the Contractor local office and will include faults from night inspection and any other faults/defects report daily by the public. 2 Upon receipt of fault instruction the Contractor shall arrange to visit site and carry out all necessary works under his control to ensure that all defective items are repaired/replaced and date recorded as required. 3 Faults which cannot be rectified within 5 working days of issue to the Contractor due to conditions the Overseeing Organisation deems to be outside the reasonable control of the Contractor are to be reported as soon as possible to the Overseeing Organisation but in any case, within 6 working days of issue. 4 Additional works instruction may be issued by the Overseeing Organisation to cover faults on the Overseeing Organisation’s private cable network; these may be in addition to the initial fault instruction. 5 Random replacement/repair to Electrical Equipment shall include visual inspection specified in Clause 1454AR. 6 Any temporary repair must be reported to the Overseeing Organisation by the end of the working day. The Overseeing Organisation will determine if variation to the time allocation will be required to enable full rectification/repair to be carried out. 7 When lamp replacement is carried out under Clause 1451AR the Contractor shall also carry out operations under Clause 1454AR the completed date will be entered on the records for updated bulk lamp replacement frequency. 8 When the Contractor observes lamps which normally are operational at night but burning during the day (day burner) he shall investigate and report to the Overseeing Organisation who shall issue an instruction to cover the fault. 9 Where the Contractor considers that a lantern or equipment is beyond economic repair he shall request approval from the Overseeing Organisation to replace. 10 The Overseeing Organisation may require an Early Response to a report of faulty/defective equipment, on such occasions the fault/defect is to be rectified within 24 hours of the issue of instruction to the Contractor. |
| 1452AR | Minor Works 1 The Contractor shall carry out Minor Works which will include for attendance on site to carry out works of a minor nature, these works shall include for refix dislodged, missing or insecure door with a permanent door of like type. If the correct door type is not readily available the Contractor shall undertake a temporary repair which shall effectively make the base compartment electrically safe and seal the base compartment from unauthorized access and the ingress of moisture until such time, but within 28 days a permanent door can be fitted. 2 Minor Works shall also include for the refix of dislodged photoelectric cell detector units, refix dislodged bollard shell after vandalism or road traffic collision, refix dislodged lighting units, bowls globes, lenses and lamps, realignment and tighten fixing of standard sign faces up to and including 1.2m square area, realignment and tighten fixing of bracket for lighting unit, refix dislodged fuses and carriers and reset MCB, reconnect dislodged earth bonding, verify electrical supply and inventory details and inform Overseeing Organisation of any outstanding defects. 3 Minor Works shall include for attendance on site on the instruction of the Overseeing Organisation to verify correct operation of the Traffic/Pedestrian Signals and shall include visual inspection of the installation, check electrical supply and if found defective it shall include for appropriate temporary signs to be installed and the temporary switch-off signals together with appropriate signs and restore to operation on the instruction of the Overseeing Organisation. |
| 1453AR | - Random/Bulk Changing of LED Lanterns 1 Bulk replacement of LED lanterns shall only be carried out upon instruction of the Overseeing Organisation. 2 LED lanterns shall be of the appropriate voltage, wattage and specification rating, and shall be indelibly marked by the Contractor with the month and year of installation. All removed lanterns, together with any packaging, shall be returned to the Depot. Following inspection by the Overseeing Organisation, the Contractor shall be responsible for disposal in accordance with current legislation, ensuring suitable precautions are taken for electronic waste. During replacement, mounting arrangements and electrical connections shall be inspected, and any evidence of electrical stress, deterioration or mechanical damage shall be rectified or replaced. 3 The operation of the LED lantern shall be tested after installation. Should the lantern fail to operate correctly, associated electrical circuits, drivers, control gear, or components shall be inspected and replaced as necessary. 4 Random/Bulk replacement of LED lanterns shall include cleaning, inspection, and any maintenance activities specified in Clause 1454AR. . |
| 1454AR | Cleaning, Lubricating and Visual Inspection 1 Cleaning will be carried out on instruction of the Overseeing Organisation. All lantern bowls, refractors, reflectors, photoelectric detectors and other optical components affecting the performance of the luminaire shall be thoroughly cleaned internally and externally using suitable cleaning fluid approved by the Overseeing Organisation. No abrasive material shall be used. The attention of the Contractor is drawn to the dangers which may arise from the dropping of material and/or cleaning solution onto vehicles passing below or adjacent to the cleaning vehicle. After the use of the cleaning solution all surfaces treated shall be wiped with a clean muslin or soft cloth to remove all traces of the solution. External cleaning only will be required to the bowls of sealed units. The external surfaces of bollards and the faces of all signs illuminated externally or internally shall be cleaned. All control gear compartments of columns and feeder pillars are to be thoroughly cleaned. 2 The column/sign reference numbers, overhead cable and other warning notices where applicable shall be wiped clean, legible and fixing visually inspected. If found to be defective they shall be replaced. 3 Lubricate and properly tighten all toggles using nuts, hinges, door locks, set screws etc. on luminaires, columns and feeder pillars they shall be checked lubricated, properly tightened and replaced where required. 4 Examination of defects shall include the brackets, columns, luminaires, control gear, switch gear, feeder pillars and paintwork. |
|  | The electrical installation shall be checked to ensure compliance with current wiring regulations, electrical termination and insulation shall be examined, tightened and replaced where necessary, particular attention to the security of earthing termination. Examine fuses and include for supply and replacement where necessary with the correct type and rating, inform Overseeing Organisation of any outstanding defects. Columns, brackets and luminaires shall be checked for security and adjusted to correct alignment. Column and Pillar doors shall be checked for correct fitting and security. General conditions of all paintwork, finishes and numbering shall be checked. 5 The Overseeing Organisation must be notified of amendments and/or corrections to the street lighting and road furniture inventory for updating purposes. The details to be recorded with the aid of electronic data transfer compatible with the Overseeing Organisation. |
| 1455AR | Electrical Test of Lighting Units 1 Electrical testing of Lighting Units shall be carried out at intervals determined by the Overseeing Organisation and BS7671 but can coincide with visits under Clause 1453AR and 1454AR. 2 Where double pole isolation is required the Contractor is to inform the Overseeing Organisation who may instruct the Contractor to install a suitable double pole isolator. 3 Electrical inspections shall comply with requirements of BS7671 Requirements for Electrical Installations using approved and properly calibrated test equipment by suitable trained and competent persons (ERG37) and the results recorded with the aid of electronic data transfer compatible with the Overseeing Organisation. 4 All electrical terminations shall be checked for tightness; fuses shall be checked and replaced where necessary with the correct type and rating. Fuses shall be supplied by the contractor. 5 Columns brackets and luminaries shall be checked for correct fitting and security. 6 Column and feeder pillar doors shall be checked for correct fitting and security. 7 Any defects found not covered by Clause 1455AR shall be notified to the Overseeing Organisation within 2 days. |
| 1456AR | Structural Inspection of Lighting Units Routine Inspections 1 Routine structural inspection of Lighting Units shall be carried out at intervals determined by the Overseeing Organisation but can coincide with visit under Clause 1452 AR, 1453 AR, 1454 AR and 1455 AR. 2 The columns shall be inspected for any structural or mechanical defects in accordance with the Institution of Lighting Engineer GN22 (ATOMS) and condition and defects recorded electronically by data transfer compatible with the Overseeing Organisation. 3 Column bracket and luminaires shall be checked for correct alignment, security and adjustment of bracket and lantern carried out if required. 4 Column and feeder pillar doors shall be checked for correct fitting and security. Detailed Inspection 1 Detailed structural inspection of Lighting Works shall be carried out at intervals determined by the Overseeing Organisation, but can coincide with visits under Clause 1452AR, 1453AR, 1454AR and 1455AR. 2 The columns shall be inspected by a competent and experienced person in lighting column assessments for structural defects, in accordance with the Institution of Lighting Engineers GN22 (ATOMS) and report recorded on form HL2B or recorded electronically by data transfer compatible with the Overseeing Organisation. |
| 1457AR | Re-alignment of Lighting Units 1 Details of all items to be realigned shall be issued to the Contractor by the Overseeing Organisation and shall be completed within 10 days of issue to the Contractor. 2 Column and posts for signs realignment work may necessitate the disconnection/removal/re-connection of the supply cable. Where this cable is the property of Regional Electricity Company (R.E.C) the disconnection/removal/re-connection of the supply cable must be undertaken by the Regional Electricity Company (R.E.C). 3 The Contractor shall carefully excavate, by hand in any material, around the column/post down below the cable entry slot to the existing concrete foundation, taking extreme caution not to damage the electricity supply cables. The column/post should be examined for any damage below ground level which would make the column/post unsafe. The Contractor shall notify the Overseeing Organisation if the item is to be replaced. 4 Unless the Contractor has entered into a tripartite agreement for public lighting contestable works the Contractor must notify the Overseeing Organisation in order that arrangements may be made for attendance on site of R.E.C, on site works with R.E.C. will be co-ordinated by the Contractor. The excavation shall be kept free from water and loose material whilst the column/post is being restored to the vertical and shall be supported until back filling and compaction has been completed. 5 The Contractor shall be responsible for the making good any damage to the column/post or its protective system however caused by the Contractor during this operation. |
| 1458AR | No Supply 1 The Contractor shall be entitled to payment following attendance to a site, upon instruction by the Overseeing Organisation and being unable to affect a repair due to circumstances beyond his reasonable control. Such circumstances being agreed between the Overseeing Organisation and the Contractor. 2 Where the defect cannot be rectified due to R.E.C. supply failure/fault, the Contractor shall before the end of the working day report the supply fault/failure to the Overseeing Organisation and be recorded electronically by data transfer compatible with the Overseeing Organisation. 3 Supply failure/faults shall include no supply, faulty or damaged cutouts but not fuses. |
| 1459AR | Identification Numbering and Danger Warning Labels 1 All Lighting Units shall be identified by means of a numbering system, Class 2 reflective, adhered onto the units or by bonding to a plate with the number adhered to it. 2 The numbering system shall comprise of numerals on a white or yellow background panel according to category of lighting. 3 The numbering system shall be 75mm high coloured black onto white/yellow background for lighting columns and have a blue background for road traffic signs and bollards. 4 The number shall be located on the Lighting Units as follows: - (a) On Road Lighting Units the number shall be located min. m above ground level facing directional flow of traffic. (b) On Road Lighting Units mounted on wooden poles or buildings the number shall be located on the gear box door or other conspicuous location. (c) In the case of Lit Sign Units as directed by the Overseeing Organisation. 5 This item of work shall be carried out within 5 days of receipt of the instruction in the case of random numbering or within the specified period for bulk lamp and clean or at random visits due to faults. |
|  | 6 Where Lighting Units have been erected in close proximity to live overhead conductors or when the operation of a tower maintenance vehicle may encroach safety clearances “Danger Warning” labels shall be attached to the Lighting Units in a conspicuous location at approximately 3m height to draw attention to the operator of the need to risk assess the maintenance operation and to proceed accordingly to enable him to carry out his duties in a safe manner. The Contractor is to notify the Overseeing Organisation as soon as possible of any such Health and Safety issues to enable instructions to be issued to install warning notices and for recording and updating of inventory data. |
| 1460AR | Trimming of Trees/Shrubs Any necessary trimming of hedges or shrubs to gain access to the base compartment of Lighting Units, or other street furniture for the purposes of maintenance work shall be carried out by the Contractor who shall dispose of any trimming to an approved tip. Pruning of trees may be required around lanterns to maintain effectiveness of lighting, or around traffic signs faces to allow adequate visibility of signing, street furniture e.g. Where this operation can be carried out using secateurs or long-handed loppers or handsaws the cost involved are deemed to be included in the maintenance operation. Where it is necessary to use power saws to remove branches, this will be considered as additional work and a separate instruction will be issued. In either case the Contractor shall dispose of any trimmings to an approved tip. |
| 1461AR | Disposal of Waste The Contractor shall obtain written permission from the appropriate authority for the disposal of solid and liquid toxic and other waste and pay all waste disposal charges to comply with WEEE Regulations 2013 (as amended). |
| 1462AR | Special Columns Clause Numbers 1450AR to 1460AR shall apply to lighting mounted on hinged/lowering columns. The lowering tool, when required will be provided by the Overseeing Organisation, other safety requirements to be the responsibility of the Contractor. Keys shall be supplied by the Overseeing Organisation. The additional equipment such as hinges, locks and mountings shall be in addition to the equipment described in 1454AR. Details of special columns are listed in Appendix 14/9. At the end of the contract period all lowering tools, keys etc. shall be returned to the Overseeing Organisation. |
| 1463AR | Night Inspections (i) Programmed nighttime safety inspection shall be carried out by the Contractor and shall identify and record all lighting failures, poor light outputs and defects. Inspection routes should be organised to cover the maximum number of units over minimum vehicle mileage. All failures and defects shall be reported to the Overseeing Organisation electronically in an approved manner by 10:00am on the first working day following inspection. (ii) The inspections shall be carried out at intervals of 28 calendar days. (iii) The Overseeing Organisation may instruct the Contractor to increase the frequency of the Winter inspection and / or suspend the summer inspection. (iv) Works instruction will be issued by the Overseeing Organisation to cover monthly inspection period. (v) The Contractor is to provide to the Overseeing Organisation within 5 days of the commencement of the Contract a statement detailing the inspections, this should indicate the proposed route for each patrol or scout area, manning levels and the day of the week when the patrols will be carried out, the total number of patrols would be equal to one complete night inspection of all units. These routes are to be agreed with the Overseeing Organisation, and no alteration shall be made without the prior consent of the Overseeing Organisation. (vi) The Contractor should carry out a risk assessment on this operation and to satisfy himself as to the manning levels and safe method of work. The Contractor shall provide the Overseeing Organisation with copies of his assessment. |
| 1930AR | Painting Painting shall be carried out in accordance with Series 1900 and shall include for painting of new, weathered galvanized columns, previously painted columns and pole/wall brackets. |

SERIES 2700: - SPECIFICATION FOR COMMUNITY GANGS

2701	INTRODUCTION

1.0	Since 2007 the Isle of Anglesey County Council have reintroduced the old length men services and are now “The Community Gang”, 4 in total based to work in their own “engineer” area.  These current gangs have valuable local knowledge of their areas.

1.1	There are 4 inspector areas, each with their own Community Gang.

1.2	The Community Gang comprises of two number operatives, a suitable vehicle, tools and plant to carry out duties as instructed by the Overseeing Organisation, unless otherwise instructed.

1.3	Definitions

- “Street” “road” or “car parks” shall include all highways maintainable at public expense and any part thereof.

Typical works carried out by the Community Gang

- Opening grips to allow water to run from roads.

- Minor ditching work.

- Siding, this is the clearing of the roadside.

- Minor drainage work.

- Cutting vegetation by road signs.

- Repairing and cleaning road signs.

- Filling in holes in the road.

- Concreting around gullies.

- Minor repairs.

- Hand spreading salt as required.

- Investigating problems.

- Work which can be completed in a day.

- Work which cannot be done by machine.

1.4	Other maintenance work

It should be emphasised that the Community Gang will not replace the usual gangs who carry out highway maintenance work.  Work of a more substantial nature, involving larger items of equipment and of greater cost will still be undertaken by the other gangs.

These works along with other minor works will still be carried out promptly and to an acceptable standard.

1.5	The Community Gang is expected to attend site/inspector’s area on the Isle of Anglesey between the hours of 7:30am-5pm Monday to Thursday and 7:30am-3:30pm Friday, travelling to and from site shall be carried out outside these hours.

1.6	The Community Gangs have compounds in Menai Bridge and Gaerwen Depots and one at Pencaledog (payable by the Contractor, currently £1,856.40 per annum).

These compounds are served by a metal container, these are used to store tools, signs etc. for the use of these gangs.

Each Community Gang is to be supplied by the Contractor the following tools at no additional cost to the gang item.

List of Tools

a)  Mixer and stand

b)  Wacker Vibrating Plate

c)  Brush Cutter/Blades

d)  Concrete Cutter/Blades - water container

e)  Metal Detector

f)   Metal Steel Container

g)  CAT detectors/Genny’s

The Contractor will service and maintain these items during the contract to the required standards at their cost.  Vehicles for use by the Community Gang, provided by the Contractor and must be able to carry at least 2T without tools, to include storage for clothing and PPE.

1.7 	The Area Engineer (AE) conduct monthly inspections on a variety of roads and footways. These inspections need to be conducted within a given time scale and these need to be adhered to by the AE. Their workload throughout the month will dictate when and where these inspections take place within the working week (Monday to Friday). Through these inspections, repairs are noted and need to be completed to a given time.

These repairs vary and repair times are described within the current Highway Inspection Policy (HIP) and vary from 2-hour response, 24-hour response, 7- and 28-day response.

The timescales within the inspection and repairs are used as part of the Authority’s defence against third party claims and these response times must be adhered to.

Currently the Authority uses a third-party software system “Alloy” which generates inspections each month as described and following the HIP. These roads can be Class 1, 2, 3 and unclassified roads and footways and vary in traffic usage.

The AE completes their distributed inspections, through electronic tablets using mobile internet access, on an ad hoc basis throughout the month and generate defect orders, which are transmitted electronically to the contractor and the community gangs (CG). The tablets and software required by the CG will be borne by the Contractor.

These defect orders can then be filtered to supply response times and locations to rectify these defects following the HIP timescales.

It is in both parties’ interest to ensure that the defects are repaired within the timescales, and both the AE and CG will work closely together to ensure that these response times are met. They will, jointly, assess the defect work programme so that the inspections and defect rectification list is not too exhaustive so that the repair times are manageable.

Ultimately, it is the responsibility of the contractor to manage the effectiveness of the CG and the productivity, and since the provision of the CG is on a day rate basis, they need to show value for money.

Under the CDM regulations the CG will be noted as contractor and all relevant duties will fall under the duties as such.

SERIES 2800: - SPECIFICATION FOR GULLY CLEANSING SERVICE

2801	General

1.0	Gully Cleansing Service comprises of the cleansing of gullies, traps, buckets, manholes, catch pits, interceptors, flow control devices and polychannels throughout the site on an annual (or 6 monthly) cycles unless otherwise instructed.

Definitions

2.0	"Street" or "road" shall include all highways maintainable at the public expense and any part thereof.

2.1	“Car Parks”

3.0	"Cleanse" or "cleaning" shall include: -

3.1	The emptying and cleansing of gullies, traps, buckets, manholes, catch pits, polychannels, interceptors and any other drainage items specified by the Overseeing Organisation.

3.2	The emptying and cleansing of gullies, traps, buckets, manholes, catch pits, interceptors, flow control devices and polychannels, and any other drainage item specified by the Overseeing Organisation in emergency situations, e.g. following chemical spillages.

3.3	The disposal of all detritus to a controlled disposal site which must be authorised in accordance with the Control of Pollution Act 2012 and the Environmental Protection Act 2011 or any amendment thereof.

4.0	"Conventional Mechanical" shall mean the use of equipment which is similar in size and performance to a whole 1800 gallon/8150 litres compartment ratio: 3650 litres clean 4500 litres dirty capacity gully emptier, mounted on the appropriate vehicle chassis with means of GPS equipment, with tolerance of less than a metre as approved by the Overseeing Organisation.

The Overseeing Organisation current used an asset management system “Korec” to carry out checks on gully location and cleansing this system provides:

A position update with ignition on every 1 minute, giving driving speed and position on a map and in a report.

A mapped trail of each journey along with a street-by-street report is to be available.

Detailed position of gully cleaned.

Duration of gully cleaning.

Data available to import into your GIS or a CSV report.

This cost is to be borne by the Contractor.

Height indicator: Visual warning in cab to alert driver if overhead equipment or tipping body not stowed correctly, to comply with regulations Construction and Use 1986 Amendment SI 1997 No 530.

Gully arm type: 5” full bore, 300-degree rotation, counterbalanced and air assisted.  Sludge breaker fitted to downpipe.

Water intake: Approved no back syphon hydrant filler.

Jetting pump: 240 high pressure pump (120 l/min at 120 bar).

Main hose reel: 100m x 3/4” HP Sewer Jetting hose.

2nd hose reel: Manually operated reel on nearside with 30m x 1/2” high pressure hose.

Equipment: Hand lance with fan and needle nozzles.

Safety: Emergency engines stop in rear control panel.

Other storage: Storage for road signs.

Regulations: Construction and use S.I. 128-2000.

P.E.D.: To comply with the Pressure Systems Regulations S.I. 2002 No. 1267 Directive No. 97/23/EC.

Quality Assurance: Design and Construction insurance approval plate.

Lighting: Amber flashing beacons fitted at rear on offside and nearside.  Worklight fitted on gully boom.  Worklight fitted at rear.

Hose quantity: 3 off 3m x 4” Diameter Plastic Hose with URT connections.

Other features: Standpipe including double check valve and key.  Automatic PTO disengagement linked to handbrake.  Electrics handwash unit.  Adjustable traffic directing arrow to rear, 750mm diameter.

2802	Performance Requirements and Standards

Standard of Cleansing

1.0	Gullies, traps, buckets, etc. shall be cleansed by conventional mechanical means unless specified otherwise by the Overseeing Organisation.

2.0	The standard method of cleansing shall be: -

2.1	To remove grating/cover with appropriate tools.

2.2	To remove material lodged in or around the grating/cover.

2.3	To cut back roadside vegetation hindering access to the chamber.

2.4	To extract silt, debris and detritus from the chamber by suction or by other means.

2.5	To flush the chamber with water to ensure that the connection is clear and to refill the chamber with clean water if appropriate.  In the case of trapped gullies checks shall be carried out to ensure the plug is fitted correctly into the rodding eye.

2.6	To apply a thin layer of grease to the sides of the grating/cover which shall then be replaced flush into the frame.

2.7	To sweep or clean the surface around the chamber to leave it in a clean and tidy condition.  All debris, silt and detritus shall be disposed of to a controlled disposal site provided by the Contractor.

3.0	The Contractor shall inform the Overseeing Organisation of any missing or defective chambers, gratings or covers, or any blockages in the drainage system.

3.1	Replacement or repair of missing or defective chambers, gratings or covers shall be carried out as instructed by the Overseeing Organisation.

3.2	When instructed by the Overseeing Organisation, blockages in the drainage system shall be cleared by jetting or other means.

4.0	FREQUENCY OF CLEANSING

4.1	Gullies are to be cleansed on an annual basis but may be more frequent than in 6 monthly cycles unless otherwise specified.  In certain areas, identified separately in schedules, gullies, traps and buckets shall be cleansed monthly during the period from September to March inclusive, with one additional visit in June.

4.2	Polychannels or similar units shall be cleansed on annual basis but may be more frequent than in 6 monthly cycles unless otherwise specified.

4.3	Manholes and catch pits shall be cleansed when and at the locations instructed by the Overseeing Organisation.

4.4	Interceptors and flow control devices to be cleaned biannually as per schedule.

2803	TRAFFIC SAFETY AND MANAGEMENT

1.0	Work on traffic sensitive county roads will not be allowed during the period between 1 July and 7 September without the written approval of the Overseeing Organisation.

2.0	The Contractor's programme should take account of obstructions in the highway such as parked vehicles, skips, roadworks, etc. and must report back to the Overseeing Organisation within seven days with a list of the obstructions.  The Contractor must then re-programme the works and report back to the Overseeing Organisation within 30 days.

2804	COMPLAINTS AND COMMENDATIONS

1.0	The Contractor shall keep a written record of all complaints and commendations received and of the action taken to resolve such complaints.  The record shall be kept available for inspection by the Overseeing Organisation at all reasonable times.

2805	METHOD STATEMENT

The Contractor shall provide details of his operations in the form of a Method Statement within 21 days of the commencement of the Works for the Overseeing Organisation's approval and annually within the first 2 weeks of April.  Risk Assessment should be incorporated into the Contractor's Method Statement.  The Contractor shall produce a 6 monthly schedule for the Overseeing Organisation’s approval.

SERIES 2900: - SPECIFICATION FOR EMERGENCY SERVICE

2901 - GENERAL

This Specification covers the Contractor's obligation in respect of the response required in emergency situations.

2902 - DEFINITIONS

In the Contract, the following words and expressions shall have the meanings assigned to them except where the context otherwise requires: -

a)	"Emergency Service" means the stand-by and call-out service provided by the Overseeing Organisation of the Isle of Anglesey County Council to deal with emergency situations occurring within the County.

b)	"Emergency Situation" means an event occurring requiring immediate action.

c)	"Response Time" (within 1.0 hour to site) means the time taken from when the instruction was given by the Overseeing Organisation to the time of beginning the Emergency Service on site at the location of the "Emergency Situation”.

d)	"Contact Officer" means the Contractor's employee who receives and administers instructions from the Overseeing Organisation in Emergency situations.

e)	"Emergency Crew" means the Contractor's plant, labour and materials providing the Emergency Service.

2903 - TYPICAL EMERGENCY SITUATIONS

The following are typical emergency situations which require provision of the Emergency Service: -

a)  vehicular accidents which could also involve oil or chemical spillages, fire or physical damage to the highway fabric, road lighting columns, traffic signs and bollards.

b)  tree clearance due to storm conditions.

c)  clearance of debris.

d)  flood water - reactive works response.

e)  obstruction of the highway due to events to boundary properties such as fire or storm	damage.

f)  major emergency situations requiring emergency technical support, including weather events.

g)  traffic management failures such as traffic signals.

h)  failure of Statutory Undertakers apparatus such as water main failure.

i)   clearance of blocked sewers and drains.

j)  snow clearance to supplement winter maintenance operations.

k) clearance of blood. A specialist contractor may have to be deployed by the Contractor.

l)  vandalism, damage by third party to equipment including “doors off” lighting columns, fallen lighting columns etc.

This list is not exhaustive, and the Contractor could be instructed to attend other events not detailed above.

2904 - CONTRACTOR'S OBLIGATIONS

1.0	The Contractor shall ensure that stand-by cover is available 24 hours a day on every day of the year.

2.0	The cover shall include: -

(i)    a Contact Officer who is authorised to receive instructions from the Overseeing Organisation and who can pass on these instructions to the Emergency Crew.

(ii)   a two-man Emergency Crew together with vehicles (and trailers) and emergency service equipment and.

(iii)  a three-man Emergency Crew together with vehicles (and trailers) and emergency service equipment.

(iv) must include a lighting operative (electrician) and access to a hi-ab vehicle or similar.

3.0	Emergency service equipment shall include: traffic signs, cones, lamps, sand bags, oil absorbent material, drain rods, small hand tools such as hammers, sledge hammers, shovels, hand saws, crowbars and the like and power tools such as chainsaws and disc cutters to enable (where practicable and safe) minor works or debris removal to be effected to ensure that the Emergency Situation is made safe and traffic flow maintained or restored as soon as is possible subject to the agreement of the Overseeing Organisation.

4.0	Adequate traffic signs shall be provided and maintained by the Contractor to deal with traffic management on single and three lane carriageway roads.  All signing work shall comply with Chapter 8, The Traffic Signs Regulations and General Directions 2004 and the Traffic Signs (Welsh and English Language Provisions) Regulations and General Directions 2004.  The Welsh legend should appear uppermost on all signs.

5.0	When the Emergency Crew have made the situation safe and further remedial works are necessary, the Overseeing Organisation will issue a Works Task to the Contractor to undertake the remedial works.  The Contractor should be aware that the Overseeing Organisation may issue instructions to carry out the remedial works outside normal working hours should the situation dictate.

6.0	The Contact Officer and Emergency Crew should be suitably experienced, trained and qualified to deal with emergency situations including dealing with hazardous materials.

7.0	The Contractor shall notify the Overseeing Organisation of the name(s), address(es) and the emergency telephone number of the Contact Officer/s.  Any change in personnel or location of the personnel shall be notified to the Overseeing Organisation in writing.

2905 - RESPONSE TO EMERGENCY CALL-OUT

1.0	The Contractor shall ensure that the Emergency Crew can attend any location

within the County within 1.0 hour of receipt of receiving an instruction from the Overseeing Organisation.

2.0	The Contractor shall, thereafter, as soon as is practicably possible, keep the Overseeing Organisation informed of the Emergency situation and be prepared to act on further instructions of the Overseeing Organisation.

3.0	In the event of accident damage the Contractor shall wherever possible obtain and pass in writing to the Overseeing Organisation, details of any personnel vehicles or property or highway fabric damaged or involved in the Emergency situation.  Where the Police are present the Contractor shall also forward names, rank and number of the officers attending the incident.

4.0	Details of the Emergency Service action, including labour, plant and materials and response times records shall be reported to the Overseeing Organisation by 10am the next working day.

5.0	The Contractor shall collect all traffic management signs after all emergencies, including floods etc.  These works are to be carried out at no cost to the Overseeing Organisation and removed on the 7th day at the contractors’ cost, unless otherwise instructed by the Overseeing Organisation.

6.0	The Contractor will keep a rota of labour on call throughout the year and implement an on-call system at no extra cost to the Overseeing Organisation.

7.0	The provision of service shall include the cost to providing the emergency services

8.0 	Rest periods shall be included in the call out order as a works item to be recoverable by the Contractor.

SERIES 3100: - SPECIFICATION FOR WINTER SERVICE

3101	GENERALS

1.0	The Specification sets out the standards to be achieved in respect of Well Managed Infrastructure and the Winter Service Plan of the Isle of Anglesey County Council as amended.

2.0	Isle of Anglesey County Council's aim is to provide efficient Winter Service which will permit the safe movement of traffic based on route importance and priority needs.

3.0	Definitions

Winter Service is defined as: -

3.1	"Precautionary salting" - treatment to prevent the formation of ice.

3.2	"Post salting" - treatment to melt ice and snow already formed.

3.3	"Snow Clearance" - removal of accumulated snow by plough or other method.

3.4	"Successive Salting" - continuous salting to melt hard packed snow and ice.

3.5	"Gritting" - treatment with salt or single size abrasive aggregate or sharp sand.

3.6	“Handwork” - treatment with salt or single size abrasive aggregate or sharp sand.

3.7	“Duty Officer” - refers to the designated officer on out of hours duty rota for winter maintenance and emergencies.

3.8	“Basic Facility” is the provision of a winter service.

4.0	WINTER SERVICE PROVISION

4.1	A sum will be paid for the provision of a “winter maintenance service” payable in seven instalments from October to April inclusive.  If gritting is required outside this period, it will be undertaken by the Contractor at no additional cost. It is envisaged that this will cover the cost of having the necessary operatives on call during this period including supervision along with the necessary means of communication to receive instruction within and outside hours.

4.2	The normal period during which winter service operations shall be required is the 1 October to 30 April of each year of the contract unless otherwise instructed by the Overseeing Organisation or the Contract period dictates otherwise.  Winter service works shall be carried out during both normal working hours and outside normal working hours at night, Saturdays, Sundays and Bank Holidays as determined by the ambient conditions and directed by the Overseeing Organisation under Clause 3102.

4.3	From October 2008 the Authority have decided to use “Safecote” as an additive to the gritting process to improve the effectiveness of the gritting process.  It is envisaged that this use will continue.

5.0	NETWORK PRIORITY HIERARCHY FOR WINTER SERVICE OPERATIONS

The Overseeing Organisation has designated a hierarchy of carriageways, footways, and other paved areas, with differing levels of priority for winter service operations as defined below: -

5.1	First Priority

5.1.1	To be treated on the instruction of the Overseeing Organisation when adverse winter conditions are predicted: -

a)	County Class 1 and 2 roads as identified in the Winter Service Plan.

b)	Other important connecting roads as identified in the Winter Service Plan.

(i)	are through traffic routes with high volume of traffic.

(ii)	provide at least one access to centres which respond to emergencies or receive emergency admission.

(iii)	County Class 3 roads which provide at least one access to towns and villages.

(iv)	 Roads identified as requiring attention due to gradients.

5.2	Second Priority

5.2.1	To be treated on the instruction of the Overseeing Organisation when adverse weather conditions persist at noon (after 24 hours of a First Priority Instruction) where the surfaces have not been treated within the preceding 24 hours: -

a)	Main shopping centre footways and pedestrianised areas as identified in the Winter Service Plan.

b)	other areas as identified by the Overseeing Organisation’s instruction.

5.2.2	Second Priority Routes will be treated on completion of treatment to First Priority Routes.

5.3	Third Priority

5.3.1	To be treated on the instruction of the Overseeing Organisation when adverse weather conditions persist at noon (after 36 hours of a First Priority Instruction) where the surfaces have not been treated within the preceding 24 hours: -

a)	other areas as identified by the Overseeing Organisation’s instruction

b)	Car parks as identified by the Overseeing Organisation’s instruction.

5.3.2	Third Priority Routes will be treated on completion of treatment to Second Priority Routes.

6.0	METHOD STATEMENT OF CONTRACTOR'S OPERATIONS

6.1 	The Overseeing Organisation shall supply gritting vehicles with snow ploughs, as described in Clause Z40.  The vehicles shall be maintained fully in compliance with the current Road Traffic Regulations by the Contractor.

6.2	The Contractor shall provide plans and operational procedures for the routes in the form of a method statement within 21 days of the commencement of works for the Overseeing Organisation’s approval and annually within the first two weeks of September.  Risk Assessment should be incorporated into the Contractor's Method Statement.

7.0	SALT BINS FOR PUBLIC USE

7.1	Salt bins provided by the Overseeing Organisation shall be located as indicated in the Winter Service Plan.

7.2	The Overseeing Organisation will provide salt, and the Contractor shall provide other materials as required for the contract together with the items of plant and equipment to comply with response and treatment times.

3102	COMMUNICATION AND OVERSEEING ORGANISATION'S INSTRUCTIONS

1.0	The Overseeing Organisation will designate on a weekly basis the Duty Officer who will be responsible for winter service operations.  The Duty Officer will be on 24-hour stand-by.  No member of staff other than the Duty Officer should be contacted outside normal working hours, except in exceptional circumstances.

2.0	During normal working hours 9:00am to 5:00pm Mondays to Fridays (except statutory holidays) the Duty Officer can be contacted at the Council's headquarters (telephone number (01248) 750057).  Outside normal hours the Duty Officer will be based at his place of residence, and the telephone number will be (01248) 750057 or as directed by the Overseeing Organisation.

3.0	The Contractor shall provide telephone numbers and mobile numbers to which winter service instructions can be transmitted by the Overseeing Organisation or the Duty Officer.  Telephones and mobiles shall be continuously manned on a 24-hour basis by a person nominated to be responsible for Winter Service to receive and implement instructions.

4.0	Instructions for the level of service are at the discretion of the Overseeing Organisation or the Duty Officer and will be dependent on weather conditions predicted by the meteorological office or others.  Verbal instructions or e-mails will be confirmed by written notice by the Overseeing Organisation or the Duty Officer.

5.0	If during operations, the weather conditions change, the Duty Officer must be informed immediately by the Contractor.

3103	PERFORMANCE REQUIREMENTS AND STANDARDS

Precautionary Salting, Post treatment salting, snow clearance, successive salting and gritting

1.0	The winter service operations are as defined in the Winter Plan, and the Contractor will be instructed which operation is to be implemented on each occasion by the Overseeing Organisation or his Duty Officer.

2.0	AREAS OF SPREAD

Salt and other gritting materials shall be spread uniformly over the surface of the carriageway, footway or other paved area.

3.0	TIMESCALES

The Contractor shall complete winter service operations within the following response and treatment times:

3.1	First Priority

3.1.1	Precautionary salting shall commence within 1 hour of instruction and be completed within 31/2 hours of commencement.

3.1.2	Post salting shall commence within 1 hour of instruction and be completed within 31/2 hours of commencement.

3.1.3	All other treatments shall commence within 1 hour of instruction and continue until further instruction from the Overseeing Organisation.

3.2	Second Priority

3.2.1	Precautionary salting shall not be carried out on second priority areas.

3.2.2	Post treatment salting shall commence after completion of first priority and shall be completed within 4 hours of commencement.

3.2.3	All other treatments - to commence within 3 1/2 hours of instruction and to continue until further instruction from the Overseeing Organisation.

3.3	Third Priority

3.3.1	Precautionary salting shall not be carried out on Third Priority areas.

3.3.2	Post treatment salting shall commence on instruction from the Overseeing Organisation.

3.3.3	All other treatments to commence within 2 hours of instruction and to continue until further instruction from the Overseeing Organisation.

4.0	General Requirements

4.1	Salt spreading width and pattern shall be adjusted from the vehicle to suit the width of carriageway, footway or other paved area.  Care should be taken to ensure strong crosswinds do not affect the distance that treatment is spread.  Spreading machines shall be calibrated annually, by the Contractor at paid for by the Overseeing Organisation, and clearly marked for distinct rate of spreads, up to a maximum of 45g/m2.

4.2	In the event of vehicle breakdown the Contractor shall arrange for alternative provisions, but two vehicles are spare at Gaerwen Depot.

4.3	Snowploughs shall be fitted with rubber skirts.  Plough blades shall always be locked in the raised position when travelling between sites.

4.4	Snow clearance operations may involve prolonged periods of continuous work, and the Contractor is required to ensure adequate manning levels are available for this purpose.

4.5	In the event of, or during extreme conditions of snowfall and on the instructions of the Overseeing Organisation, sufficient manpower, vehicles and plant of appropriate capacity will be mobilised to priority sites as directed.  Clearance of sites would be carried out strictly in accordance with instructions and where sufficient storage space is available on site, snow would be stockpiled with due regard to maintaining access, safety of the public or occupants and the effects of later snow melt and resultant run-off.  Where storage facilities are not readily available, snow would be removed from site to an appropriate tip.  As and when instructed, cleared areas would be salted and maintained in a safe condition.

4.6	Clearance of snow by ploughing shall be based on full widths coinciding with existing carriageway widths wherever possible.  All roads will be ploughed to the nearside verge where possible.

4.7	The Contractor must report to the Overseeing Organisation or Duty Officer regularly at 2-hour intervals on severity of conditions.

4.8	The Contractor may be instructed to provide additional manual labour above the normal total Highway Maintenance Contract workforce for manual snow clearing and salting operations.

4.9	The Contractor may be instructed to hire snow moving equipment from outside sources.  Operators of this equipment should be given precise orders and a responsible member of the Contractor's staff be detailed to direct work and keep a log of hours worked by operators and machines.

4.10	In the event of emergency or severe winter conditions as confirmed by written notice of the Overseeing Organisation, the Contractor will be required to co-operate with the Overseeing Organisation and/or Police or other Emergency services.  This may involve the use of the Contractor's staff, offices, plant and equipment and the provision of additional plant, equipment and labour as required.  In such circumstances, the Contractor will be paid on daywork rates for all reasonable works not included elsewhere in the Contract.  However, these works shall not be the sole prerogative of the Contractor, and the Overseeing Organisation reserves the right to employ other contractors for this purpose.  The Contractor shall co-operate to ensure the smooth running of snow clearing operations by all the contractors.

4.11	Should it become apparent that snow is building up to such an extent that traffic flow is disrupted, or blizzard conditions exist, the Overseeing Organisation should be informed by the Contractor.

4.12	Should it become impractical or dangerous to continue ploughing, the Contractor should advise the Overseeing Organisation and request further instructions.

3104	REPORTING REGIME

After each operational day the Contractor shall complete a report sheet and return such to the Overseeing Organisation by noon on the following day.  The report shall include: -

1.0	Vehicle route/time information.

2.0	Stock use information.

3.0	Details of any non-compliance with performance requirements and standards and the reason for same.

4.0	Any other reasonable information required by the Overseeing Organisation.

3105	SUPPLY AND STORAGE OF SALT AND OTHER GRITTING MATERIALS

1.0	The Contractor will be responsible for the maintenance of the salt barn and all stored salt and other materials for the duration of the Contract.  Plans of the location and layout of the salt barn are contained in Drawing S4.

2.0	The Contractor shall ensure that the site is kept in a safe and tidy state and areas outside the salt barn are kept free of salt.

3.0	The Overseeing Organisation will be responsible for ordering and maintaining salt stock levels with a minimum stock level of 2000 tonnes at all times during the winter service period.

4.0	The amount of salt used shall be notified to the Overseeing Organisation daily by the Contractor when winter service operations are undertaken.  The Overseeing Organisation will ensure at the start of the contract period that approximately 5500 tonnes of salt is available in the barn. The Overseeing Organisation will replenish the barn each year.

5.0	Salt and other gritting materials shall be placed in a pile in a safe manner to prevent the formation of steep faces liable to collapse.

6.0	The Contractor will provide enough suitable loading machines and operatives which must be available to load vehicles or receive deliveries.

7.0	The Contractor shall allow for stacking of salt and other gritting materials and for the rotation and double handling of stocks at any time to ensure the proper storage and use of the material.

3106	HEALTH AND SAFETY

1.0	The Contractor shall provide protection of the public during Winter Service operation.  Problems which could arise include:

(i)	spraying pedestrians with salt or slush when gritting or ploughing.

(ii)	hazards to pedestrians using vehicles in pedestrian areas including footways.

(iii)	ploughing/manoeuvring in restricted areas.

(iv)	other road vehicles not under proper control.

(v)	children playing in the snow.

2.0	The Contractor shall give appropriate training to his personnel in such matters as:

(i)	the safe and efficient use of plant and equipment (including radio communication).

(ii)	driving in hazardous conditions (including a duty of care to other road users).

(iii)	awareness of statutory undertakers' operations.

3.0	The Contractor shall carry out a risk assessment of all the winter operations and details of his proposals to reduce the risks to his personnel shall be submitted to the Overseeing Organisation by 15 October each year.  The assessments should give due consideration to the manning levels.  For different operations, in particular snowploughing and when conditions are hazardous (e.g. when precautionary or emergency treatment is required on particularly isolated stretches of road, where difficult manoeuvres are unavoidable or when visibility is poor).

4.0	When vehicles are double manned the second operative must not be the next rota driver for winter service duties.

3107	Operator Training, Assessment and Registration

1.0 	Winter Service Vehicles shall only be operated by trained staff.  Winter Service operators and drivers shall have been trained and assessed to meet the requirements of the Winter Service Operators Qualification awarded by the City and Guilds Institute (Scheme 6159) or equivalent recognised qualification by a state of European Union.  Qualified operators and drivers shall be registered with the Local Government Management Board.

2.0	Driver hours and the working time directive costs are to be noted and passed to the Overseeing Organisation using the item cost for rest periods.

3.0	The Overseeing Organisation, expects to have available during the next working day Community Gangs available, and that at least one operative is a regular member of the gang for the Area.

APPENDIX 0/2:

CONTRACT-SPECIFIC MINOR ALTERATIONS TO EXISTING CLAUSES,

TABLES AND FIGURES INCLUDED IN THE CONTRACT

Clause No. Table or Figure No. Alterations to be made

APPENDIX F

The following additions shall be made to the list of publications in Volume 1, Appendix F.

British Standards

BS 5969	Specification for sound level meters

BS 5837	Guide for trees in relation to construction

Departmental Standards

HA 51/93	The introduction of the Design Manual for Roads and Bridges

HA 48/93	Maintenance of Highway Earthworks and Drainage

HA40/89	Determination of Pipe and Bedding Combinations for Drainage Works

SA 3/93	Testing in Highway Contracts

TD 21/85	Portable Traffic Signals at Roadworks on Single Carriageway Works

ILE 		Code of Practice for Electrical Safety in Highway Electrical Operations

1401	Sub-Clause 2(iv) insert “and Isolators”

APPENDIX F

British Standards (or revised)

BS 7671	            Requirements for Electrical Installations

BS 4553		P.V.C Insulated Split Concentric cables

BSEN 61672		Specification for sound level meters

BS 5837		Guide for trees in relation to construction

BSEN 50363		Polythene Insulation of Electric Cables Highway Electrical

BSEN 62560               Safety requirements for self‑ballasted LED lamps

BSEN 62612               Performance requirements for retrofit LED lamps

BSEN 60598               General safety requirements for LED luminaires

BSEN 62722               Performance of LED luminaires

BSEN 62384               Performance requirements for LED drivers (control gear)

BSEN 62471               Photobiological safety of LED lamps and lamp systems

BSEN 60529               Ingress protection (IP ratings) for optical and gear compartments

APPENDIX 0/3:

LIST OF NUMBERED APPENDICES REFERRED TO IN THE SPECIFICATION

AND INCLUDED IN THE CONTRACT

Appendix 0/3 is comprised of two lists, A and B, of Numbered Appendices as follows:

List "A" is a complete list of the Numbered Appendices referred to in the Specification for Highway Works with those not adopted marked "Not Used".  Those identified by the letters T or C shall be completed by the Tenderer or Contractor respectively

Guide to types of Numbered Appendices - who compiles/completes.

Symbol

(Co)		Compiler compiles: Identified in the Notes for Guidance examples by the term ‘Sample’ included in their title.

(Co/C)		Compiler partially compiles and Contractor completes and returns to Overseeing Organisation.

(Co/T)		Compiler partially compiles and Tenderer completes and returns with Tender.

(C)		Contractor completes and returns to Overseeing Organisation.

(P)		This indicates the Appendix is a national proforma and format must not be altered.

List "B" gives the list of Contract-specific Numbered Appendices devised for the Contract.

List "A": List of Numbered Appendices Referred to in the

Specification for Highway Works

| Completed by | Appendix Number | Title |
| --- | --- | --- |
|  |  | INTRODUCTION |
| (Co) | 0/1 | Contract-specific additional, substitute and cancelled Clauses, tables and figures included in the Contract. |
| (Co) | 0/2 | Contract-specific minor alterations to existing Clauses, tables and figures included in the Contract. |
| (Co) | 0/3 | List of numbered Appendices referred to in the Specification and included in the Contract. |
| (Co) | 0/4 | List of drawings included in the Contract. |
| (Co) | 0/5 | Special National alterations of the Overseeing Organisations of Scotland/Wales/Northern Ireland. |
|  |  | PRELIMINARIES |
|  | 1/1-1/4 | All not used |
| (Co) | 1/5 | Testing to be carried out by the Contractor. |
| (Co) | 1/6 | Supply and delivery of samples to the Overseeing Organisation. |
| (Co) | 1/7 | Site extent and limitations on use. |
| (Co) | 1/8 | Operatives for the Overseeing Organisation. |
| (Co) | 1/9 | Control of noise and vibration |
| (Co) | 1/10 | Structures to be designed by the Contractor. |
| (Co) | 1/11 | Structural elements and other features to be designed by the Contractor. |
| (Co) | 1/12 | Setting out and existing ground levels. |
| (Co) | 1/13 | Programme of works. |
| (Co) | 1/14 | Payment applications. |
| (Co) | 1/15 | Accommodation works. |
| (Co)(C) | 1/16 | Privately and publicly owned services and supplies. |
| (Co) | 1/17 | Traffic safety and management. |
| (Co) | 1/18 | Temporary diversions for traffic. |
| (Co) | 1/19 | Routing of vehicles. |
|  | 1/20 | Not used |
| (Co) | 1/21 | Information boards. |
| (Co) | 1/22 | Progress photographs. |
| (Co) | 1/23 | Risk to Health and Safety from materials or substances. |
| (Co) | 1/24 | Quality Management System. |
|  | 1/25-1/27 | All not used |
|  |  | SITE CLEARANCE |
| (Co) | 2/1 | List of buildings, etc. to be demolished or partially demolished. |
| (Co) | 2/2 | Filling of trenches and pipes. |
| (Co) | 2/3 | Retention of material arising from site clearance. |
| (Co) | 2/4 | Explosives and blasting. |
| (Co) | 2/5 | Hazardous materials. |
|  |  | FENCING |
| (Co) | 3/1 | Fencing, gates and stiles. |
|  |  | ROAD RESTRAINT SYSTEMS (VEHICLE AND PEDESTRIAN) |
| (Co) | 4/1 | Road restraint systems (vehicle and pedestrian). |
| (Co) | 4/2 | Information required to demonstrate compliance of road restraint systems to BS EN 1317-1, BS EN 1317-2, BS EN 1317-3 and DD ENV 1317-4:2002 |
|  |  | DRAINAGE AND SERVICE DUCTS |
| (Co) | 5/1 | Drainage requirements. |
| (Co) | 5/2 | Service duct requirements. |
| (Co) | 5/3 | Surface water channels and drainage channel blocks. |
| (Co) | 5/4 | Fin drains and narrow filter drains. |
| (Co) | 5/5 | Combined drainage and kerb systems. |
| (Co) | 5/6 | Linear drainage channel systems. |
|  | 5/7 | Not used |
|  |  | EARTHWORKS |
| (Co) | 6/1 | Requirements for acceptability and testing, etc. of earthworks materials. |
| (Co) | 6/2 | Requirements for dealing with Class U1B and Class U2 unacceptable materials. |
| (Co) | 6/3 | Requirements for excavation, deposition, compaction (other than dynamic compaction). |
| (Co) | 6/4 | Requirements for Class 3 materials. |
| (Co) | 6/5 | Geotextiles used to separate earthworks materials. |
| (Co) | 6/6 | Fill to structures and fill above structural foundations. |
| (Co) | 6/7 | Sub-formation and capping and preparation and surface treatment of formation. |
| (Co) | 6/8 | Topsoiling |
| (Co) | 6/9 | Earthwork environmental bunds, landscape areas and strengthened embankments. |
| (Co) | 6/10 | Ground anchorages, crib walling and gabions. |
| (Co) | 6/11 | Swallow holes and other naturally occurring cavities and disused my workings. |
| (Co) | 6/12 | Instrumentation and monitoring. |
| (Co) | 6/13 | Ground Improvement. |
| (Co) | 6/14 | Limiting values for pollution of controlled waters. |
| (Co) | 6/15 | Limiting values for harm to human health and the environment. |
|  |  | ROAD PAVEMENTS – GENERAL |
| (Co) | 7/1 | Permitted pavement options (Schedules 1, 2, 3, 4 and 5). |
| (Co) | 7/2 | Excavation, trimming and reinstatement of existing surfaces. |
| (Co)(C) | 7/3 | Surface dressing - performance specification (Sheets 1, 2 and 3). |
| (Co)(C) | 7/4 | Bond coats, tack coats and other bituminous sprays (Sheets 1, 2 and binder data sheet). |
|  | 7/5 | In situ recycling - the remix and repave processes. |
| (Co)(C) | 7/6 | Breaking up or perforation of existing pavement. |
| (Co)(C) | 7/7 | Slurry surfacing incorporating micro surfacing (sheets 1, 2 and 3). |
|  | 7/8 | Not used |
| (Co)(C) | 7/9 | Cold milling (planning) of bituminous bound flexible pavement. |
| (C) | 7/10 | Worksheet pro forma for results of testing for constituent. |
|  |  | Materials in recycled aggregate and recycled concrete aggregate. |
| (Co) | 7/11 | Overbend and inlaid crack sealing systems. |
|  | 7/12 | Not used |
| (Co) | 7/13 | Saw-cut crack and seal bituminous overlays on existing jointed concrete pavements. |
| (C) | 7/14 | Preparation of jointed concrete pavements prior to overlaying and saw-cut and seal of the bituminous overlay. |
|  | 7/15-7/17 | All not used |
| (Co) | 7/18 | Site specific details and requirements for cold recycled bitumen bound material. |
| (Co) | 7/19 | Site specific details and requirements for recycled cement bound material. |
|  | 7/20 | Not used |
| (Co)(C) | 7/21 | Surface dressing - recipe specification (Sheets 1, 2 and binder data sheet). |
| (Co)(C) (T) | 7/22 10/1 | Repairs to potholes. ROAD PAVEMENTS – CONCRETE AND CEMENT BOUND MATERIALS Plant and Equipment for the Construction of Exposed Aggregate Concrete Surface |
|  |  | KERBS, FOOTWAYS AND PAVED AREAS |
| (Co) | 11/1 | Kerbs, footways and paved areas. |
| (Co) | 11/2 | Access Steps. |
|  |  | TRAFFIC SIGNS |
| (Co) | 12/1 | Traffic signs: general. |
| (Co) | 12/2 | Traffic signs: marker posts. |
| (Co) | 12/3 | Traffic signs: road markings and studs. |
| (Co) | 12/4 | Traffic signs: cones, cylinders, FTDs and other traffic delineators. |
| (Co) | 12/5 | Traffic Signs: Traffic Signals. |
|  | 12/6 | Not used |
| (Co) (C0/C) (P) | 13/1 13/2 13/3 13/4-9 | ROAD LIGHTING COLUMNS AND BRACKETS, CCTV MASTS AND CANTILEVER MASTS Information to be provided by the Engineer when specifying Lighting Columns and Brackets. Columns and Brackets Data Sheets 1 and 2. Instructions for completion of Column and Bracket Data Sheet. Not used |
| (Co) (Co) (Co) (Co/C) (Co/C) | 14/1 14/2 14/3 14/4 14/5 | ELECTRICAL WORK FOR ROAD LIGHTING AND TRAFFIC SIGNS Site Records. Location of Lighting Units and Feeder Pillars. Temporary Lighting. Electrical Equipment for Road Lighting. Electrical Equipment for Traffic Signals. |
|  |  | MOTORWAY COMMUNICATIONS |
|  | 15/1-15/2 | Not used |
|  |  | PILING AND EMBEDDED RETAINING WALLS |
|  | 16/1-16/18 | All Not used |
|  |  | STRUCTURAL CONCRETE |
| (Co) | 17/1 | Schedule for the specification of designed concrete. |
|  | 17/2 | Concrete - impregnation schedule. |
| (Co) | 17/3 | Concrete - surface finishes. |
| (Co) | 17/4 | Concrete general. |
|  | 17/5-17/6 17/7 | Not used Precast Concrete Products. |
|  |  | STRUCTURAL STEELWORK |
| (Co) | 18/1 | Requirements for structural steelwork. |
|  |  | PROTECTION OF STEELWORK AGAINST CORROSION |
|  | 19/1-19/5 | All not used |
|  |  | WATERPROOFING FOR CONCRETE STRUCTURES |
| (Co) | 20/1 | Waterproofing for concrete structures. |
|  |  | BRIDGE BEARINGS |
| (Co) | 21/1 | Bridge Bearing Schedule. |
|  |  | PARAPETS |
|  | 22/1 | Not used |
|  |  | BRIDGE EXPANSION JOINTS AND SEALING OF GAPS |
| (Co) | 23/1 | Bridge deck expansion joints schedule. |
| (Co) | 23/2 | Sealing of gaps schedule (other than in bridge deck expansion joints). |
|  |  | BRICKWORK, BLOCKWORK AND STONEWORK |
| (Co) | 24/1 | Brickwork, blockwork and stonework. |
|  |  | SPECIAL STRUCTURES |
|  | 25/1-25/5 | All not used |
|  |  | MISCELLANEOUS |
| (Co) | 26/1 | Ancillary concrete. |
| (Co) | 26/2 | Bedding mortar. |
|  | 26/3 | Not used |
|  |  | LANDSCAPE AND ECOLOGY |
| (Co)(C)(P) | 30/1 | General sheets 1, 2 and 3. |
| (Co) | 30/2 | Weed control. |
|  | 30/3 | Not used |
| (Co) | 30/4 | Ground preparation. |
| (Co) | 30/5 | Grass seeding, wildflower seeding and turfing. |
| (Co)(C)(P) | 30/6 | Planting, sheets 1 and 2. |
| (Co) | 30/7 | Grass, bulbs and wildflower maintenance. |
| (Co) | 30/8 | Watering |
| (Co) | 30/9 | Establishment maintenance for planting. |
| (Co) | 30/10 | Maintenance of established trees and shrubs. |
| (Co) | 30/11 | Management of waterbodies. |
| (Co) (C)(P) (C)(P) (C)(P) (C)(P) (Co) | 30/12 50/1 50/2 50/3 50/4 50/5 | Special ecological measures. MAINTENANCE PAINTING OF STEELWORK (Specification for Highway Works) Form HA/P1 (Maintenance) Paint System Sheet. Requirements for Other Work. (Specification for Highway Works) Form HA/P2 Paint Data Sheet. (Specification for Highway Works) Form HA/P3 Paint Sample Despatch List: Sheets 1 and 2. General Requirements. |

LIST "B": LIST OF CONTRACT-SPECIFIC NUMBERED APPENDICES

DEVISED FOR THE CONTRACT

| Completed by | Appendix Number | Appendix Title |
| --- | --- | --- |
| (C) (C) (Co) (C0) (Co/T) (Co/T) (Co/T) (Co/T) | 0/6 1/13 12/9 13/7 14/6 14/7 14/8 14/9 14/10 27/1 28/1 29/1 31/1 | Contract Specific Forms to be Completed by Contractor. Programme of works. Traffic Signs. Traffic Signs: On site Equipment and Manufacturer details Road Lighting Columns and Brackets. On site Equipment and Manufacturer details Electrical Work for Road Lighting and Traffic Signs. Earthing Inspection, Commissioning and Testing. On site Equipment and Manufacturing Details. Special Columns. Location of Illuminated Road Signs. Community Gang. Gully Cleansing Service. Emergency Service. Winter Service. |

APPENDIX 0/4:

LIST OF DRAWINGS & TABLES INCLUDED IN THE CONTRACT

1.0	CONTRACT-SPECIFIC DRAWINGS & TABLES

SUPPLIED TO EACH TENDERER

| Drawing Number | Title |
| --- | --- |
| S1 S2 S3 S4 S4a S5 S6 S7 S8 S9 SVW1A & SVW2A G1 G2 G3 G4a and G4b G5 G6 Winter Service Plan Highway Inspection Policy | Isle of Anglesey Highway Infrastructure Public Rights of Way Infrastructure Depot Locations Menai Bride Depot Gaerwen Depot Notification Sign Locations of Safety Fences Community Gang Vehicle Logo Partnership Board Locations of Community Gang Compounds Areas 1-5 Plan to show Height and Weight on Structures Single Vehicle Working Traffic Management Layouts Schedule of Gullies List of Locations of Grillages Weed killing Schedule Schedule of Gully Emptying List of Location of Tyzaks and Polychannels Plans of Locations of Interceptors Winter Service Plan Highway Inspection Policy |
| SLSD1 SLSD2 SLSD3 SLSD4 1 8 SLSD4 9 SLSD5 0 SLSD5 1 | Baselit Bollard Installation and Access/Inspection Chamber detail Feeder Pillar Typical Layout Typical Lighting Column Foundation Detail Aids to Movement Standard Detail Standard Detail Street Nameplates Standard Details Street Nameplates Aids to Movement Standard Detail |

2.0	STANDARD DRAWINGS

2(i)	Supplied to Each Tenderer

| Drawing Number | Title |
| --- | --- |
| B30 | Precast concrete kerb Types A- J |
| B31 B32 E1A F1A F2 F11A | Precast concrete edging Types 1 - 3 Typical plastic trapped gully Type 1/Type 2 marker post Surface water drains, trench and bedding details Filter drains, trench and bedding details Type 7A and B chamber (900 1050 catchpit) |
| F40 F40A | Brick soakaway Precast concrete soakaway |
| F41 | Headwall Type A |
| F42 | Headwall Type B for use with Channel Type B or C |
| F43 | Outlet headwall Type D |
| F44 | Inlet headwall Type E |
| F45 | Headwall Type F |
| F46 F47 | Kerb headwall Typical grillage detail |
| H50 | Step through stile |
| H52 | Stile types 3, 4 and 5 |
| H55 | Galvanised steel kissing gate |
| H56 | Sleeper footbridge |
| H57 | Way marking post |
| H58 | Timber steps |
| H60 H60A S5 W1 W2 W3 W4 W5 W6 W7 8A 8B 8C | Woven Wire Fencing Temporary fencing pig wire fencing Typical partnership information board Uncoursed random rubble wall copings Uncoursed random wall and fences Stonefaced earth bank Retaining wall 1.5m high Retaining wall 1.5m high with BT detail Retaining wall 1.8m high Retaining wall 1.8m high with BT detail Bridge 3, 4 and 5m footbridge Footpath sign |

2(ii)	Inspected by tenderers

There are no Drawings to be made available for inspection by Tenderers or copies to be supplied to the Contractor.

2(iii)	Brought into the Contract by Reference

HCD published by HMSO as Volume 3 of the Manual of Contract Documents for Highway Works contains the following drawings amended brought into the Contract by reference.  Unless otherwise stated below the whole drawing is brought into the Contract.

| Drawing Number | Title | Aspect/Alternative(s) Required if not Whole Drawing |
| --- | --- | --- |
| B1 | Cuttings - combined surface water and groundwater filter drains | - |
| B3 | Cuttings - surface water channel for flexible carriageway | - |
| B4 | Cuttings - drainage channel blocks and drains | - |
| B5 | Central reserve - combined surface water and groundwater filter drains | - |
| B7 | Central reserve - surface water channel for flexible carriageway | - |
| B8 | Central reserve - drainage channel blocks and drains | - |
| B9 | Embankments - channels formed by kerbs | - |
| B10 | Embankments - external kerbs and drainage channel blocks | - |
| B12 | Embankments - surface water channel for flexible carriageway | - |
| B13 | Embankments - verge drainage or verge and carriageway drainage over embankment slope | - |
| B14 | Cross section of concrete surface water channel | - |
| B15 | Cuttings and central reserve - combined surface water and groundwater filter drains | - |
| B16 | Embankments combined drainage and kerb block | - |
| F2 | Filter drains - trench and bedding details | - |
| F3 | Type 1 chamber (brick or in situ concrete manhole) | - |
| F4 | Type 2 chamber (brick or in situ concrete manhole) | - |
| F5 | Type 3 chamber (precast concrete manhole) | - |
| F6 | Type 4 chamber (precast concrete manhole) | - |
| F7 | Type 5 chamber (precast concrete manhole) | - |
| F9 | Type 5 chamber grating details | - |
| F10 | Chamber fittings - ladder, handhold and safety chain | - |
| F11 | Type 7 chamber (1050 catchpit) | - |
| F12 | Type 8 chamber (600 catchpit) | - |
| F13 | Precast and in situ cast gullies | - |
| F14 | Sumpless gully chamber and alternative rising section | - |
| F15 F16 F17 F18 F19 F20 F21 | Drainage channel block types A, B and C Drainage channel block types D, E and F Keyways and keys for manholes and kerb type gully tops Edge of pavement drains - fin drains and narrow filter drains Edge of pavement drains - installation of fin drains Edge of pavement drains- installation of narrow filter drains Edge of pavement drains - under channel drainage layers | - - - - - - - |
| F25 F26 F27 | Type 9 chamber (brick or insitu concrete shallow inspection chamber) Type 10 chamber (brick or insitu concrete shallow inspection chamber) Type 11 chamber precast concrete deep inspection chamber |  |
| H3 H15 H17 H18 H19 H20 H21 H22 H23 H24 | Motorway and accommodation work wooden post and 4 (or 5) rail fences Timber post and 3 rail fences Steel single field gate Steel half mesh single field gate Steel extra wide single field gate Steel double field gate Timber single field gate Timber double field gate Timber wicket gate Type 1 Timber wicket gate Type 2 | - |
| H25 H26 H27 H28 H29 H30 H31 H32 H33 | Timber kissing gate Hinges for steel field gates Latch Type A for steel single field gates Sliding boat latch Type B for steel single field gate Tubular steel latch for steel double field gate Hinges for timber field gate Spring catch for timber single field gate Latch and drop bolt for timber double field gate Standard gate stops | - |
| H34 | Timber stile - Type 1 | - |
| H35 | Timber stile - Type 2 | - |
| H36 | Diagrammatic methods of attaching fencing to structures | - |
| I1 | Longitudinal sections and details of transverse ducts | - |
| I2 | Duct trench cross-sections and details of mandrel | - |
| All Sections | Vehicle Restraint Systems Safety barriers (deformable and rigid) Non-Proprietary safety barrier systems (Revision 1) Part C layout and component detail schedules | - |

APPENDIX 0/5:

SPECIAL ALTERATIONS

The following Additions, Substitutions, Cancellations and minor alterations shall be made: -

| Clause No., Table or Figure No. | Title |
| --- | --- |
| 151A | Dimensional Accuracy Insert the following Clause: 1 Before commencing any part of the Works, the Contractor shall check that all co-ordinates, levels and dimensions shown in the Contract Documents are correct. He shall check and satisfy himself that the various parts fit satisfactorily together and in relation to existing physical features. Checking shall be commenced sufficiently in advance of construction to allow for corrections and approvals. The Contractor shall be fully responsible for correctness, and the Overseeing Organisation will not accept any liability whatsoever for any errors therein. |

1302.4A	The excavation for the column base foundation shall be undertaken by hand unless otherwise agreed by the Overseeing Organisation.  Excavation of existing surfaces shall be carried out in a careful manner to ensure that the reinstatement achieves a similar or better standard of surface finish as that which existed previously.

1305.16A	Where flanged columns are used, then the design should include the use of an additional locknut to secure the column and protection of any exposed thread.

APPENDIX 0/6:

STREET LIGHTING CONTRACT SPECIFIC FORMS TO BE COMPLETED BY THE CONTRACTOR

HL forms detailed in this Appendix to be completed by the Contractor and submitted to the Overseeing Organisation

Form Number			Title

HL3				Emergency Work

5.3 APPENDICIES 1/0

APPENDIX 1/5:

TESTING TO BE CARRIED OUT BY CONTRACTOR

APPENDIX 1/5 (Continued)

Key

†      Indicates a requirement in SHW for the test to be carried out by the Contractor; such tests should therefore be scheduled in Appendix 1/5.

††     indicates a statement in SHW that the test may/will be carried out under the direction of the Overseeing Organisation; samples for such tests should therefore be required in Appendix 1/6.

*       Indicates that the frequency of testing is given for general guidance and is only indicative of the frequency that may be appropriate (i.e. no frequency is given in the SHW or reference documents).  Where materials are known to be marginal or if initial test results show them to be such, the frequency of testing should be increased.  Conversely where material properties are consistently more than specified minimum requirements or well below specified maximum limits, then the frequency of testing should be reduced.

Notes:

(N) indicates that a UKAS accredited laboratory sampling and test report or certificate is required.

APPENDIX 1/7:

WORKING SITE EXTENT AND LIMITATIONS ON USE

1	Working Area

1.1	The working area is that area included within and adjacent to the highway network of the administrative area of the Isle of Anglesey County Council as indicated on Drawing No. S1.

2	Limitations on Use

Appendix 1/23 Substances Hazardous to Health.

APPENDIX 1/9:

CONTROL OF NOISE AND VIBRATION

Noise

1.	The Local Authority has informally agreed that the following measures would be acceptable, and these are given as a guide; however, it is proposed for the Contractor to decide whether to seek the Local Authority's formal consent to his proposed methods of work and to the steps he proposes in order to minimise noise.

2.	The normal working hours except for winter maintenance within the Site shall be Monday to Friday between 0700 and 1900 hours, with no working on weekends or public holidays.  Exceptionally, consent for work outside these hours may be given after any necessary consultation.  28 days’ notice is required from the Contractor when seeking such consent.

3.	The noise levels (see Note (i) below) scheduled below for periods outside normal working hours will only be permitted when consent has been given to exceptional working.

4.	The ambient noise level, Leq (see Note (ii) below) from all sources when measured 1.2-1.5m above the ground 3.5m outside the facade of any occupied building shall either not exceed the appropriate level given in the Schedule or not exceed by more than 3dB(A) the existing ambient noise level, Leq (see Note (iii) below), at the control station measured over the same period, whichever level is the greater.  The maximum sound level at any noise control station shall not exceed the level given in the Schedule.  Exceptionally the Contractor may be given permission to carry out work which exceeds the noise levels in the Schedule, provided that 28 days' notice of the date and timing of these works is given to the Overseeing Organisation and the Contractor demonstrates that he intends to take all reasonable measures to mitigate the noise nuisance.  After consultations with the Local Authority and any other interested bodies a decision will be made within 14 days of receipt of the notice.

| TOTAL NOISE LEVELS AT CONTROL STATIONS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Period | Hours | Ambient Noise Level, Leq measured at facade of occupied building dB(A) | Period of Hours over which Leq is applicable. | Maximum Sound Level LA max (See Note (iv) below) measured at facade of occupied building dB(A) |
| Monday to Saturdays Sundays All unattended plant outside normal working hours | 0700 - 1900 0900 - 1300 2300 - 0700 | 70 65 45 | 12 6 8 | 85 70 60 |

Notes:

(i)	Noise levels relate to free field conditions.  Where noise control stations are located 1m from facades of buildings, the permitted noise levels can be increased by 3dB(A).

(ii)	The ambient noise level, Leq. at a noise control station is the total Leq from all the noise sources in the vicinity over the specified period.

(iii)	The existing ambient noise level, Leq. at a control station is the total Leq. from all the noise sources in the vicinity over the specified period prior to the commencement of the Works.

(iv)	Maximum sound level is the highest value indicated on a sound level meter which meets the requirements of BS EN60651 Type 1 or 2 set to FAST response and frequency weighting A or on an integrating - averaging sound level meter to BS EN61672:2003.  Class 1 or 2 set to fast response and frequency weighting A.

(v)	BS 5228-1:1997.

Vibration

(i)	Locations where vibration limits are to be complied with.

(ii)	Limits of vibrational amplitude and resultant peak particle velocity.

(iii)	Requirements for instrumentation and monitoring.

(iv)	Overseeing Organisation’s arrangements for Contractor to monitor vibration in property off Site.

(v)	The Contractor to provide a full structural survey of properties within 0.5m of the carriageway during any surfacing works.

APPENDIX 1/12:

SETTING OUT AND EXISTING GROUND LEVELS

1.	The Contractor shall, in accordance with information included in the contract, with the approval of the Overseeing Organisation, set out, mark and maintain until they are no longer required, all reference lines, templates, benchmarks and markers, permanent or temporary, necessary for the setting out and checking of the works.  The Contractor shall keep updated schedules and drawings or such information which he shall supply to the Overseeing Organisation as setting out proceeds.

2.	Where setting out markers are likely to be disturbed during the works, the Contractor shall, subject to the prior approval to the Overseeing Organisation, transfer such markers to an adjacent point.

3.	The chainages shown on the layout plans and the 100m chainage positions marked on site are measured as slope distances and are intended as a guide to positions along the site.

4.	Levels of proposed carriageway are either to be set as per existing levels or were indicated as fixed overlay thicknesses relative to the existing carriageway levels.

5.	Where new kerbs are laid the kerb height will be set at 125mm above water line of new carriageway levels.

APPENDIX 1/13:

PROGRAMME OF WORKS

1.	After issue of Works Task for individual schemes the Contractor shall provide the Programme of Works in the form of a bar chart produced because of a "critical path analysis" and must abide by the constraints below.  It shall show the level of detail appropriate to each stage of the Works and all activities and restraints, each of which shall be given a short title.  All events shall be numbered and annotated with earliest and latest event dates.

a)	its title.

b)	its earliest and latest start and finish dates.

c)	its duration.

d)	any relevant "must" start or finish dates.

e)	a complete list of all its immediate predecessor and successor activities together with the nature of the links and any lead or lag delays.

f)	details of its resources.

Any other data, without which a true representation of the Contractor's Programme of Works cannot be created, must also be provided.

2.	Schedule of Constraints

i)	Work to Privately and Publicly owned services and supplies. (Appendix 1/16 Table 1 refers if applicable).

ii)	Traffic Orders (Appendix 1/17 refers).

iii)	Traffic safety and management including notice requirements and restrictions arising from the use of substances hazardous to health.

The level of detail should not be less than the following.

Level 1

To be submitted within 7 days of issue of Works Task.

i)	Roadworks - main route and for each side road, link road and slip road.

a)	Fencing

b)	Site Clearance

c)	Drainage

d)	Sub-base

e)	Base-course

f)	Surface material

g)	Landscaping.

ii)	Major privately and publicly owned services and supplies.

iii)	Traffic management measures including operation of site accesses, plant crossings and temporary diversions.

Level II

Not applicable

Level III

Not applicable

Level IV

Not applicable

3.	One week prior to each monthly progress meeting or at such other times as agreed by the Overseeing Organisation, the Contractor shall supply to the Overseeing Organisation, a report on Progress of the Works in the form of an update of his Programme of Works incorporating Works Task for the monthly section of Works.

4.	The report which shall be presented as a table of data, shall indicate which activities have been started and/or finished since the previous progress report and must include the relevant actual start and finish dates.  For those activities which are in progress, the report shall state the estimated time remaining to completion of these activities.

5.	This report shall be used as a basis for progress discussions at the progress meeting and, if any discrepancy occurs between the Contractor's report and the Overseeing Organisation’s observations, or if an analysis of the updated Programme of Works reveals a change in completion dates for any related activities, the Contractor shall, within 7 days, supply the Overseeing Organisation an explanation of the discrepancy/changes and an assessment of their likely effects.  All such explanations and assessments shall be recorded in the minutes of the following progress meeting or other meeting agreed with the Overseeing Organisation. Compliance to the programme of works will form a Key Performance Indicator

6.	Monitoring Contractual Compliance

One week prior to each monthly meeting arranged to monitor contractual compliance by the Contractor, or at such other times as agreed by the Overseeing Organisation, the Contractor shall supply the Overseeing Organisation with details of any notices of non-compliance forming part of the Certification for payment system.

This report shall be presented as a table of data indicating the specific areas of non-compliance identified.  The Contractor shall indicate his proposals to remedy the areas of non-compliance actions taken, or final assessments.

All such identified non-compliance reports shall be recorded in the minutes of the "Monitoring Contractual Compliance" meeting or other such meeting agreed with the Overseeing Organisation.

Outstanding non-compliance matters shall be included in detail on any subsequent report, together with a schedule of "signed off" non-conformities resulting in Certification of Payment.

The report shall be used generally as an indicator of the Contractors overall Contractual Compliance and performance.

APPENDIX 1/15:

ACCOMMODATION WORKS

1.	Copies of land reference plans or schedules will be available for inspection after issue of Works Task for individual schemes at: -

Overseeing Organisation’s Office - Head of Service (Highways, Waste and Property),

Council Offices,

LLANGEFNI

LL77 7TW

(01248) 750057

and will be supplied to the main Contractor before the commencement of work.  Further information will be provided in respect of accommodation works when this has been agreed.

2.	Details of accommodation works already determined are as follows: - none

3.	Details of accommodation works will be issued with Works Tasks for individual schemes.

APPENDIX 1/16:

PRIVATELY AND PUBLICLY OWNED SERVICES AND SUPPLIES

1.	This Appendix contains details of services and supplies affected by the Works, details of preliminary arrangements that have been made with Statutory Undertakers and others for the alteration of services and supplies affected by the Works and details of any orders placed.

2.	The Contractor shall plan with the Statutory Undertakers and others concerned for the co-ordination of his work with all work which needs to be done by them or their Contractors concurrently with the Works.  Compliance with the periods of notice given in this Appendix does not relieve the Contractor of his obligations.

3.	Private Services to individual properties will not generally be listed or shown on the Drawings.  The Contractor shall plan with the Statutory Undertakers and others concerned for the phasing of all necessary disconnections and diversions of private services affected by the Works.

4.	Disconnected apparatus shall be removed by the Contractor only with the prior consent of the Authority concerned.

5.	The names, addresses and telephone numbers of the authorities serving in the locality are listed below (may change without notice)

AUTHORITIES AND STATUTORY UNDERTAKERS

SERVING IN THE LOCALITY

| NAME | CONTACT ADDRESS AND TELEPHONE NUMBER |
| --- | --- |
| Overseeing Organisation Isle of Anglesey County Council | Head of Service (Highways, Waste and Property) Cyngor Sir Ynys Môn Council Offices LLANGEFNI Ynys Môn LL77 7TW (01248) 750057 |
| British Telecommunications PLC. | Openreach BT Post Point A Huddersfield W Yorks HD1 1TA 01484513793 |
| SP Energy Network | Avondale House Bellshill Glasgow ML43NJ |
| National Grid | National Grid Transco Wayleave and Easements Enquiry Centre PO Box 3484 Warwick CV34 6TG (01926) 655920 |
| Dwr Cymru | Dwr Cymru/Welsh Water Pentwyn Road Nelson Treharris CF46 6LY 08000520130 Clean 08000853968 Foul |
| National Resource Wales | Llwyn Brain Ffordd Penlan Parc Menai Bangor Gwynedd LL57 4DE (01248) 670770 |
| Natural Resources Wales | Natural Resources Wales Llwyn Brain Ffordd Penlan Parc Menai Bangor Gwynedd LL57 4BP (01248) 670770 |
| National Assembly for Wales Agriculture Department | Cathays Park Cardiff CF10 3NQ 0300 200 6565 |
| UK Highways A55 Limited | Plot No. 7 Mona Industrial Park Mona Anglesey LL65 4RJ (01407) 721296 |
| Network Rail | Room 103 Rail House Store Street Manchester M60 7RT (01612) 284046 |

SERVICES AND SUPPLIES AFFECTED BY THE WORKS

The Contractor shall obtain details of services and supplies affected by the works from the statutory undertakers.  If the Employer has obtained the information for design purposes the information will be issued with the Works Instruction.

(The Contractor is to always comply with Clause 116 of the Specification.)

Contractor will be notified of the Work priority as follows:

A	Work expected to be completed before the commencement of the Works.

B	Work required after commencement of the Works which does not require prior work by the Contractor.

C	Work required after commencement of the Works which does require prior work by the Contractor.

D	Work expected to be in progress at the commencement of the Works.

E	Work to be wholly undertaken by the Contractor

APPENDIX 1/17:

TRAFFIC SAFETY AND MANAGEMENT

1.	Traffic Safety and Management Requirements (Individual Schemes)

i)	Phasing of Works

a)	The maximum length of single way running 300m long.  Refer to paragraphs D5.4 to D5.4.5 for restrictions on traffic signals and one-way working.  One set of traffic management allowed within the length of the individual works.  The only exception to this is to allow temporary traffic management as part of the safe maintenance of the works.

b)	Full details of the phasing of the works as it affects all public and private roads, bridleways, public footpaths and public and private accesses shall be provided by the Contractor before any meeting held between the Contractor and the Overseeing Organisation’s representative and any other interested parties to discuss Traffic Safety and Management.

c)	In the event of a mandatory speed limit of 30mph applying to the existing road for the whole length of the Works, Advisory speed limits of 10mph alongside working areas will be permitted, under the direction of the Traffic Signs Regulations and General Direction 2006 (SI No. 1519), the Traffic Signs (Welsh and English Language Provisions) Regulations and General Directions 1985 (SI No. 713), the Road Traffic Regulation Act 1984 and the Highways Act 1980 together with any amendments thereto.  Refer to Appendix 1/17 Table 3.

ii)	Traffic Management Layouts

The layout No.’s referred to in Section 100 Traffic Safety and Management are those contained within Chapter 8 Traffic Signs Manual 2006.  Traffic Safety Measures and Signs for Road Works and Temporary Situations.

Drawings showing traffic management layouts as they affect public and private roads, bridleways, public footpaths and public and private accesses shall include the following details as necessary.

These drawings, together with any other relevant information, shall be made available at the same time as the submission of details required under paragraph 1(i) above.

a)	Position of traffic signals.

b)	Widths of lanes.

c)	Working area.

d)	Safety zone.

e) 	Highway.

f)	Pedestrian facilities including school crossings.

g)	Provision for servicing properties.

h)	Bus stops/vehicular access.

i)	Access arrangements to all identified accesses affected by the Works.

iii)	Timing of the Operation

Full details of the proposed timings of traffic safety and management operations shall be provided by the Contractor at the same time as the submission of the details required under paragraph 1 (i) above.

iv)	Road lighting requirements

Maintain existing street lighting operation throughout the Works period, except for changeover to new system.  Temporary lighting required when directed by the Overseeing Organisation. (refer to 117.6 and 117.15).

v)	Hazard warning lights are not an acceptable alternative to a roof mounted amber flashing light and shall conform with the Road Vehicle Lighting Regulations.

vi)	Traffic Safety Control Officer (117.19)

The Contractor shall appoint a Traffic Safety and Control Officer who shall make all arrangements necessary for traffic safety and control for all individual schemes.

A mobile phone shall be provided for use by the traffic signal operative in emergency situations, either to alert the emergency services of an incident or to enable the operative to be contacted by the emergency services if necessary.  The telephone shall be used to contact the Contractors site offices.  The mobile phone shall be capable of blocking outgoing calls except to authorised numbers as identified above.

vii) Restrictions arising out of the use of substances hazardous to Health.

The Contractor shall notify the Overseeing Organisation of any proposals to the use of substances hazardous to health and shall adhere strictly to the manufacturer’s instructions and recommendations.

2.	Maintenance Requirements

a)	Cross overs	-	not applicable.

b)	Highways	-	as specified within Limits of Works.

c)	Timescale	-	not applicable.

3.	Notice Requirements

The notice required by the Overseeing Organisation for him to arrange for: -

i)	amending or making traffic orders - 8 weeks

ii)	authorising of non-prescribed signs - not applicable to this Contract.

iii)	authorising temporary traffic signals 	- 2 days for 2-way lights.

- 7 days for 3-way lights

iv)	moving signs to be compatible with the state of the works as described in sub-clause 117.11 - not applicable to this contract.

4.	Details of Events that could have a Bearing on the Works

a)	The Contractor will be advised by the Overseeing Organisation as part of Works Task when relevant.

b)	A55 Trunk Road and County Road approaches are affected over 24-hour periods daily throughout the year by tidal ferry traffic flows generated by the Port of Holyhead.  Traffic characteristics include substantial levels of heavy goods vehicles and tourist traffic.

c)	Generally, throughout the network tourist traffic flow levels are to be anticipated during April - October period more than indigenous traffic flows.

d)	Roads are utilised by heavy goods vehicles and agricultural traffic gaining access to remote centres, particularly chicken processing plants and Quarries.

e)	Anglesey Show at Mona mid-August annually.

f)	Valley Air Show biannually late August.

5.	Highways, Private Roads and Other Ways Affected by the Works

5.1	i)	Details of Usage, Speeds, Traffic Controls are given in Appendix 1/17 Table 1 and paragraphs 5.2 and 5.3 below.

ii)	County Roads, footpaths and bridleways described above or listed in Appendix 1/19 are under the responsibility of:

Overseeing Organisation:	Head of Service (Highways, Waste and Property)

Cyngor Sir Ynys Môn

County Offices

LLANGEFNI

Ynys Môn

LL77 7TW

5.2	Types of Traffic Control - Traffic Signals

i)	The use of traffic signals shall be restricted to the minimum necessary to carry out the works.  In no cases shall the distance between signal heads exceed 200m.  Three-way traffic signals will be permitted.

ii)	During peak periods of traffic flow, it will be necessary for any traffic signal installation to be manually controlled and monitored.

5.3	Single way Working

i)	Single-way working will only be permitted for the purpose of executing the following works:

a)	Roadworks

b)	Drainage or service ducts

c)	Construction of the tie-ins

d)	General Engineering Works

Table 1 - Public and private road, and other ways affected by the Works

| Description | Predicted 24 hours AADT or usage | 85% percentile speed of cars (mph) | speed limit (mph) mandatory | type of traffic control | special facilities | to be kept open or closed |
| --- | --- | --- | --- | --- | --- | --- |
| Details will be provided for individual schemes as part of Works Task or as identified on scheme drawings. All public or private roads and other vehicular or pedestrian ways to be kept open unless otherwise instructed by the Overseeing Organisation. |  |  |  |  |  |  |

Table 2 Private Access/Adjacent Properties

Details of types and owners

| LOCATIONS TYPE OF ENTRANCE OWNER/TENANT All accesses for individual schemes to be constructed and maintained throughout the works unless otherwise instructed. |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 3 Temporary Traffic Restrictions

| The Road Traffic Regulations Act 1984 Section 14(1) as amended by the Road Traffic (Temporary Restrictions) Act 1991 |  |
| --- | --- |
| 1 | Details will be provided for individual schemes as part of Works Task or as identified on scheme drawings. |
| 2 | Existing restrictions to be suspended - Details will be provided for individual schemes as part of Works Task or as identified on scheme drawings. |
| 3 | Description of Alternative routes - Not anticipated but details will be provided for individual schemes as part of Works Task or as identified on scheme drawings. |

ADDITION TO APPENDIX 1/17:

This document to be adopted for Traffic Management set ups and amendments SVW1A or SVW2A (Appendix B) to the Chapter 8 2007 Editions.

Single Vehicle Maintenance Operation on 2-Lane Single Carriageways

Chapter 8 Implementation Document

Contents

1.	Introduction

2.	Background

3.	Operational Issues

4.	Carriageway Category and Reference Plans

i)	General sites

ii)	Moderate sites

iii)	Difficult Sites

5.	Escort Vehicle Risk Assessment

6.	Proposed Traffic Management Plans

i)	Basic Traffic Management plans

ii)	Site length requirements

7.	General Operational Requirements

i)	Operative Competency

ii)	Vehicle Specification

8.	Partner Authority Operational and Site-Specific Risk Assessments

9.	Conclusions

10.	Recommendations

1.	Introduction

1.1	Many routine and cyclic maintenance operations currently undertaken on single carriageways by the North and Mid Wales Trunk Road Agency (NMWTRA) Partner Authorities (PA’s) on behalf of Welsh Government (WG) utilise slow moving single vehicles including tractors and gully emptiers.

1.2	Historically these operations have been undertaken utilising advance warning signs on the approaches to working areas in accordance with Chapter 8 of the Traffic Signs Manual 1991. Specifically identified difficult sites have generally utilised site specific traffic management systems.

1.3	The principal maintenance operations undertaken using the above methods include: grass cutting, gully emptying and weed control.

1.4	Grass cutting operations are generally undertaken during May to mid-July prior to the Trunk Road summer embargo period.  This operation involves multiple sites with more than 40 tractor units in operation on Trunk and County Roads during the above programme.

1.5 	The majority of sites are on unrestricted sections of single carriageway.

1.6	In general, this document only provides guidance for 2-lane single carriageways. Three lane sections will have to be dealt with on a site-specific basis.

2.	Background

2.1	The implementation in January 2007 of the new Traffic Signs Manual, Chapter 8 – Traffic Safety Measures and Signs for Road Works and Temporary Situations 2006, introduces changes to traffic management requirements for the above operations. Section O8 makes specific provision for single vehicle working with requirements identified for works in areas with traffic speeds up to 30mph, 40mph and unrestricted sections of highway. (This document should be read in conjunction with the above code of practice)

2.2	The principal clauses determining traffic management provision are Cl O8.1.5, Cl O8.1.6 and Cl O8.1.9.

2.3	For speed limits of 40mph and above Cl O8.1.5 recommends, subject to risk assessment that a lorry mounted crash cushion be fitted to an escort vehicle located behind the working vehicle or alternatively fitted to the working vehicle.

2.4	For speed limits of 40mph and above, Cl O8.1.6 requires that the working vehicle shall carry a sign to diagram 7403 or, alternatively, carry a sign to diagram 610 provided an escort vehicle is used displaying a sign to diagram 7403.

2.5	Cl O8.1.9 identifies the need for a risk assessment to determine whether traffic control by means of stop-go boards is required for TM within a 30mph speed restriction.

3.	Operational Issues

3.1	The implications of providing the above signing and escort vehicle/crash cushion arrangement, were discussed at length during a Chapter 8 Workshop on 14th May 2007. The Workshop was attended by WAG, all North Wales PA’s and the NWTRA Management Unit.

3.2	The principal concerns raised at the workshop were:

i)		It was not practical to fit a sign 7403 to the rear of the working vehicles used for grass cutting or gully emptying.  This would restrict visibility and cause significant operational difficulties.

ii)	The introduction of an escort vehicle or crash cushion would significantly increase the length of obstruction that vehicles would need to pass.  Typically, an overall length of approximately 40m compared with approximately 6m of the working vehicle.  This would significantly increase the exposure of passing vehicles to oncoming traffic.

iii)	The introduction of an escort vehicle or crash cushion would create a possible sandwich effect whereby vehicles attempting to pass both escort vehicle and works vehicle and misjudging oncoming traffic movements would take evasive action by entering the work zone between escort and works vehicles.

iv)	Provision of crash cushions or escort vehicles for 40+ No. works sites for Trunk and County roads was impractical from a resource point of view.

vi)	The above provision would significantly extend the overall duration of these works.

vii)	The above provision would add significant costs to the maintenance operations.

3.3	The above views have been confirmed by Atkins following trials on behalf of the Highways Agency using escort vehicles undertaken in the Area 6 MAC.  The trial had to be abandoned after 15 minutes due to the number of near misses.  It is also understood that a further live trial in Area 11 resulted in an actual rear end collision with a gully emptier.

4.	Carriageway Category and Reference Plans

4.1	The workshop identified the need to categorise sites in respect of carriageway width, verge width, visibility sight lines and traffic flows, together with work activities.  Three categories were agreed.

i)	General sites:  Good visibility available for speed category of highway, 7.0m carriageway or more and verge widths greater than 1.0m or adequate alternative locations to place signage.

ii)	Moderate sites:  Fair visibility with no severe blind bends, carriageway widths typically between 6.0m and 7.0m, verge widths of 1.0m or more to at least one lane or adequate alternative locations to place signage

iii)	Difficult Sites:  Poor visibility, tortuous alignment with blind bends, carriageway widths less than 6.0m, verges of less than 1.0m.

The above categories are given for general guidance and are subject to the use of engineering judgement and risk assessment dependent on circumstances.

4.2	The workshop agreed that only Categories (i) and (ii) would be subject to general TM plans and traffic control with Stop / Go boards supplemented with radio communication between operatives. Difficult sites would require site specific TM control systems extending to lane closures with traffic signal control. (Note: These sites are not covered by this implementation document).

4.3	Partner Authorities will be responsible for assessing sites and work activity type and providing separate reference plans for each work activity type at an appropriate scale identifying Category (ii) and (iii) sites. Sections of highway other than Category (ii) and (iii) shall be deemed to be Category (i).

5.	Escort Vehicle Risk Assessment

5.1	In order to address the operational issues identified in Section 3 above, a formal risk assessment for provision of an escort vehicle or crash cushion was undertaken.  The Risk Assessment can be found at Appendix A.

5.2	The outcome of the risk assessment identified several significant risks to both the travelling public and site operatives resulting from the introduction of the escort vehicle with initial Risk Priority Numbers (RPN) at the intolerable level.

5.3	Identified mitigation measures included the deletion of the escort vehicle and fundamentally, introducing a temporary 30mph speed restriction to the work site in conjunction with standard Chapter 8 TM plans SVW1 or SVW2.

5.4	The introduction of a temporary 30mph speed restriction is compatible with Section D3.7 of Chapter 8 Part 1 Design - Speed Control / Temporary Speed Limits.

5.5	Section D3.7 recommends mandatory speed limits.  However, given the current timescale for the 2007 cyclic maintenance programme it is unlikely that the necessary traffic orders can be processed in time.  Clause D3.7.4 advises that if it is impractical to obtain a mandatory speed limit, then advisory speed limits should be adopted.  This document proposes that advisory temporary 30mph speed limits are adopted in accordance with Clause D3.7.4 for the current programme of works but strongly recommends that WAG process blanket orders to facilitate mandatory temporary speed limits of 30mph for subsequent years.

5.6	The mitigation measures included in the Risk Assessment have been incorporated into the following proposed traffic management plans to be utilised subject to PA operational and site-specific risk assessments for Category (i) and (ii) sites:

- modified TM plan SVW1A is envisaged to be suitable for most Category (i) sites.

- modified TM plan SVW2A is envisaged to be appropriate for most Category (ii) sites.

(Modified plans incorporating advisory 30mph speed restrictions are included in Appendix B).

5.7	The proposed plans are compliant with the requirements of the 2006 version of Chapter 8 and should be read in conjunction with this code of practice.

6.	Proposed Traffic Management Plans

6.1 	The basic TM plans to be adopted shall be those detailed in Appendix B, which are modified versions of TM plans SVW1 and SVW2 of Traffic Signs Manual Chapter 8, Part 2, 2006.  The modified TM plans SVW1A and SVW2A are regarded as minimum requirements.  However, if additional mitigation methods are identified by appropriate risk assessments, these may be implemented in conjunction with the modified TM plans.

6.2 	For modified TM plan SVW1A, 3 advance signs are required:

- First sign to Diag 7001 (“road works”) with supplementary plate to Diag 7001.1 (wording – “Mobile Road Works”) and positioned as shown in plan SVW1A.

- Second sign to Diag 7294 (“advisory speed limit”) to be positioned as shown in plan SVW1A.

- Third sign to Diag 517 (“road narrows”) with supplementary plate to Diag 518 (“single file traffic”) to be positioned as shown in plan SVW1A.

6.3 	For SVW2A, an additional sign to plan SVW1A to Diag 7010.1 (“Traffic Control Ahead”) is also required, together with Stop / Go board provision as shown on plan SVW2A.

6.4 	Imposition of temporary / advisory maximum speed limits can only be implemented on works exceeding 400m in length measured between the initial ‘speed limit’ sign (road works sign) and 45 metres beyond the end of the works.

6.5 	If mandatory speed limits are used, and where the length of work is more than 800m, repeater signs at intervals, shown in Table 3.4 and note 1, shall be provided.  No work length shall exceed 1600m between final Diag 5.17 for each direction.  For temporary advisory speed limits, repeater signs are not required.

6.6	If carriageway, verges or footway widths preclude use of repeaters, working length of less than 800m shall be utilised thus precluding the need for repeaters.

6.7	Use of these plans shall only be acceptable during periods of low risk, and at all times conditions imposed via O8.1.1, O8.1.2, O8.1.4, O8.1.7, O8.1.8, O8.1.10, O8.1.11, O8.1.12, O8.1.13, O8.1.14, O8.1.15, O8.1.16, O8.1.17, O8.1.18 shall apply as appropriate.

7.	General Operational Requirements

7.1	Operative Competency

Workforce issues relating to works carried out within the constraints of Traffic Signs Manual Ch 8 2006, are dealt with in detail in document ‘Guidance for Safer Temporary Traffic Management’ (CSS/HA/HSE/2002) where reference to other documents giving further detailed guidance can be found.

7.2 	Vehicle Specification

The PA shall provide vehicles suitable for purposes, i.e. type, engine size and gearbox should reflect the manoeuvrability and operational needs of the exercise it is to perform.

In terms of conspicuity, the vehicle should be a conspicuous colour, e.g. yellow or white.  Where feasible a non-reflective yellow colour, No. 355 (lemon) to Table 1 of BS3816:1996 is recommended.

All vehicles stopping on the highway shall be equipped with high visibility rear markings as per O5.2.3 of Traffic Signs Manual, Part 2, 2006.

Works vehicles shall be kept clean.

Vehicles exceeding 7.5 tonnes and trailers with maximum gross weight exceeding 3.5 tonnes must be fitted with rear markings in accordance with Lighting Regulations.

Vehicles shall be identified by display of signs to Diagram 7404.  Any vehicle stopping on the highway shall be equipped with beacons defined in O5.31., O5.3.2, O5.3.3, O5.34 and O5.3.5.

Vehicles used for installation, maintenance and removal of static traffic management on trunk roads shall comply with the specification in paragraphs O5.5.2 to O5.5.4 of Traffic Signs Manual, Chapter 8, 2006.

8.	Partner Authority Operational and Site-Specific Risk Assessments

8.1	In relation to the Construction Design and Management Regulations the NMWTRA Service Provider Agreements (SPA) for works on the Trunk Road Network will require that the Partner Authorities undertake the role of Principal Contractor.  This places responsibility for management of the health and safety aspects of all Trunk Road works procured through the PA including the single vehicle works covered by this document.

8.2	Taken cumulatively the overall operation exposes a significant level of risk to the workforce. Consequently, all operations shall be subject to risk assessment and development of appropriate method statements.  It is proposed that the format adopted for risk assessments is the same as included in Appendix A of this document.  This will enable comparison and consistency of approach to be maintained across all PAs.

8.3	It is envisaged that a generic risk assessment will be appropriate for Category (i) sites and site-specific risk assessments will be required for Category (ii) and (iii) sites.

8.4	The approach to risk assessments was discussed by all PAs at a follow-up workshop on 8th June 2007 to ensure consistency of interpretation and approach.

9.	Conclusions

9.1	Following extensive consultation and discussion with Works Unit Managers, DSO Managers and Client Maintenance Engineers from all North Wales Local Authorities, and WG Route Managers, it is considered that, subject to risk assessment, the use of escort vehicles or crash cushions are inappropriate for general use on unrestricted rural 2-lane single carriageways for single vehicle maintenance operations.  The collective view was that the introduction of escort vehicles resulted in an additional obstruction of excessive length and that the associated risks were unacceptable.

9.2	It must be noted that the accident statistics for single vehicle working in North Wales under the superseded version of Chapter 8 were negligible with the only reported incident associated with a difficult site in the Wrexham area.  Working methods were adjusted to reflect this and it is noted that this document does not address difficult sites.

9.3	It was accepted that single vehicle operations on the network do present a risk to both the travelling public and vehicle operatives but more appropriate mitigation measures than the use of either escort vehicles or crash cushions were to introduce a temporary 30mph speed restriction and adopt modified Chapter 8 TM plans SVW1A and SVW2A (see Appendix B).

10.	Recommendations

10.1	It is recommended that the proposals in this document are adopted across the North Wales region for both Trunk and County single vehicle operations on 2-lane single carriageways and that for consistency consideration be given to adopting its proposals across all Wales.

10.2	WG should investigate the practicalities of facilitating a mandatory temporary 30mph blanket speed restriction for these operations prior to the 2008 cyclic maintenance programme.

Appendix 0/4 – Modified Traffic Management Plans SVW1A and SVW2A

Traffic Management Act 2004 New Roads and Street Works Act 1991 Registration and Notification of Works for Road Purposes Major Highway Works Road Maintenance Street Lighting Special Events

TRAFFIC MANAGEMENT ACT 2004

Introduction

1.0 	The Traffic Management Act 2004 (TMA) places a Network Management Duty on all local traffic authorities.  This means they have a statutory duty to ensure road networks are managed effectively to minimise congestion and disruption to both vehicles and pedestrians.  It requires both street works and works for road purposes to be planned and co-ordinated to consider the impact on road users and on neighbouring traffic authorities.

1.1 	It is important to recognise that the TMA aims to create a level playing field for both local traffic (highway) authorities’ works and works by statutory undertakers.  From 1st April 2008 onwards, the Act requires both statutory undertakers and local authority works to adopt new notice procedures.

1.2 	Under the TMA, the Welsh Assembly Government has powers to intervene if they consider that a local traffic authority is failing to perform any of its network management duties using an ‘intervention Notice’ or ‘Intervention Order’.  A guidance document, The Traffic Management Guidance on Intervention Criteria (Wales) Order 2007 has been published by WG on the 27th of November 2007.  It sets out what the Assembly Government would consider in determining whether a local traffic authority is performing its network management duties.

1.3 	The general rules for compliance with an authority’s statutory duty is to manage the network are as follows: -

(i) 		Co-ordination and planning works and known events.

(ii)		Ensuring parity with others, by applying the same standards and approaches to an authority’s own work as to those of statutory undertakers and other works promoters.

(iii) 	Any other additional duties, for instance, part III of the Disability Discrimination Act 1995 (DDA), amended by the DDA 2005, gives disabled people a right of access to goods, facilities, services and premises.  By providing integrated and accessible transport and barrier-free pedestrian environment, local traffic and highway authorities will help deliver this right and fulfil their obligations.

1.4 	The highway network is Anglesey’s most asset, and the Authority’s approach in protecting its integrity is recognised as a high priority.

1.5 	The policy document is intended to provide guidance and advice to officers involved in the planning, implementation and execution of works on Anglesey’s highway network.  It also applies to special events.

NEW ROADS AND STREET WORKS ACT 1991

2.0 	The New Roads and Street Works Act 1991, together with the attendant SI Regulations and Codes of Practice, provides a legislative framework for street works by undertakers; and works for road purposes.

2.1 	Key amendments to the 1991 Act, contained within ‘The Code of Practice for Co-ordination of Street Works and Works for Road Purposes and Related Matters’ (March 2008), reflects various changes made to the provisions of NRSWA under TMA 2004.  The TMA tightens the regulatory framework within, which undertakes dig up roads, giving authorities more powers to co-ordinate, control and direct works effectively, with the aim of minimising disruption and damage to the highway assets.

2.2 	The Code incorporates the requirements set out in the following regulations

- The Street Works (Register, Notices, Directions and Designations) (Wales) (Nº 2) Regulations 2008, SI 2008 Nº 540(W.52)

- The Street Works (Fixed Penalty) (Wales) Regulations 2008, SI 2008, SI 2008 Nº 102 (W.15)

NOTIFICATION AND REGISTRATION OF WORKS FOR ROAD PURPOSES

3.0 	The street authority’s network management duties require it to hold information on the “works and activities” carried out on its highway network.  Registering works for road purposes is the first and most important step to achieving that.

3.1 	Section 53 of NRSWA requires an authority to place information about its own works on the street works register and this should be done in similar timescales to undertakers.  The Regulations specify that this applies to works that involve: -

- Breaking up any part of the highway

- Opening the carriageway of traffic sensitive streets at traffic sensitive times or reducing the width of the carriageway by one or more lanes, or by more than one third.

Therefore, works equivalent to those carried out by undertakers will be registered, as set out in paragraph 3.4.  Registration identifies responsibility for the works and underlines the importance of advance notification.  This is essential for planning and co-ordinating works so that they limit both disturbance of the road structure and disruption to users.

3.2 	In effect, highway, transport and bridge authorities are giving notice of their proposals to all other parties.  They should therefore use the protocols in the Technical Specification for E to N for notices for this purpose.  In general, these works will use similar classification criteria as those for street works.

3.3 	Registration is not required for road markings and similar works that do not involve breaking up or reducing the width of the carriageway.

3.4 	An Authority undertaking works on its highway network must register, or notice, all works that;

(a) 	Involve the breaking up or resurfacing any street.

(b) 	Involve opening the carriageway or cycleway of traffic sensitive streets at traffic-sensitive times.

(c) 	Require any form of temporary traffic control as defined in the Code of Practice for Safety at Street works and Road Works.

(d) 	Reduce the lanes available on a carriageway of three or more lanes.

(e) 	Require a temporary traffic regulation order or notice, or the suspension of street at a traffic-sensitive time.

(f) 	Require a reduction in the width of the existing carriageway of a traffic-sensitive street at a traffic-sensitive time.

Highway Authorities do not need to register;

- Traffic census survey – because disclosure prior to a census can encourage a change in formal traffic flows.

- Pole testing involving excavation – does not require registration, unless one or more of rules (b) -(f) above apply.  However, the work must be registered within 10 days of completion.

- Road markings – that are not part of a larger set of works and do not reduce the width of the carriageway, as they do not involve breaking up of the highway.

3.5 	Street Lighting

The definition of works for road purposes may include some works carried out by undertakers, such as street lighting.  It is for works promoters to ensure such works are notified or registered, as appropriate.  Responsibility for new supply connection works to the highway authority asset rests with the highway authority itself

Isle of Anglesey County Council has a statutory duty to comply with its Network Management Duties. All works undertaken by, on behalf of, or licenced by the Council will be in strict compliance with TMA and NRSWA, regarding Notification Procedures. The Authority currently operates the Mayrise StreetWorks package to Eton 6 and the Contractor is to purchase (or similar) a package to allow for the electronic transfer of notices on behalf of the Authority at their cost. Compliance to the noticing arrangements will be monitored by a Key Performance Indicator and will operate in addition to compliance to the NRSWA noticing fixed penalty notice. KEY PERFORMANCE INDICATOR KPI 2– NOTIFICA TION OF WORKS TO THE STREET WORKS SECTION

WORKS CATEGORIES

4.0 	The Street Works (Register, Notices, Directions and Designations) (Wales) Nº 2 Regulations 2008 provides a statutory interpretation on the various works that must be notified and registered by street authorities.  Works are classified by the Regulations as follows: -

4.1	 Major Works

Major works undertaken by authorities are described as follows: -

(a) 	Works which have been identified in the annual operating programme of an authority or which, though not specifically identified in such programme, would normally be planned or known about at least six months in advance of the date proposed for the works;

(b) 	any works which would require an order under Section 14 of the 1984 Act (temporary prohibition or restriction on roads) is required; or

(c) 	any works, other than immediate works, the planned duration of which exceeds ten (10) days.

4.2 	Standard works

Standard works are defined as works, other than immediate works or major works, with a planned duration of between four and ten days.

4.3 	Minor works

Minor works are works other than immediate or major works, with a planned duration of three days or less.

4.4 	Immediate works

Immediate works are either emergency or urgent works required to end, or prevent circumstances, either existing or imminent, which might cause damage to people or property.  The term includes works whose execution is required to prevent, or put an end to, an unplanned interruption to any supply or service provided by the authority; for example, drainage repairs to prevent flooding to property and renewal of a street lighting cable.

Work Categories – Works for Road Purposes

|  | Category | Duration (days) |  |
| --- | --- | --- | --- |
|  | Major | > 10 |  |
|  | Standard | 4 - 10 |  |
|  | Minor | < 4 |  |
|  | Immediate | See 4.4 above |  |

WORKS FOR ROAD PURPOSES

NOTICE REQUIREMENTS AND VALIDITY

5.0	This section provides guidance on the statutory requirement to register and notify local authority works in accordance with the new noticing regime resulting from TMA revised regulations and the Code of Practice.  It is intended to provide guidance to ensure correct and consistent compliance with the new noticing regime as follows.

Policy 2N Advance Notification for Major Highway Works and Works for Road Purposes shall be provided in accordance with Section 54, 55 and 57 of NRSWA. Responsibility for providing Notices rests with the WORKS PROMOTER – transferred to the Contractor.

5.1 	Advance Notice for Major Highways Works (Section 54)

Major works requires a minimum notice of 3 months prior to commencement date to comply with Section 54 of NRSWA as amended by Section 49 of the TMA.  A Notice of starting date must also be provided 10 days prior to commencement date.  To promote co-ordination, the notice must include all available information about the works, including details of proposed traffic management and the estimated end date.

5.2 	Each notice shall contain information about works in only one street; therefore, schemes which affect several streets will require a separate notification process for each street.

5.3 	Advance Notice for Standard Works

For standard works a notice must be provided at least 10 days prior to commencement date in accordance with Section 55 of NRSWA.

5.4 	Minor Works

For minor works, a notice must be provided at least 3 days prior to commencement date (Section 55 of NRSWA).

5.5 	Immediate Works

An immediate works notice is either an emergency works notice required under Section 57 of NRSWA or, in the case of urgent works, a notice indicating a start of works under Section 55.  Immediate works notice is to be provided as soon as reasonably practicable and, in any event, within 2 hours of the works starting.

5.6 	For ‘out of hours’ and ‘weekend works’ notices must be provided by 10:00am on the first available working day (Monday – Friday) following commencement of the works.

Works which have not been correctly notified in accordance with the provisions of NRSWA will not be allowed to proceed unless by prior agreement with the Street Authority.

5.7 	Notice Validity – Major Works (Section 54)

An advance notice provided under Section 54 must specify a provisional start date which is

- No earlier than 3 months.

- Works must commence within 15 days beginning with the starting date stated on the notice, although a street authority may allow a further period of validity.

5.8 	Standard/Minor Works (Section 55)

For notices provided for Standard and Minor Highway Works, the works may not start earlier than expiry of the notice period, except with the prior agreement of the street authority.  However, the notice remains valid for the following time periods and therefore the works may start

- For 10-day notice – no later than five days from and including the provisional start date.

- For 3-day notice – within 2 days of the start date specified on the notice.

5.9 	Matrix of Notice Periods and Start Dates

|  | Minimum Notice Period |  |  | Validity Period |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S 54 | S 55 | S 57 | S 54 | S 55 | S 57 |
| Major | 3 months | 10 days |  | 15 days | 5 days |  |
| Standard |  | 10 days |  |  | 5 days |  |
| Minor |  | 3 days |  |  | 2 days |  |
| Immediate |  |  | 2 hours after |  |  |  |
| Works discovered on site, and which have not been correctly notified may, at the discretion of the Street Authority, be subject to a stop notice. A directive issued by the Street Authority will detail action and corrective measures to be taken by the works promoter and may include immediate closure of the works and removal of any traffic management measures. |  |  |  |  |  |  |

6.0	Actual Start Date Notice (Section 74)

Because the validity period, covered in 5.8 means that works do not have to start on the date proposed on the Section 55 notice, for highway works, a notice of ‘actual start date’ must be provided to comply with Section 73 of NRSWA.

A notice of ‘Actual Start Date’ must be provided within 2 days of commencing the works.  For Immediate works, the date specified on the notice is deemed to be the actual start date.

6.1 	Works Closed Notice

A Works Closed Notice is used following permanent reinstatement and completion of the works.  The Notice of Works Closed must be given by the end of the working day following the one which the works were closed.

6.2 	Notice of Completion of Reinstatement

A notice of ‘Completion of Reinstatement’ must be provided within 10 days of completing a reinstatement, carriageway resurfacing including any reconstruction and surface dressing to the street authority.  The notice must state whether the reinstatement is interim or permanent.  It must also contain the following: -

(i) 		NGR, either

- One in the centre of a small reinstatement

- One at each end of highway surfacing works and trenches

(ii) 	The dimensions and description of every reinstatement.

(iii) 	The date the site was reinstated.

(iv) 	Construction method.

BLANK PAGE

BLANK PAGE

NOTICE CANCELLATION

6.3 	Section 54 – Major Works

If the works promoter has not issued a notice of intended start date (10-day notice under Section 55) before the start date given on the 3-month notice, then it must issue a notice informing the street authority of its intention with respect to the proposed works within 2 days of that date.  A revised commencement date outside the 15-day period of validation for the Section 54 notice must be agreed with the street authority otherwise a new 3-month notice is required.

6.4 	Section 54 – Standard and Minor works Notice

If the works have not started within the validity period, then a further notice shall be sent to the authority to confirm

- Reasons for the work starting.

- Whether the work is intended to start and when.

If the works does not proceed, cancellation notice must be submitted no later than the end of the next working day after the expiry of the validity period.  A new Section 55 notice is required prior to commencement of the works.

BLANK PAGE

GENERAL RULES APPLYING TO NOTICEES

7.0 	Notices must be given electronically and must comply with the requirements set out in the Technical Specification for EToN.  The definitive format and contents of the notice is given in the Technical Specification and reproduced as Appendix A in this guidance notes.

7.1 	The initial “opening notice” must provide the following information: -

- Identification of works promoter

- Unique reference number for each job.

- Separate notice must be provided for each street (USRN).

- Description of works.

- Estimated start and end dates.

- The notice period and “date of issue” starts when the street authority receives the notice, not when it is sent.

- Location of the works.

- Correct USRN (unique street reference number obtained from Gwynedd’s National Street Gazetteer’s NSG).

- Grid co-ordinates.

Notices which do not provide the requisite information will be rejected by the street authority.

REFERENCE DOCUMENTS

- New Roads and Street Works Act 1991.

- Traffic Management Act 2004.

- The Street Works (Registers, Notices, Directions and Designations) (Wales) (Nº 2) Regulations 2008, SI 2008 Nº 540(2.52).

- The Street Works (fixed penalty) (Wales) Regulations 2008, SI 2008 Nº 102 (W.15).

- NRSWA 1991

Code of Practice for the co-ordination of Street Works and Works for Road Purposes and Related Matters (March 2008).

- NRSWA 1991

Technical specification for the Electronic Transfer of Notices (EToN).

- The Traffic Management (Guidance on Intervention Criteria) (Wales) Order 2007.

- Traffic Management Act 2004

Network Management

Duty Guidance.

Part 4.  	Streetworks Enforcement.

Section 43:	Direction of the Timing of Street Works. (Amendment to Section 56 of NRSWA)

The Street Authority have a duty to direct the timings (both days and times) of works if it is of the opinion that the works will cause serious disruption to traffic and the disruption will be reduced if the works are rescheduled to a certain time and/or day.

The Authority may give the undertaker directions as appropriate.  An Undertaker shall not be taken to have failed to fulfil any statutory duty to afford a supply or service because of a direction under this Section of the Act.

The form that shall be used under this direction is attached

Section 44: 	Directions as to placing of apparatus (Amendment to Section 56 of NRSWA)

The Street Authority have powers to give direction as to the placing of an apparatus if it is of the opinion that the works will cause serious disruption to traffic and the disruption will be reduced if the apparatus is placed in another street.

The Authority may by direction require the undertaker not to place the apparatus in a street. (but may not require him to place the apparatus in another street).

The conditions attached to this section are that.

a)		Disruption to traffic would be avoided or reduced if the apparatus were to be placed in an alternative street.

b) 		Placing the apparatus in the alternative street would be a reasonable way of achieving the purpose of which the apparatus is to be placed.

c) 		It is reasonable to require the Undertaker not to place the apparatus in the proposed street.

A direction under this section may be varied or revoked.

An undertaker who executes works in contravention of a direction under this section commits an offence and is liable on a summary conviction to a fine not exceeding level 5 on the standard scale.

Recommendation

Assess notices and take action where appropriate

Section 45:   The Street Works Register (Section 53 of NRSWA)

This is an amendment to Section 53 of the New Road and Street Works Act which requires the Street Authority to input details of:

a) 		Apparatus placed or proposed to be placed in the street.

b) 		Skips to be placed on the Street (Or other building material to be deposited).

c) 		Scaffolding to be placed on the Street.

Section 46:  Records of location of Apparatus (Section 79 of NRSWA)

Section 79 of the New Road and Street works Act 1991 is amended to include.

An undertaker may include a record of any item belonging to him stating the nature of the apparatus and whether it is in use.

Section 47:   Duty relating to the location of an unexpected apparatus (Section 80 of NRSWA)

This is where an Undertaker or Utility discovers an apparatus in the ground which is not recorded on PUSWA plans.  The discovering utility / HA has a duty to report it to the owner.

Section 48: Duty to inspect records (Section 52 of NRSWA)

This is where the Undertaker must prove that he has inspected prescribed statutory records before commencement of works.

Failure to comply can result in a fine up to Level 5 on the Standard summary scale.

Section 49:  Notice of streetworks (Section 54 of NRSWA, advanced notice of works)

Section 50:   Qualifications of supervisors and operatives (Section 67 of NRSWA)

The highway authority can, by notice, require an Undertaker who is executing street works to notify the name of the qualified person who will be on site at all times and each other street works qualified person who is also on site.

Recommendation

Circulate to internal Sections, assess notices and sites, and take action where appropriate

Section 51:  Restriction on works following substantial Roadworks (Section 58 NRSWA)

Following Substantial Road works (Section 58) or Substantial Street Works (Section 58 A) (works which are greater than 30 metres in length or which take up 1/3 of the carriageway or 2/3 of a footway or cycle track).  Restrictions may be made by the Highway Authority under Section 58 and/or Section 58A.

A notice must be served under Section 58 or 58A a minimum of 3 months prior to the start date of the works.

The Undertaker must respond with details of their works within 20 days of receiving a Section 58 (58A) notice.

The works must be carried out within 6 months of the start date contained in the Section 58 Notice.  Failure to do so will result in the notice being invalid.

Once the Road works has been completed.  The Highway Authority must give a notice to all the Utilities stating that the works have been completed and that the restrictions are in force.  (This should be prompted when the Highway Authority serves a works closed notice on the Street Authority).

Creating a restriction under Section 58A

To create a restriction following substantial Street Works (Not Road works) the Authority must, on receipt of a notice under Section 54 of NRSWA, publish a notice of its intention to create a restriction on the Authority’s website, giving at least 20 days’ notice of its intention to create a restriction.  The Street Authority must allow all other known works to be completed prior to the restriction coming into force.

Duration of restriction

The duration of the restriction depends upon the type of works

Works Type		Traffic Sensitive	Reinstatement

or reinstatement	Category 3 and 4

Category 1 and 2

1.  Reconstruction		5 year	5 years

2.  Resurfacing with or      		3 years	3 years

without change in levels.

3.  Other substantial road 		1 year	6 months

or street works

A Section 58 notice must be served on the following persons

1. 	All public utility companies.

2. 	The public sewer authority

3. 	The Bridge authority

4. 	The occupiers of any premises which have a frontage onto the street in question.

A copy of the notice must be sent by the works instigator (i.e. Highway Authority) to the authority’s street gazetteer officer (Currently Julie Jones).

A copy of the notice must be sent to the Street Works Section.

Copies of the notices shall be sent to the above persons/organisation by post or using an electronic method.  The Authority may, if they wish, publish a copy of the notice in the newspaper, however this is not a requirement.

Works promoter must reply to a Section 58 notice within 20 days from receipt, if they have works of their own to carry out on the street.

Works during restriction

The only works that is permitted to be carried out during a restriction is emergency works or works authorised by the Street Authority.

Customer connections; See clause 6.5.2.

Recommendation

Circulate a copy of this guidance notice internally and advise all Sections that they must comply.

Street Works Licences

Section 171 and Section 50

The Streetworks Section is required under Section 7.2 (Street Works Licences) to give a minimum of 10 days’ notice to undertakers and the Highway Authority of any proposed works which are to be carried out.

The Street Authority must inform each applicant of its following obligations under NRSWA.

1. 	Safety, signing, lighting and guarding.

2. 	Qualifications of operatives and supervisors.

3. 	Delays and obstructions.

4. 	Other undertakers’ apparatus.

5. 	Reinstatement.

6. 	Records of Apparatus.

7. 	The need of persons with disabilities.

The Street Authority should also inform the licensee of the following

1. 	Restrictions on streets subject to special controls

2. 	Relevant Section 58 and 58a notices.

3. 	That we may direct working times and recover the inspection costs.

Recommendation

Section 50

Liaise with other Street Authorities to see what they are proposing to do.

The Street Authority shall write to all contractors who have carried out works Under Section 50 licence and inform them that the Authority will require all applications to be submitted 1 month in advance.

Change the terms of the licence to reflect the above.  Amend the Section 50 procedure accordingly.

Section 171

Liaise with other Street Authorities to see what they are proposing to do.

Section 184 Works are deemed to be Major Highway Works, therefore fall within the jurisdiction of the Diversionary Works Code of Practice AND Section 84/85 NRSWA.  This would require a 3 Month Advance Notice which is consistent with both the Diversionary Works Procedures and TMA advance notice procedure.

As an Authority we need to decide if we are going to carry on as we are (time-line wise) by allowing early starts on Section 171 works or do we insist on a three-month notice period?

7.3	Registerable works

The T.M.A. has introduced additional criterias to this section, namely, that “works for road purposes” (highway works) is now notifiable in the same way as street works.

Recommendations

The Authority have put measures into place to arrange noticing to be applied via Eton 4.  This process will need to be tightened up and monitored, possibly using K.P.I.’s as a tool.

Alan to liaise with other Street Authorities to see what they are proposing to do and come back with a further recommendation.

8        Notice Requirements and Validity

There is too much for me to go into detail in this brief.  However, I enclose a copy of the chapter which is easy enough to understand.  The main changes are the notice requirement time shown in Table 8.1 whereby

Major works notice is increased from 	1 month        	to         3 months

Standard works notice is increased from	7 days           	to        10 days

Minor works from	Daily whereabouts   to          3 days

Immediate / urgent	2 hrs after works commence

There are other notices that need to be served in connection with the above, please see table 8.1.

ADDITION TO APPENDIX 1/17 (CONTINUED):

TRAFFIC SENSITIVE ROUTES

| Town / Street | USRN |  | Designation |
| --- | --- | --- | --- |
| Amlwch |  |  |  |
| Salem Street | 18300459 | Easter/Bank Holidays | All year |
| Queen Street | 18300453 | Easter/Bank Holidays | All year |
| Beaumaris |  |  |  |
| A545 - Castle Street, Beaumaris | 18300042 |  | All year/Tourist |
| A545 - Church Street, Beaumaris | 18300077 |  | All year/Tourist |
| Benllech |  |  |  |
| Beach Road | 18300598 |  | July & August |
| Bay View Road | 18300051 |  | July & August |
| Cemaes Bay |  |  |  |
| High Street | 18300423 | Easter/Bank Holidays | July & August |
| Bridge Street | 18300391 | Easter/Bank Holidays | July & August |
| Ffordd y Traeth | 18300413 | Easter/Bank Holidays | July & August |
| Holyhead |  |  |  |
| Stanley Street | 18301013 |  | All year |
| Market Street | 18301012 |  | All year |
| Boston Street | 18301305 |  | All year |
| Thomas Street | 18301323 |  | All year |
| William Street | 18301333 |  | All year |
| Trearddur Square | 18301326 |  | All year |
| Hill Street | 18301341 |  | All year |
| Cambria Street | 18301315 |  | All year |
| Newry Street | 18301223 |  | All year |
| Newry Fawr | 18301220 |  | All year |
| Prince of Wales Road | 18301001 |  | All year |
| Victoria Road | 18300962 |  | All year |
| London Road | 18300963 |  | All year |
| Llanfairpwll |  |  |  |
| Ffordd Caergybi | 18300957 |  | All year |
| Llangefni |  |  |  |
| Glanhwfa Road | 18300041 |  | All year |
| High Street | 18300058 |  | All year |
| Bridge Street | 18300060 |  | All year |
| Field Street | 18300085 |  | All year |
| Ffordd Glandwr | 18310340 |  | All year |
| Church Street | 18300084 |  | All year |
| Menai Bridge |  |  |  |
| High Street | 18300045 |  | All year |
| Ffordd Telford | 18300047 |  | All year |
| Stryd y Bont | 18300046 |  | All year |
| Ffordd y Ffair | 18300916 |  | All year |
| Cil Bedlam (Dale Street) | 18300105 |  | All year |
| Ffordd Cadnant | 18300044 |  | All year |
| Town / Street | USRN |  | Designation |
| Newborough |  |  |  |
| Church Street | 18301183 |  | July & August |
| Rhosneigr |  |  |  |
| Ffordd Maelog | 18300934 |  | July & August |
| Station Road | 18300932 |  | July & August |
| Ffordd Maelog | 18300933 |  | July & August |
| Beach Road | 18301592 |  | July & August |
| High Street | 18301034 |  | July & August |
| Trearddur Bay |  |  |  |
| Lon Isallt | 18301010 |  | July & August |
| Valley |  |  |  |
| Station Road | 18300994 |  | All year |
| Holyhead Road | 18300964 |  | All year |
| A4080 - Link 1 | 18300929 |  | Tourist |
| A4080 - Link 10 | 18300945 |  | Tourist |
| A4080 - Link 14 | 18300955 |  | Tourist |
| A4080 - Link 9 | 18300942 |  | Tourist |
| A4080 - Link 13 | 18300954 |  | Tourist |
| A4080 - Link 12 | 18300951 |  | Tourist |
| A4080 - Link 7 | 18300938 |  | Tourist |
| A4080 - Link 6 | 18300936 |  | Tourist |
| A4080 - Link 10 | 18300944 |  | Tourist |
| A4080 - Link 12 | 18300950 |  | Tourist |
| A4080 - Link 7 | 18300937 |  | Tourist |
| A4080 - Link 6 | 18300935 |  | Tourist |
| A4080 - Link 14 | 18300956 |  | Tourist |
| A4080 - Link 11 | 18300949 |  | Tourist |
| A4080 - Link 8 | 18300940 |  | Tourist |
| A4080 - Link 8 | 18300941 |  | Tourist |
| A4080 (T) | 18300034 |  | Tourist |
| A5 (T) - Link 1 | 18300959 |  | Tourist |
| A5025 - Link 1 | 18300033 |  | Tourist |
| A5025 - Link 1 | 18300001 |  | Tourist |
| A5025 - Link 18 | 18300024 |  | Tourist |
| A5025 - Link 14 | 18300016 |  | Tourist |
| A5025 - Link 25 | 18300032 |  | Tourist |
| A5025 - Link 5 | 18300005 |  | Tourist |
| A5025 - Link 15 | 18300019 |  | Tourist |
| A5025 - Link 9 | 18300009 |  | Tourist |
| A5025 - Link 23 | 18300030 |  | Tourist |
| A5025 - Link 24 | 18300031 |  | Tourist |
| A5025 - Link 8 | 18300008 |  | Tourist |
| A5025 - Link 17 | 18300021 |  | Tourist |
| A5025 - Link 19 | 18300025 |  | Tourist |
| A5025 - Link 22 | 18300029 |  | Tourist |
| A5025 - Link 7 | 18300007 |  | Tourist |
| A5025 - Link 13 | 18300015 |  | Tourist |
| A5025 - Link 20 | 18300026 |  | Tourist |
| A5025 - Link 6 | 18300006 |  | Tourist |
| A5025 - Link 3 | 18300003 |  | Tourist |
| A5025 - Link 4 | 18300004 |  | Tourist |
| Town / Street | USRN |  | Designation |
| A5025 - Link 16 | 18300020 |  | Tourist |
| A5025 - Link 10 | 18300011 |  | Tourist |
| A5025 - Link 21 | 18300028 |  | Tourist |
| A5025 - Link 12 | 18300014 |  | Tourist |
| A5025 - Link 11 | 18300013 |  | Tourist |
| A5108 - Link 3 | 18300040 |  | Tourist |
| A5108 - Link 2 | 18300039 |  | Tourist |
| A5108 - Link 1 | 18300038 |  | Tourist |
| B4545 - Link 3 | 18300996 |  | Tourist |
| B4545 - Link 2 | 18300995 |  | Tourist |
| A545 - Stryd Fawr, Porthaethwy | 18300045 |  | Tourist |
| A545 - Ffordd Cadnant, Porthaethwy | 18300044 |  | Tourist |
| A545 - Ffordd Beaumaris | 18300043 |  | Tourist |
| Ffordd Traeth Bychan | 18300301 |  | Tourist |
| Red Wharf Bay | 18300172 |  | Tourist |
| Red Wharf Bay | 18300171 |  | Tourist |
| Red Wharf Bay | 18300170 |  | Tourist |
|  | Table 1 |  |  |

ADDITION TO APPENDIX 1/17 (CONTINUED):

| Type of Work |  | Major Works (3 months’ notice min.) | Standard Works (10 days’ notice min.) | Minor Works (3 days’ notice min.) | Emergency Works (next working day/within 2 hours) |
| --- | --- | --- | --- | --- | --- |
|  |  | (duration of 10 days or more) | (Duration of 4 to 10 days) | (Duration of 1 to 3 days) | (dangerous) |
| FOOTWAY | Alterations to level of carriageway or footway |  |  |  |  |
|  | Footway reconstruction/widening |  |  |  |  |
|  | New footway more than 30m |  |  |  |  |
|  | Footway surfacing over 30m (including slurry seal) |  |  |  |  |
|  | New kerb line over 30m |  |  |  |  |
|  | Replacement or repairs to kerbs over 30m in a 500m section |  |  |  |  |
|  | New footway less than 30m in length |  |  |  |  |
|  | Repairs or replacement to existing footway less than 30m in length, over 2m in length within 500m section |  |  |  |  |
|  | New kerbs less than 30m in length |  |  |  |  |
|  | Repairs or replacement to existing kerbs less than 30m in length but more than 2m in length |  |  |  |  |
|  | Replacement of damaged paving, block or flags under 2m in length within 500m section |  |  |  |  |
| SURFACING | Highway improvement/widening |  |  |  |  |
|  | Alterations to level of carriageway or footway |  |  |  |  |
|  | Surfacing works |  |  |  |  |
|  | Surface dressing |  |  |  |  |
|  | Road markings including cats’ eyes |  |  |  |  |
|  | Patching not greater than 20m² within 500m section |  |  |  |  |
| DRAINAGE | Drainage works more than 30m in length |  |  |  |  |
|  | New drainage works less than 30m in length in 500m section |  |  |  |  |
|  | Repairs/replacement to existing drainage less than 30m in length in 500m section |  |  |  |  |
|  | New replacement or repairs gully |  |  |  |  |
|  | New replacement or repairs manhole |  |  |  |  |
|  | New replacement or repairs pipe works |  |  |  |  |
|  | Trial holes |  |  |  |  |
| OTHER | Fencing and walling work more than 30m in a 500m section |  |  |  |  |
|  | Construction or removal of road hump |  |  |  |  |
|  | Works carried out under Section 171 of Highways Act |  |  |  |  |
|  | Provision of cattle grid |  |  |  |  |
|  | Tunnelling or boring under highway |  |  |  |  |
|  | Erection of signpost, lighting column, traffic signals |  |  |  |  |
|  | Lamp or electrical testing, bulb and sensor renewal, column painting |  |  |  |  |

APPENDIX 1/18:

TEMPORARY DIVERSION OF TRAFFIC

1A	Temporary Diversion of Traffic Specified by the Overseeing Organisation

No off-site traffic diversions are proposed.

1B	Requirements for the Execution of Temporary Diversions Necessitated by the Works (excluding A above).

i)	Details of Geometric Standards to be applied for Temporary Diversions are shown on Table 1 Geometric Standards (see also Appendix 1/17 Table 2).

ii)	Lane widths specified in Table 1 shall be increased where curves occur to provide the necessary swept paths for all classes of heavy goods vehicles and buses.

iii)	Where, due to construction there is a difference in level exceeding 0.3 metres between lanes, the lane edges shall be delineated with 300mm x 300mm baulks of timber, fastened to the road and to each other, painted in an alternative red and white pattern, and the ground between lanes shall slope at no more than 1 in 1.

iv)	The maximum difference in levels between lanes shall not at any time exceed 1.0 metre.  A 1.0 metre reservation is also required at the edge of a lane where there is a drop exceeding 0.3 metres in height and in this case baulks of timber shall be employed to delineate the carriageway.

v)	All temporary diversions/running surfaces and ramps to be cement or bituminous bound and maintained in a pothole free condition to the satisfaction of the Overseeing Organisation.

vi)	Steel plates are not permitted within the Highway during the installation of ducts and Drainage works for the purposes of carrying vehicular traffic without the prior approval of the Overseeing Organisation.

Table 1:

Geometric Standards

| TWO WAY WORKING |  |  | ONE WAY WORKING |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CATEGORY 1A | CATEGORY 1B |  | CATEGORY 2A | CATEGORY 2B | FOOTPATH CATEGORY IV | PRIVATE ACCESS CATEGORY V |
| Effective carriageway width (minimum) | 7.3m or 2 x 3.6m | 5.5m or 2 x 3.0m |  | 3.0m | 2.5m | 1.5m (between fence of which 1.0m surfaced) | 2.5m (ref. Para B(ii)) |
| Effective carriageway width (minimum) |  |  |  | 3.7m | 3.7m |  |  |
| Horizontal radius to Centreline (minimum) | 250m | 130m |  | 130m | 40m |  | 20m |
| Sag Vertical Radius (minimum) | 1000m | 500m |  | 500m | 100m |  | 100m |
| Summit Vertical Radius (minimum) | 2000m | 1000m |  | 1000m | 100m | 20m | 20m |
| Sight distance (min.) (from 1.05m above road surface to 1.05m above road surface) | 100m | 60m |  | 60m | 30m | 20m | 20m |
| Gradient (maximum) | 7% | 7% |  | 7% | 7% | 10% | 10% |
| Length of One-way | N/A | N/A |  | 200m | 200m | N/A | 150m working (max). |

Table 2

(i)	Highways open to vehicles

| DESCRIPTION | DRAWING NO. | CONSTRUCTION REQUIREMENTS GEOMETRIC REQUIREMENTS: Refer to App1/18 Table | MAINTENANCE REQUIREMENTS: Refer to App1/17 Para 2 |
| --- | --- | --- | --- |
|  | Scheme Drawing |  | None |

As Table 1 for individual schemes or as amended by Works Tasks or scheme drawing.

(ii)	Other Highways and Private Rights of Way

| Private means of access | Scheme drawing | V | None |
| --- | --- | --- | --- |

As Table 1 for individual schemes or as amended by Works Tasks or scheme drawings and to be always kept open for access.

Table 3 Proposed Works to Adjacent Properties

| LOCATION/CLIENT | TYPE OF WORKS | ACCESS | NOTES |
| --- | --- | --- | --- |
| 1 Details to be provided as part of Works Task or as identified on scheme drawings. |  |  |  |

APPENDIX 1/19:

ROUTING OF VEHICLES

1	Permitted Access Routes to and From the Site

1.1	See table below and plan S9 in Contract Drawings.

| LOCATION | CLASS OF ROAD | GRID REFERENCE | RESTRICTION |
| --- | --- | --- | --- |
| Porth y Felin, Holyhead | Unclassified road | SH 239 832 | Height Restriction 10’ 9” (3.225m) |
| Malltraeth Viaduct | Class III road | SH 413 691 | Height Restriction 15’ 0” (4.500m) |
| Rhosneigr Overbridge | A4080 | SH 328 738 | Height Restriction 9’ 6” (2.850m) |
| Shire hall Overbridge Llangefni | A5114 | SH 458 754 | Height Restriction 13’ 6” (4.050m) |
| Aberffraw “Glanrafon” Overbridge | Class III road | SH 364 716 | Height Restriction 14’ 3” (4.275m) |
| Aberffraw “Graig” Overbridge | Class III road | SH 377 710 | Height Restriction 15’ 6” (4.650m) |
| Aberffraw “Tre Dafydd Uchaf” Overbridge | Class III road | SH 378 708 | Height Restriction 18’ 0” (5.49m) |
| Llangaffo Overbridge | Class III road | SH 437 692 | Height Restriction 13’ 3” (3.975m) |
| Rhosgoch Overbridge | Unclassified road | SH 411 897 | Height Restriction 10’ 0” (3.0m) |
| Rhosgoch Overbridge | Class III road | SH 413 886 | Height Restriction 13’ 9” (4.19m) |
| Llanfairpwll Overbridge Nr Carreg Môn | Unclassified road | SH 537 713 | Height Restriction 13’ 0” (3.9m) |
| Burwen Road, Amlwch | B5111 | TBC | Weight Restrictions |
| Cemaes, Harbour Bridge | TBC | TBC | Weight Restrictions |
| Near Rhoscefnhir, Pentraeth | TBC | TBC | Weight Restrictions |
| Pont Fatri Cymunod Nr Bryngwran | TBC | TBC | Weight Restrictions |
| London Road, Holyhead | A5 |  | Weight Restrictions |

2	Movement of Machinery and Plant across Public Roads

2.1	Movement of Machinery and Plant across Public Roads is permitted.

3	Temporary Structure for Construction Traffic Spanning Areas used by the Public

3.1	None are proposed.

APPENDIX 1/21:

INFORMATION BOARDS

The Overseeing Organisation has a typical partnership information board detail drawing No. S5. This is a street works requirement.

APPENDIX 1/22:

CLAUSE 122 PROGRESS PHOTOGRAPHS

1	PROGRESS PHOTOGRAPHS

Set of progress photographs as required by the Overseeing Organisation.  Minimum resolution 	600 x 480 DPI medium quality.  Format to be either JPEG or BMP to be supplied on CD Rom for larger quantity or file share software.

2	PROGRESS PHOTOGRAPHS

Colour progress photographs shall be taken at locations to be agreed by the Overseeing Organisation at weekly intervals or at such intervals as the Overseeing Organisation may direct.

Each set of prints shall be supplied and handed over to the Overseeing Organisation upon substantial completion of the works.

APPENDIX 1/23:

SUBSTANCES HAZARDOUS TO HEALTH

Details will be provided by the Overseeing Organisation for individual schemes as part of Works Task or as identified on Scheme Drawings where substances hazardous to health are known to exist.

APPENDIX 1/24:

QUALITY MANAGEMENT SYSTEM

1. 	See DOCUMENT 3B1: RESPONSE DOCUMENT (Qualitative):

5.4 APPENDICIES 2/0

APPENDIX 2/1:

LIST OF BUILDINGS, ETC. TO BE DEMOLISHED OR PARTIALLY DEMOLISHED

List the buildings and structures to be demolished or partially demolished, together with any further requirements.

NONE.

APPENDIX 2/2:

FILLING OF TRENCHES AND PIPES

All existing gullies, sewers, drains and ducts that are to be abandoned are to be demolished or excavated to formation level and then treated as follows: -

a)	for gullies, the remaining void backfilled with concrete ST1, or the structure removed and replaced with General Fill material complying with Clause 601 and Table 6/1 and compacted in compliance with Clause 612 as directed by the Overseeing Organisation.

b)	for sewers, drains, ducts etc., the remaining sewer grouted with 1:10 cement; PFA mix or removed and replaced with General Fill material, all in accordance with Clause 506(3) as directed by the Overseeing Organisation.

Prior to grouting, all gullies, sewers and drains shall be thoroughly cleaned of silt and sediment and flushed with water for the approval of the Overseeing Organisation.

APPENDIX 2/3:

RETENTION OF MATERIAL ARISING FROM SITE CLEARANCE

Given below are a table of site clearance items and their delivery or required actions.

These items are indicated on the site clearance drawings for individual Works Task.

Items to be retained as directed by the Overseeing Organisation for individual Works Task.

| Description | Location | Delivered to | Requirements |
| --- | --- | --- | --- |
| Kerbs, quadrants Paving Setts Chamber covers Gully gratings & frames Gates and fencing Safety fence components Traffic signs Bollards Road studs Traffic signals Road lighting columns Luminaires Electrical equipment Communications equipment Cables |  |  |  |

(The column headed ‘Requirements’ is available for including such instructions as:)

i)    Disconnection of electrical supplies.

ii)   Transportation of equipment.

iii)  Stacking/storage of material.

iv)  Reinstatement of voids left by removal of equipment.

APPENDIX 2/4:

EXPLOSIVES AND BLASTING FOR SITE CLEARANCE

1.	The Contractor’s attention is drawn to the measures for the control of noise and vibration which are included in Appendix 1/9.

2.  	Explosives shall not be used without the consent of the Overseeing Organisation.

APPENDIX 2/5:

HAZARDOUS MATERIALS

The Contractor shall agree with the Overseeing Organisation and the Waste Disposal Authority acceptable measures for the handling and disposal of hazardous materials found in site clearance as instructed on individual Works Tasks.

5.5 APPENDIX 3/0

APPENDIX 3/1:

FENCING, GATES AND STILES

1.	Temporary Fencing

i)	 Requirements for temporary fencing if different from requirements of sub-clause 302.1 and 303.1.

ii)	 Timing of removal of temporary fencing if different from sub-clause 302.2.

iii)	 Requirements for any preservation treatment to temporary fencing (303.3).

2.	Timber Quality

i)	 Requirements for timber if different from the requirements of sub-clause 304.3.

3.	Fittings

i)	 Requirements for bolts, screws and nuts if different from the requirements of sub-clause 305.1.

4.	Permanent Fencing: Wooden Fencing, Gates and Stiles including Planting Works Fencing

i)	 Flowing alignment and trimming ground to regular level on fence line.

ii)	 Requirements for joining permanent fencing to existing hedges, fences and to other structures    if different from the requirements of sub-clause 306.1.

iii)	 Details of additional stock proofing required.

iv)	 Details of painting required.

v)	 Concrete surround to base of posts.

vi)   Details of fitting required.

vii)  Details of type of Planting Works Fencing (Location and details).

viii) Details of security treatment below access gates in fencing that incorporates wildlife mesh.

5.	Permanent Fencing: Wire Dropper Fencing

i)	 Flowing alignment and trimming ground to regular level on fence line.

ii)	 Requirements for joining permanent fencing to existing hedges, fences and to other structures if different from the requirements of sub-clause 306.1.

iii)	 Details of additional stock proofing required.

iv)	 Requirements for painting with plastic paint in accordance with sub-clause 312.1.

v)	 Zinc coated wire only.

vi)	 Spacing of posts and requirement for turning posts.

6.	Wire Mesh to Permanent or Existing Fencing

i)	 Details of wire mesh attachments to fencing including the appropriate side of an existing fence to which the mesh is fixed.

ii)	 Treatment of turned-out portion of netting (Location and details).

7.	Preservation of Timber

i)	 Details of preservative to be used if different from sub-clause 311.1 and 311.2.

8.	Badger Gates

i)	 Details of requirements for badger gates including whether two-way gates are required (Location and Details).

9.	Fenced Tree Guards

i) 	Details of requirements for fenced tree guards (Location and Details).

Other

i)	 Colour of plastic coating to high tensile wire (2605.1, 2605.2, 2605.3 and 2605.4).

5.6 APPENDIX 4/0

Please see contract drawings Volume 3 for locations of Vehicle Restraint Systems

5.7 APPENDICIES 5/0

APPENDIX 5/1:

DRAINAGE REQUIREMENTS

1.	The detailed requirements of the drainage for the scheme are shown on Drawings to be issued with Individual Works Task.

2.	Other drainage requirements are contained within Volume 4 of the Contract Documents and Highway Construction Details Section 1 Series F as listed in Appendix 0/4.

3.	Pipe and bed combinations for Carrier Drains will be detailed with every Works Task and in accordance with (Advice Note HA.40).

4.	Pipe stiffness and impact resistance for plastic pipes shall comply with BS 4962:1989 specifically,

Pipe stiffness: -	Ultimate pipe stiffness (50-year life), STES 50yr must be greater than 350 Pa when tested in accordance with Appendix B and C of BS 4962:1989.

Initial pipe stiffness, STIS (5 min) shall be

| Class | Test | Acceptable Limits Lower | Within: - Upper | Note |
| --- | --- | --- | --- | --- |
| 1A 1B 4 6F2 8 | mc mc - 10% fines mc | optimum – 2% optimum – 2% - 50Kn optimum – 2% | optimum +2% optimum +2% - - optimum +2% | see note(i) see note(i) see note(ii)&(iii) see note(i) |

(i)   90% of STES value (for STES values 350 - 470 Pa)

(ii)  75% of STES value (for STES values greater than 470 Pa)

Impact Resistance: -	This should comply with the testing requirements and procedure stipulated in BS 4962:1989 (Appendix E, Procedure E5).

5.	Joints for pipes which are intended to form part of the Highway drainage system will have to be watertight.

6.	Details of connections of existing drains/land drains to the new Highway drainage system are shown on Drawings to be issued with individual Works Task.

7.	Chamber covers, gully gratings and frames to be constructed in compliance with BS 497: Part 1.  Gully gratings shall be heavy duty Grade A at all carriageway/parking bays locations.  Chamber covers and frames shall be recessed type of dimensions as specified in Price List capable of withstanding a minimum of 5kN (0.5t) loading.

APPENDIX 5/2:

SERVICE DUCT REQUIREMENTS

1.	Service duct requirements are orange 100mm internal diameter ducts.  Depth to duct in carriageway - 750mm and 450mm in footway, on bedding Type Z to Highway Construction details Drawing F1.  Warning tape saying ‘Cable Buried Below’ to be buried 200mm below finished surface and above ducting.

2.	The location and specific details of service ducts are shown in the drawings to be issued with Individual Works Task.

3.	The details of duct marker blocks are shown on Highways Construction Details drawing No.  I 1

4.	Details of duct mandrel are shown on Highway Construction drawing No.  I 2

5.	Service duct requirements adjacent to trees to be carried out in accordance with BS5837 1991.  ‘Trees in relation to Construction’.  Attention drawn to paragraphs 7 and 8 as enclosed.

6.	Ducts are to be laid, so far as is practicable to a straight line and at an even gradient.  Any alterations to the gradient due to the topography of the site will be accommodated by minor deflections along the length of the pipes.  No deflections along the length of the pipes.  No deflection is to occur at pipe joints.

The principal changes in direction are to be accommodated at inspection chambers.  Inspection chambers to be 290 x 275 mm, 450 x 300 mm, 450 x 450 mm, or 600 x 600 mm with Grade A cover and frame.

All bends shall be to a radius not less than 600mm.

7.	Electrical installation inspection chambers shall be rotationally moulded polyethylene sections and shall be of strong robust construction to prevent distortion during backfill and shall positively interlock with a minimum of 50mm skirt to prevent ingress and ensure chamber integrity. Where chambers are to be constructed in footpaths, paved areas, grassed areas or flower beds these shall be installed as per drawing SL/SD 1 and be provided with frame and cover to suit.

8.	Chambers shall be 610 x 460 internal dimensions precast concrete with mortar joints between segments.  Where chambers are to be constructed in footpaths, grassed areas or flower beds these shall be placed on a foundation of 100mm thick class ST2 concrete and be provided with a medium duty cover.

Where chambers are to be constructed in roads or parking areas these shall be with a 150mm thick bed and surround of concrete class ST2 which will extend to below the cover slab.  The covers will be of heavy-duty type.

APPENDIX 5/3:

location of CHANNELS

Please see contract drawings Volume 3

APPENDIX 5/8:

LOCATION OF GRILLAGES

Please see contract drawings Volume 3

5.8 APPENDICES 6/0

APPENDIX 6/1:

REQUIREMENTS FOR ACCEPTABILITY AND

TESTING OF EARTHWORKS MATERIALS

1. 	The classification and confirmation of acceptability of the earthwork’s materials shall be carried out by the Contractor at the point of deposition for imported materials.

The Contractor shall submit 2 copies of all test results to the Overseeing Organisation within one working day of the completion of the test.  The copies shall be signed by the Contractor’s responsible engineer/technician.

The Contractor shall allow for the following tests:

TABLE 1

i)	20 No. moisture content,

ii)	10 No. Dry Density/Moisture Content Relation (2.5kg rammer) BS 1377 Test 12,

iii)	10 No. Particle size distribution BS 1337 Test 7.

2. 	Tests and Acceptability Limits (Supplementary to Table 6/1)

Notes:

i)	Optimum moisture content determined from BS1377 Test 12.

ii)	Regulating layers of sub-base/capping material are to be formed of similar material types to these existing:

iii)	Sub-base layers above regulating layers are to be formed of similar material types to those used in the regulating layer.

APPENDIX 6/2:

REQUIREMENTS FOR DEALING WITH CLASS U2

UNACCEPTABLE MATERIAL

1. 	Refer to drawing issued with Individual Works Task for location of known unacceptable material.

2. 	Details of materials encountered to be notified to the Overseeing Organisation.

3. 	The Contractor shall agree with the Overseeing Organisation measures for the handling and disposal of class U2 unacceptable material as instructed in individual Works Tasks.

4.	Contractor’s method statement to include: -

i)  Method of excavation, precautions and requirements for handling.

ii)  Special requirements for dealing with leachate and contaminated water.

iii) Requirements for special drainage and for sealing exposed surfaces of contaminated materials.

5.  The new recycling targets set by the Welsh Assembly include domestic waste and all municipal            waste arising, this in effect allows the Local Authority to recycle all the waste arising on the Island         whatever the source.

APPENDIX 6/3:

REQUIREMENTS FOR EXCAVATION, DEPOSITION AND COMPACTION

(OTHER THEN DYNAMIC COMPACTION)

1. 	No area of sub-base more than 50m long will be allowed to be exposed for more than 24 hours without surface protection.

2.	The Contractor is to have due regard to the appropriate authorities as regards disposal of excavated material.

3.	Work to existing watercourses to comply with the Special requirements of the Environment Agency.  Excavated material from watercourses to be removed to tip off site.

APPENDIX 6/5:

GEOTEXTILES USED TO SEPARATE EARTHWORKS MATERIALS

1	Location of geotextiles and geogrids used to separate earthworks materials will be as directed in Individual Works Task and typically will be used for separation of subsoil and structure drainage from earthworks, separation of sub-base and earthworks layers and for edge of carriageway strengthening operations.

2	Geotextiles for drainage applications shall be of synthetic materials in the following forms based on Terram Geotextiles or similar approved supplier:

Terram Global Office, A Berry Plastics Company, Fiberweb Geosynthetics Ltd, Blackwater Trading Estate, The Causeway, Maldon, Essex, CM9 4GG.

3	Drainage Applications to separate earthworks materials:

a)	Terram Filtram 1A1

- Cellars

- Small retaining walls

- Clear polymer extruded grid covered on both sides with Terram non-woven filter by full thermal bonding.

b)	Terram Filtram B1

- Vertical structure application not requiring waterproofing protection

- Black polymer extruded grid core covered on one side by Terram non-woven filter by full thermal bonding.

c)	Terram Filtram 1B1

- Vertical or inclined structure application, ground drains and protection of structural waterproofing.

- Black polymer extruded grid core covered on both sides with Terrain non-woven filter by full thermal bonding.

d)	Terram DC-1C

- High flow rate drainage applications, horizontal structure and ground drains

- Resilient polymer core with Terram non-woven filter bonded to one side.

Installation and testing to manufacturers recommendations, reference Terram Geotextiles Drainage composite installation guide and data sheets.

4	Geotextiles for separation of sub-base and earthworks layers shall be of synthetic materials in the following forms based on Tensar Netlon Geotextiles or similar approved supplier Tensar, Netlon Linked, Kelly Street, Blackburn, Lancashire, BB2 4PJ (Tel: (011254) 262431).

a)	Road applications - Sub-grade/Sub-base separation

| Netlon Geotextile Type | 151-1 | 201-3 | 351-4 |
| --- | --- | --- | --- |
| Subgrade Type | Any | Any | Any |
| Aggregate Type | 100mm down (inc SHW Clause 609) | 150mm down | 250mm down |
| Form non-woven needle punched, manufactured from staple fibres |  |  |  |
| Polymer | polypropylene | polypropylene | polypropylene |
| Weight (DIN 53854) (g/m2) | 150 | 225 | 325 |
| Thickness (DIN 53855) (mm) | 2.3 | 3 | 3.5 |
| Tensile Strength (DIN 53857) Longitudinal (kN/m) Transverse (kN/m) | 2.5 6.0 | 9.0 15.0 | 15.0 22.0 |
| Elongation (DIN 53857) Longitudinal (%) Transverse (%) | 75 60 | 75 60 | 75 60 |
| Effective Opening Size Dw (mm) | 0.17 | 0.12 | 0.09 |
| Permeability k (DIN 53936/1) @ 20kN/m2 (m/s) | 4.0 x 10-3 | 2.9 x 10-3 | 2.3 x 10-3 |
| Push through load (N) and elongation (%) (DIN54307) | 770/70 | 1800/70 | 2800/70 |
| Roll size (m) | 5.75 x 100 | 5/75 x 100 | 5.75 x 100 |

b)	Earthworks/Drainage Applications

| Filter Drain Applications | French Drain | Drainage blanket | Drainage blanket |
| --- | --- | --- | --- |
| Netlon Geotextile Type | 151-1 | 201-3 | 351-4 |
| Filter Media | 63mm max. stone size | Between 60% and 100% of grading passing 63mm | Less than 60% passing 63mm |
| Manufacture non-woven needle punched, manufactured from staple fibres |  |  |  |
| Polymer | polypropylene | polypropylene | polypropylene |
| Weight (DIN 53854) (g/m2) | 150 | 225 | 325 |
| Thickness (DIN 53855) (mm) | 2.3 | 3 | 3.5 |
| Tensile Strength (DIN53857) Longitudinal (kN/m) | 2.5 6.0 | 9.0 15.0 | 15.0 22.0 |
| Elongation (DIN 53857) Longitudinal (kN/m) Transverse (%) | 75 60 | 75 60 | 75 60 |
| Effective Opening Size Dw (mm) | 0.17 | 0.12 | 0.09 |
| Permeability k (DIN53936/1) @20kN/m2 (m/s) | 4.0 x 1—3 | 2.9 x 10-3 | 2.3 x 10-3 |
| Push through load (N) and elongation (%) (DIN 54307) | 770/70 | 1800/70 | 2800/70 |
| Roll Size (m) | 5.75 x 100 | 5.75 x 100 | 5.75 x 100 |

c)	General Earthworks Applications

|  | Pore water pressure control, capilliary barriers |
| --- | --- |
| Netlon Geotextile Type | 514R |
| Soil Type | normally fine-grained soils, e.g. silt, clay |
| Form | non-woven needle punched, manufactured from staple fibres |
| Polymer | Polypropylene |
| Weight (DIN 53854) (g/m2) | 500 |
| Thickness (DIN53855) (mm) At a load of 2 kN/m2 At a load of 200 kN/m2 | 5.0 .0 |
| Tensile Strength (DIN 53857) Longitudinal (kN/m) Transverse (kN/m) | 8 12 |
| Elongation (DIN 53857) Longitudinal (%) Transverse | 100 80 |
| Effective Opening Size Dw (mm) | 0.12 |
| Permeability k (DIN 53936/1) @ 20 kN/m2 (m/s) @ 200 kN/m2 (m2/s) | 93.5 x 10-3 6.8 x 10-4 |
| Transmissivity @ 2 kN/m2 (m2/s) @ 200 kN/m2 (m2/s) | 9.3 x 10-3 1.4 x 10-3 |
| Roll size (m) | 2.2 x 5.0 |

d)	Revetment Stabilisation

| Netlon Geotextile Type | 601S |  |  | 600 | 1004R |  | 805RS |  | 1000 |  | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subsoil Type | sand, gravel |  |  | silt | silt, clay |  | "Problem soils" |  | any |  | any |
| Form non-woven needle punched, manufactured from staple fibres |  |  |  |  |  |  |  |  |  |  |  |
| Polymer |  | polyester and polypropylene mix | polyester and polypropylene polyamide mix |  |  | polyester and polypropylene duplex |  | polyester and polypropylene duplex |  | polyester | polyester |
| Weight (g/m2) |  | 620 | 603 |  |  | 814 |  | 1700 |  | 1000 | 2000 |
| Weight of armour units placed on geotextile (kg) |  | 500 | 500 |  |  | 500 |  | 500 |  | 1000 | >2500 |
| Weight of armour units dropped on geotextile with a fall height of 1m (kg) |  | 125 | 125 |  |  | 125 |  | 125 |  | 150 | >250 |
| Thickness |  | 6.6 | 5.3 |  |  | 6.6 |  | 165 |  | 4.8 | 8.5 |
| Tensile Strength Longitudinal (kN/m) Transverse (kN/m) |  | 13.6 22.7 | 16.6 34.5 |  |  | 14.9 34.1 |  | 20.3 39.8 |  | 20.0 30.0 | 60.0 90.0 |
| Elongation Longitudinal (%) Transverse (%) |  | 129 52 | 96 56 |  |  | 111 49 |  | 125 49 |  | 55 30 | 65 30 |
| Effective opening size Dw (mm) |  | 0.15 | 0.08 |  |  | 0.07 |  | 0.08 |  | 0.07 | 0.07 |
| Permeability k |  | 6.6 x 10-3 | 1.8 x 10-3 |  |  | 4.9 x 10-3 |  | 4.6 x 10-3 |  | 2.0 x 10-3 | 6.8 x 10-4 |
| Roll size (m) |  | 5.8 x 5.0 | 5.8 x 5.0 |  |  | 4.8 x 5.0 |  | 4.6 x 25 |  | 5.8 x 50 | 4.8 x 50 |

Installation and testing to manufacturers recommendations, reference Netlon Geotextiles installation and data sheets.

5	Geogrids to stabilise weak load bearing soils shall be Tensar Geogrids SS20, SS30, SS40 or similar approved incorporating non-woven needle punched geotextiles for filtration as directed Geogrids shall be installed and tested in accordance with the manufacturer’s instructions in locations as directed by the Overseeing Organisation in Individual Works Task.

APPENDIX  6/6:

FILL TO STRUCTURES AND FILL ABOVE

STRUCTURAL FOUNDATIONS

1	Fill to structures and above structural foundations shall be in Class 6N material unless otherwise directed by the Overseeing Organisation.

APPENDIX 6/7:

SUB-FORMATION AND CAPPING AND PREPARATION AND

SURFACE TREATMENT OF FORMATION

Refer to drawing issued with Individual Works Task showing locations of capping areas, and surface treatment of formation.

APPENDIX 6/8:

TOPSOILING

Topsoiling

Weed Act 1959 (and amendments)

1.	For the duration of the Works the Contractor shall keep the site free from injurious weeds as listed in the Weeds Act 1959 and amendments.

Topsoil Stripping

2.	Prior to stripping the topsoil, the Contractor shall remove all vegetation either physically or chemically and remove the arisings from the site.

3.	The depth of topsoil strip shall vary according to depths on site.  The Contractor shall agree with the Overseeing Organisation the depth of stripping before commencing stripping.

Topsoil

4.	Topsoil shall be from a source approved by the Overseeing Organisation.  It shall comply with BS3882:1965 - Recommendations and Classifications for Topsoil and be of a medium texture with a high proportion of loamy material.  It shall be free from subsoil, rubbish, roots or perennial weeds, and other materials injurious to plant growth.

5.	The topsoil shall contain less than 20% of dry weight stones which shall be less than 50mm in any one dimension.

Topsoil Stored on Site

6.	Topsoil stockpiled on site shall be stored in mounds of maximum height 2.5m (unless otherwise agreed by the Overseeing Organisation).  Mounds shall be constructed so that they will shed water and not puddle. No trafficking of placed topsoil shall be allowed.  Locations for stockpiles shall be agreed with the Overseeing Organisation before work commences.

7.	Topsoil stockpiles are to be kept weed-free for the duration of the Contract.  Herbicide shall be applied to perennial weeds and a period recommended by the manufacturer allowed to elapse before disturbing.

Herbicides

8.	Following topsoiling and prior to planting, Glyphosphate translocated herbicides shall be spread to the manufacturer’s instructions.

Soil Handling

9.	Subsoil and topsoil shall not be handled when the soil is waterlogged, when the moisture content of the soil exceeds its Plastic Limit 9 as determined in accordance with Test 3 BS1377: 1975), during precipitation or until at least 2 hours have passed after light rainfall (less than 2mm in any one hour) or at least 4 hours have passed after heavy rainfall (more than 2mm in any one hour), or as directed by the Overseeing Organisation.

Spreading Topsoil

10.	 Topsoil shall be spread over prepared subsoil in layers of 150mm and firmed.

Preparing Topsoil

11.	Break up any compacted topsoil to full depth.  Reduce top 100mm of all topsoil to a tilth suitable for blade grading (particle size 10mm and below).  Remove undesirable material brought to the surface including stones larger than 50mm in any one dimension, roots, tufts of grass and foreign matter.

12.	When topsoil is reasonably dry and workable, grade to smooth, flowing contours with falls for adequate drainage, removing all minor hollows and ridges.

Finished levels of topsoil after settlement (unless otherwise stated) to be:

a)	30mm above adjoining paving, kerbs or grass areas.

b)	Married in with adjoining soil areas.

13.	Final grade of topsoil areas to be to the approval of the Overseeing Organisation.

14.	Once topsoil has been spread and prepared, the area shall not be subsequently traversed by vehicles or plant or used for storage purposes.

APPENDIX 6/10:

GROUND ANCHORAGES, CRIB WALLING AND GABIONS

Gabions shall be 2x1x1m with fixed diaphragms at 1.0m centres.

Mattresses shall be 6x2x0.23m with fixed diaphragms at 1.0m centres.

Gabions and mattresses shall be manufactured from steel wire to BS1052 of 2.7mm diameter and galvanised to BS443 with bonded grey PVC coating of mean wall thickness of 0.5mm to give overall diameter 3.7mm.

The mesh fabric shall be formed from flexible double twist hexagonal woven wire mesh of 80mm opening size.

All mesh edges shall be reinforced with a galvanised PVC coated selvedge wire of 3.4mm core diameter, overall, 4.4mm diameter.

All joints and connections of gabion and mattress units are to be formed using continuous lacing wires and/or high tensile stainless steel “C “rings from 3.0mm wire, attached with a pneumatic assembly tool.

PVC coated lacing wire shall be 2.2mm core diameter and to be to the same specification as the body mesh.

Gabions and mattresses shall be filled with selected granular fill, class 6G to Table 6/1 series 600 Specification for Highway Works.

5.9 APPENDICIES 7/0

APPENDIX 7/1:

PERMITTED PAVEMENT OPTIONS SHEETS 1, 2 AND 3

1	Location: General Works/Repairs to Bituminous Surfaces potholes and depressions.

2	Grid for checking surface levels of 		Longitudinal dimension: 	10m

pavement courses (702.4)			Transverse dimension: 	2m

3	Surface regularity (702.7)			Category of road: 		A and B

4	Percentage Refusal Density required (Clause 953 1-27):			93%

| SURFACING | CLAUSE | MATERIAL | GRADE OF BINDER | THICKNESS (mm) | SPECIAL REQUIREMENTS |
| --- | --- | --- | --- | --- | --- |
| Surface course | 912 | AC 14 close surface course PSV55 | 100/150 Pen | Varies | BS EN 13108-1 detailed requirements of BSI PD 6691 Annex B Nominal aggregate size 6mm excluding limestone. Min air temp for laying -1ºC on a rising thermometer Minimum PSV:60 Maximum AAV:12 |
| Binder course | 906 | Dense macadam with 14mm or 20mm aggregate | 100/150 Pen | Varies | BS EN 13108-1 detailed requirements of BSI PD 6691 Annex B Nominal Aggregate size 14mm or 20mm |
| Base (Road base) | 903 | Dense base (road base) macadam | 100/150 Pen | Varies | BS EN 13108-1 detailed requirements of BSI PD 6691 Annex B Nominal Aggregate size 14mm or 20mm |
| Type unbound mixtures | 803 | Granular Type 1 |  | Varies | Minimum CBR 30% |
| Type unbound mixtures | 803 | Granular Type 2 |  | Varies | Public footpath usage |
| Total pavement thickness varies |  |  |  |  |  |
| PROVISIONAL SUB-GRADE STRENGTHENING |  |  |  |  |  |
| Reinforcement 950AR Geogrid as directed Capping 6F2 613 As directed |  |  |  |  |  |

APPENDIX 7/2:

EXCAVATION AND REINSTATEMENT

OF EXISTING SURFACES

1	Locations of trenches, pits, etc. which require excavation and reinstatement will be identified by the Overseeing Organisation in instructions to the Contractor.

2	All reinstatements shall be carried out in accordance with the New Road and Street Works Act 1991 Code of Practice (Second Edition 2002) "Specification for the reinstatement of openings in Highways" or any amendment or revision thereof.  Cross section diagrams and details of permitted materials for typical trench reinstatements are included in this Code of Practice as follows: -

a)	Appendix A3.1 - Type 1 flexible roads

b)	Appendix A3.2 - Type 2 flexible roads

c)	Appendix A3.3 - Type 3 flexible roads

d)	Appendix A3.4 - Type 4 flexible roads

e)	Appendix A6.1 - Types 3 and 4 - modular roads - bituminous base (road base)

f)	Appendix A6.2 - Types 3 and 4 - modular roads - composite base (road base)

g)	Appendix A6.3 - Types 3 and 4 - modular roads - granular base (road base)

h) 	Appendix A7.1 - Flexible footways, footpaths and cycle tracks

i)	Appendix A7.3 - Modular footways, footpaths and cycle tracks.

APPENDIX 7/4:

BITUMINOUS SPRAYS

| Location/Situation | Type of Binder | Binder Grade or Classification | Rate of spread (/m²) | Permitted Addition | Blinding Material required |
| --- | --- | --- | --- | --- | --- |
| Curing cement bound base/sub-base and for geo-textile use. | Bitumen Emulsion | K1-60, A1-55 | 1.0 - 1.5 | - | Yes |
| Tack coat over existing road | Bitumen Emulsion | K1-40, A1-40 | 0.4 - 0.5 | - | No |

The above binders to comply with the British Board of Agreement HAPAS Roads and Bridges Certificate Series 900 Clause 920 1-13.

APPENDIX 7/6:

BREAKING UP OR PERFORATING OF EXISTING PAVEMENT

1	Locations as indicated on individual Works Task.

2	Perforation to be at 1m centres down to formation level.

APPENDIX 7/7:

SLURRY SURFACING INCORPORATING MICROSURFACING

Sheet 1: Information to be provided by the compiler:

1.	Location as per Works Task by the Overseeing Organisation.

2.	Traffic count.

3.	Traffic speed, 85 percentile and site speed limit.

4.	Category of site.

5.	Description of existing surface (918.2 and 918.4 - Texture, existing defects, variability, etc..).

6.	Thickness of Slurry Surfacing (918.11).

7.	Guarantee period if not 2 years.

8.	Minimum PSV of coarse aggregate.

9.	Maximum AAV of coarse aggregate.

10.	Preparation and masking requirements.

11.	Definition of colour required.

12.	Surface finish required for footways (if not by transverse brushing).

13.	Minimum texture depth at end of guarantee period (minimum sand patch test 1mm for more than 50 cv/land/day roads with a speed limit of 50mph, no requirement may be specified for very lightly trafficked low-speed carriageways or footways).

14.	Maximum texture depth after 4 weeks of trafficking (only required where tyre-road noise emission is a problem, normally 3mm Sand Patch texture depth).

15.	Maximum percentage decrease in texture initially measured and at the end of the guarantee period.

16.	Class of area defects (% area affected) acceptable (918.33 - Table NG 9/2).

17.	Class of linear defects (m per 100m) acceptable (918.33 - Table NG 9/3).

18.	Class of transverse regularity (918.32 and Table NG 9/5 - e.g. Class 0 if only sealing and improvement to surface characteristics are required).

19.	Class of longitudinal regularity (918.32 and Table NG 9/7 - e.g. Class 0 if only sealing and improvement to surface characteristics are required).

20.	Special restrictions (918.17, 918.24, 918.28 and 918.29 - minimum road surface temperature at which working permitted is 4°c and maximum is 40°c).

Sheet 2: Information to be provided by the Contractor:

1.	A copy of BS EN ISO9002: 1994 certificate showing at least the name of the Company, the name of the certification body and the reference number and date of the certificate.  A copy of the relevant part of the company Quality Assurance (QA) document showing the appropriate scope (Slurry Surfacing and Sector Scheme) and limitations of the certification.  The Overseeing Organisation may wish to inspect all or any of the Company’s QA documentation as part of the vendor assessment system and may wish to satisfy himself on the nature of the QA systems of the Company’s material suppliers. (918.2) (Note to compiler: omit this requirement for contracts let exclusively in Northern Ireland.)

2.	Design Proposal for Slurry Surfacing for each location and target binder content with tolerances (918.2).

3.	Estimated Design Life of the Slurry Surfacing for each location (918.2).

4.	A copy of the TAIT Certificate within the Sector Scheme for Slurry Surfacing for the proposed system together with its supporting data.  In the event of no certificates being issued a statement of any previous applications on roads similar in site type to the Contract sites containing the same data as listed in NG Sample Appendix Sheet 3 (918.4).

5.	A method statement for each site or group of similar sites showing how it is proposed to carry out the works in conformance with the specification.  (Contractors will be expected to commit enough resources to carry out the proposed design; the type and age of the Slurry Surfacing Machine should be detailed.)

6.	Proposed source or sources of coarse aggregate together with statements of properties including target grading, PSV and AAV (918.6).

7.	Proposed source or sources of fine aggregate including target grading and other constituents together with statements of properties (918.6).

8.	Proposed binder together with data sheets and cohesivity data. (918.7 Note to compiler: a suitable form for the provision of data is given attached to this Appendix, other layouts are permitted but all the required data should be supplied.)

9.	Proposals for traffic control and aftercare for each site, and reaction times for: carrying out remedial measures; sweeping; and site visits with the Overseeing Organisation (918.15, 918.18, 918.29).

10.	Contingency plans in the event of any breakdown of plant or failure of the Slurry Surfacing (918.28).

11.	An ‘As Built Manual’ as specified (918.30).

12.	If available the following information should be provided to assist the Overseeing Organisation to assess the technical merits of the Design Proposal:

i)	 Test method for binder content.

ii)	 Test for thickness of Slurry Surfacing.

iii)	 Trafficability time, including method of test.

iv)	 Wheel tracking test results at 45ºc or 60ºc or other suitable measure of the ability of the proposed system to resist deformation and flow.

v)	 Water sensitivity test results from the test used by BBA/HAPAS thin surfacing Guidelines Document or from wet wheel tracking (whichever is available).

vi)	 Permeability test carried out on the system, if it is claimed that the process seals the existing surface together with the method of test.

vii)  Accelerated ageing test results in accordance with the appropriate BBA/HAPAS test.

viii) Bond test results using the BBA/HAPAS test on either a bituminous or a concrete substrate as appropriate to the site or Bond Coat binder BBA/HAPAS certificate.

ix)	 Shaking Abrasion test results if available.

x)	 Slurry surfacing mix cohesion.

xi)	 The results of any other tests or other data the Contractor considers would assist the Overseeing Organisation in assessing the technical merit of the Design Proposal.

Sheet 3: TAIT Certificate: Information to be provided by the Contractor:

The Contractor shall provide the TAIT Certificate containing at least the following information with his tender:

Company Name and Address

QA reference number and certifying body

TAIT reference number

Date of TAIT

Self-certified within the Sector Scheme for Slurry Surfacing

Proprietary Name

Description of Material

Design procedure or method

Material thickness (if applicable)

Texture depth at 1 year (as measured and as a percentage of the initial value)

Colour retention (if applicable)

Other optional claims as declared by the installer (e.g. Profile improvement, reduced tyre-road noise emission or RSI, ability to accommodate a variable substrate, skid resistance if greater than PSV and texture would indicate, etc.)

Expected life (Estimated Design Life)

Field of application for the material:

Traffic - maximum commercial vehicles per lane per day

Traffic - total traffic per lane per day

Traffic - speed limit

Degree of Site difficulty (see HD 36 for categories)

Constraints on application for the material:

Time of year

Temperature

Variability of existing surface hardness or type

Other as declared by the installer

Name and signature of company representative responsible for the TAIT.

APPENDIX 7/10:

WORKSHEET PRO FORMA FOR RESULTS OF TESTING

FOR CONSTITUENT MATERIALS IN RECYCLED AGGREGATE AND

RECYCLED CONCRETE AGGREGATE

| Sample reference |  |
| --- | --- |
| Date |  |
| Tested by |  |

| Mass of test portion, Mtotal, Duplicate 1 |  |
| --- | --- |
| Mass of test portion, Mtotal, Duplicate 2 |  |

| Category 1 | Mass Msubscript |  | Percentage Psubscript |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Duplicate 2 | Duplicate 1 | Duplicate 2 | Duplicate | Mean |
| Asphalt (Class A) |  |  |  |  |  |
| Masonry (Class B) |  |  |  |  |  |
| Concrete and concrete products (Class C) |  |  |  |  |  |
| Glass (Class G) |  |  |  |  |  |
| Lightweight particles (Class L) |  |  |  |  |  |
| Unbound aggregate (Class U) |  |  |  |  |  |
| Other particles (Class X) |  |  |  |  |  |
| Sum* |  |  |  |  |  |

*Sum is the total of MA + MB + MC + MG + ML + MU + MX.  If Sum is not within 1% of Mtotal, repeat the test.

5.10 APPENDICIES 10/0

APPENDIX 11/1:

KERBS, FOOTPATHS AND PAVED AREAS

1.0	KERBS

Lifting of kerbs to be in accordance with BS7263 Part 1 at locations as indicated on individual Works Task.

Kerb details to be as shown on drawing B30.

2.0	FOOTWAYS AND PAVED AREAS - TYPE A

| Item | Clause | Material | Grade of Binder | Thickness (mm) | Special Requirements |
| --- | --- | --- | --- | --- | --- |
| Surface Course | 909 | Dense macadam with 6mm aggregate “AC 6 dense surf” | 100/150 pen bitumen | 20 | BS EN13108-1 detailed requirements of BSI PD 6691 Annex B Nominal aggregate size 6mm excluding limestone. Min air temp for laying -1oC on a rising thermometer Minimum PSV: 60 Maximum AAV: 12 |  |
| Binder course | 906 | Dense macadam with 14mm or 20mm aggregate. | 100/150 pen | 40 or 60 | BS EN13108-1 detailed requirements of BSI PD 6691 Annex B Nominal Aggregate size 14mm or 20mm. |  |
| Type unbound mixtures | 803 | Granular Type 1 |  | 200 | Equivalent CBR 30% |  |

TOTAL THICKNESS: 		260mm or 280mm

FOOTWAYS AND PAVED AREAS - Type B

| Item | Clause | MATERIAL | GRADE OF BINDER | THICKNESS | SPECIAL REQUIREMENTS |
| --- | --- | --- | --- | --- | --- |
| Tactile Slab |  | Precast concrete |  | 65mm |  |
| Bedding layer |  | Mortar |  | 20mm |  |
| Binder course | 906 | Dense macadam with 20mm aggregate. | 100/150 pen | 150mm | BS EN13108-1 detailed requirements of BSI PD 6691 Annex B Nominal Aggregate size 14mm or 20mm. |
| Type unbound mixtures | 803 | Granular Type 1 |  | 150mm |  |

TOTAL THICKNESS:		385mm

FOOTWAYS AND PAVED AREAS - TYPE C

| Item | Clause | Material | Grade of Binder | Thickness | Special Requirements |
| --- | --- | --- | --- | --- | --- |
| Pavers or modular slabs |  | Precast concrete or clay | - | 65mm |  |
| Bedding Layer |  | Sand |  | 35mm |  |
| Infill Material |  | Kiln dried sand |  |  |  |
| Binder course |  | 20mm aggregate open textured | 100/150 pen | 150mm | BS EN13108-1 detailed requirements of BSI PD 6691 Annex B Nominal Aggregate size 14mm or 20mm. |
| Type unbound mixtures | 803 | Granular Type 1 |  | 150mm |  |

TOTAL THICKNESS:         400mm

FOOTWAYS AND PAVED AREAS - TYPE D

| Surfacing | Clause | Material | Grade of Binder | Thickness | Special Requirements |
| --- | --- | --- | --- | --- | --- |
| Stone paving or deep stone setts |  |  |  | 50mm or 150mm |  |
| Bedding layer and infill material |  | Class 1 semi-dry mortar bed |  | 20mm |  |
| Binder course |  | Heavy duty macadam 20mm aggregate | 100/150 pen | 150mm | BS EN13108-1 detailed requirements of BSI PD 6691 Annex B Nominal Aggregate size 14mm or 20mm. |
| Type unbound mixtures | 803 | Granular Type 1 |  | 100mm |  |

TOTAL THICKNESS:           320 or 420mm

Paving slabs and block pavers should be cut with a diamond saw in the Contractor’s compound.  Water used during the process should be managed in such a way as not to leave a coloured residue in the area on completion of the works.

Sand for bedding and jointing paving slabs and pavers should be silica sand comprising at least 97% silica and graded to the following:

BS 				PERCENTAGE BY MASS PASSING

SIEVE

SIZE			LAYING COARSE SAND	JOINT FILLING SAND

5mm				90-100				90-100

2.36mm			75-100				85-100

1.18mm			55- 90				75-100

600mm				35- 90				60- 79

300mm				35- 59				12- 40

150mm				8- 30				 0- 10

FLAGS

Shapes, dimensions/colours, textures and laying patterns to be as directed by the Overseeing Organisation.

PAVING BLOCKS

Shapes, dimensions, colours and laying patterns to be as directed by the Overseeing Organisation.

APPENDIX 12/1:

TRAFFIC SIGNS: GENERAL

1.  Sign Warranty

(a)  All signs supplied shall be WARRANTED in accordance with this Specification.

(b)  Upon receipt of this Specification and prior to supplying signs against it, the sign manufacturer shall forward a WARRANTY to the Overseeing Organisation signed jointly by the sign manufacturer and the supplier of the retroflected sheeting and non-retroflected film.

2.  Retroreflective Road Traffic Signs

(a)  Class 1 retroreflective road traffic signs shall be WARRANTED life not less than 10 years and fulfill the requirements.

(b)  Class 2 retroreflective road traffic signs shall be WARRANTED life not less than 7 years and fulfill the requirements.

3.  Non-Reflective Road Traffic Signs

Non-reflective road traffic signs shall have a WARRANTED life of not less than 7 years.

4.  Repair and Replacement

The sign manufacturer and the sign face material supplier jointly shall repair or replace and when necessary, dismantle and re-erect any WARRANTED sign which fails to provide the required performance.  Such repairs or replacements shall be completed within a reasonable period as agreed with the Overseeing Organisation.

5.  Sign Marking

Finished signs shall be marked on the reverse with the sign number as directed by the Overseeing Organisation and permanently marked with the name or code of the manufacturer and the month and year of manufacture.

6.  Sign face

All reflective signs shall be manufactured from Class 1 retroflected material to BS 873 Part 6 unless otherwise stated on the scheme drawings.

7.  Sign Bases

Details of traffic sign bases are shown on drawing relevant to each instruction.

8.  Traffic Signposts

(a)  The type and size of posts will be detailed on the relevant instruction.

(b)  Post should be tubular steel plain or large base type to BS EN 12899‑1 and be capable of passing the test described in BS EN 12899‑1.

9.  Materials and Construction

(a)  Tubular steel posts shall be shot blasted and hot dipped galvanised to comply with BS EN ISO 1461 a heavy-duty grade black bituminous solution shall be applied to the root of the post to a point 250mm above ground level, this protection to be applied to both surfaces of the root.

(b)  Large base posts shall be of a steel not less than 5mm thick and shall be cylindrical with an inside diameter of not less than 140mm. An internal baseboard of dimensions not less than 450mm x 95mm x 12mm, manufactured from a substantial non-hygroscopic material, shall be fixed in the housing and an earthing bolt shall be provided.  The opening to the housing shall be not less than 450mm x 95mm and shall be provided for the opening and shall be fitted with a tamper proof lock, the same pattern of lock being used throughout.  All locks shall be liberally treated with silicone grease.  Two keys shall be provided per 10 large base post supplied.  An access slot not less than 75mm x 150mm high for cabling purposes shall be provided approximately 150mm below ground level.

10. Installation of Signposts

The erection of signposts shall comply with the requirements of Chapter 13 of the Traffic Signs Manual.  The approximate sign locations are shown on the Scheme drawing.  The exact location to be determined by the Overseeing Organisation with the Contractor before commencement of the works.

(a)  All posts shall be embedded in concrete, see Clause 2602 and Standard Drawings SL/SD 4.1 to 5.1. It will be the responsibility of the Contractor in consultation with the Overseeing Organisation to ensure that the foundations for each sign assembly shall be adequate to withstand the loading conditions imposed on either face of the erected sign.

(b)  All excavations shall be dug by hand and the bottoms of all excavations for post sizes of 110mm diameter and over shall be inspected for each sign assembly shall be adequate to withstand the loading conditions imposed on either face of the erected sign.

(c)  50mm I.D. diameter PVC service ducting tubes should be incorporated in all large base foundations, the service ducting should terminate at the cable entry slots of the large base post.

11. Non‑Illuminated Bollards

All illuminated bollards within the Authority’s highway network have been replaced with

non‑illuminated units. No new illuminated bollards shall be installed unless expressly instructed by

the Authority. Prior to undertaking any repair or replacement works, the Contractor shall ensure that

all associated electrical supplies and bollard bases are fully isolated and made safe in accordance

with BS 7671 and the Electricity at Work Regulations 1989.

Any redundant electrical equipment discovered during works shall be reported to the Authority and

removed or made permanently safe as instructed.

Bollards bases are to be secured by rag bolts embedded in a 600 x 230mm concrete. Were bollards

are to be erected on refuges the final top 75mm above the bollard foundations should be finished in

similar materials to that of the refuges and sloping slightly away from the bollard.

12. Illuminated Sign Lighting Units

(a)  All external lighting units shall comply with BS EN 12899‑1and Chapter 11 of the “Traffic Signs Manual” and with the Specification. All lanterns shall be procured by the Contractor on behalf of the Overseeing Organisation.

(b)  All units shall in addition be: -

(i)    Constructed with die cast aluminium/extruded aluminium bodies (for overhead lighting); aluminium/steel bodies (for upward independent lighting), finished in traffic grey, steel bodies being rust proofed prior to finishing.

(ii)   Fitted with vandal resistant light panels.

(iii)   Incorporate outreach arms, post top sockets or brackets (i.e. galvanised, zinc coated, etc. and painted to suit post or column finish and provided with anti-rotation device.  Where required arms being capable of extension adjustment to allow for centralizing units parallel with sign faces from offset posts.

(iv)  Integral lantern, outreach and mounting manufactured from cast aluminium are acceptable.

(v)   Fitted with stainless steel screws, nuts and washers.

(vi)   Fitted complete with LED units.

(vii)  Fitted with suitable dusk to dawn miniature electronic photoelectric cell control (low light cell).

(viii) The units shall be inconspicuously marked with the name/trademark of the manufacturer, BS EN 12899‑1, month and year of manufacture.

(ix)  Units fixed to lighting columns shall be separately fused.

APPENDIX 12/5:

TRAFFIC SIGNS - TRAFFIC SIGNALS

1.  Permanent Traffic Signals

A5 Crossroads, Valley

A5 Holyhead Road, Valley

2.  Temporary Traffic Signals

1.  Any Temporary traffic signals used to control alternative one way working, shall comply with TA 12/81, TS13/81, TA14/81, TD 7/80, TD 21/85, TD 24/86

2.  Three-way temporary traffic signals require prior authorisation of Isle of Anglesey County Council Maintenance Section.  The Contractor to provide calculations and prepared layouts 7 days in advance of erection.  For works on the County Road network a 4-week notice period is required.

3.  Zebra and Controlled Crossings

AC60 - Crossroad, Valley

AC61 - A5 London Road, Valley

AC62 - A5 London Road, Holyhead (Near Cecil Street)

AC63 - A5 Holyhead Road, Llanfairpwll

AC64 - A5154 Victoria Road, Holyhead (Near Holborn Road)

AC65 - A5154 Victoria Road/Cenotaph, Holyhead

AC66 - B5109 The Green, Beaumaris

AC67 - A5025 Inner Relief Road, Amlwch

AC68 - A5025 The Square, Benllech

AC69 - A5025 Breeze Hill, Benllech

AC70 - A5025 Cloth Square, Pentraeth

AC71 - B5109 High Street, Llangefni

AC72 - B5420 Ysgol David Hughes, Menai Bridge

AC73 - B5109 Penrallt, Llangefni

AC74 - A5 London Road, Holyhead (Near Lôn Deg)

AC75 - B4545 Kingsland Road, Holyhead (Near Asda)

AC76 - A5 Holyhead Road, Gaerwen

AC77 - B5111 Church Street, Llangefni

AC78 - B5110 Smithfield, Llangefni

AC79 - CLASS 3 Ffordd Y Coleg, Llangefni

AC80 - A545 High Street, Menai Bridge

AC81 - A5153 Penrhos Link, Holyhead

AC82 - B5109 Cildwrn Road, Llangefni

AC83 - B5109 Lôn Talwrn, Llangefni

AC84 - B4545 Kingsland Hill, Holyhead

AC85 - A5025 Co-Op Benllech, Benllech

AC86 - B5109 Ysgol Corn Hir School, Llangefni

AC87 - A4080 High Street, Malltraeth

APPENDIX 12/8:

TRAFFIC SIGNS BELISHA BEACONS

1.  General

All traffic signs shall comply with BS EN 12899‑1 (including UK National Annex) and the Traffic Signs

Regulations and General Directions (TSRGD).

All sign faces and assemblies shall be UKCA marked.

Electrical works shall comply with BS 7671 (IET Wiring Regulations, current edition).

2.  Posts

Wide base with 76mm O.D. upper section galvanised to BS EN ISO 1461 post with standard locking doors 2.6m minimum overall mounting height planted to 450mm minimum depth.

(a)  Narrow section of post painted black with 300mm alternate white and black bands commencing with white above the wide base section accommodating any variance from exact 300mm modules.  Minimum of 3 No. stripes.  White bands to be Class 1 Reflective.

3.  Globes

All lanterns shall be procured by the Contractor on behalf of the Overseeing Organisation.

Yellow plastic 300mm globe one- or two-piece construction with anti-vandal retaining collar.

4.  Supply and Lamps (Beacon)

(a)  LED lamps

(b)  LED units BS 8442:2022 + A1:2023

APPENDIX 12/9:

TRAFFIC SIGNS: ON SITE EQUIPMENT AND MANUFACTURER DETAILS

Type 1 Road Sign Lantern Unit – Manufacturer Simmon Signs Ltd, Mallatite, Portland, Glasdon.

Bracket for Illuminated Road Signs

Type 					Manufacturer

Post Top				Simmon Signs

Column Fixing				Signature Signs

Single and Double Arm		Haldo

76mm, 89mm, 114mm		Truesigns

APPENDIX 13/1:

INFORMATION TO BE PROVIDED BY THE OVERSEEING ORGANISATION

WHEN SPECIFYING LIGHTING COLUMN

A complete road lighting installation is required to have a life expectancy of at least 30 years on all items of equipment.  The manufacturers and suppliers are required to offer their products on this basis and requested to provide details of maintenance needed to keep them in good condition.  The Contractor shall submit the information with his tender and in addition to Appendix 13/2.

1.  Lighting Columns

(i)   Lighting columns and brackets shall be supplied and installed in compliance with BS EN 40‑3‑1:2013 and incorporate CE marking to EN40-5.

(ii)  Lighting columns and brackets shall be tapered or tubular manufactured from steel to BS5649 Part 3.  Any baseplates shall be manufactured from steel to BS5649 Part 3.

(iii)  Planted lighting columns and brackets in manufactured from steel to BSEN40 Part 3 shall be protected from corrosion by hot dip galvanising in accordance with requirements of BS EN ISO 1461.  The internal and external root section shall receive further treatment of:

(a)  Degrease and the application of ‘T’ wash.

(b)  Application coat of item 150 Pitch Epoxy (2 packs) AS, mdft, 100 microns, black (as D.O.T/W.A. G approvals) applied to a point 250mm above ground level or similar.

(iv) Location marks for inspection and maintenance purposes are to be provided on the columns after erection as follows: -

(a) Columns mounted in the central reserve are to have 75mm high numbering on both sides of the columns; each number is to face the oncoming traffic at an angle of approximately 45 degrees.

(b) Verge mounted columns are to have 50mm high numbering on one side of the columns, facing the oncoming traffic which will be class 2 reflective.  The numbering shall be in accordance with the column reference numbers shown on the drawings.  The numbers colour reference shall be as specified by Overseeing Organisation.  The numbers shall be fixed 3.0 metres above adjacent ground level.

(v)  Each column shall be earthed with an 8mm (minimum) dia. earthing terminal with two plain washers and nuts, all of which shall be manufactured from brass.  The earthing stud shall be so positioned as to be readily accessible through the door opening.

(vi)  All columns will have a permanent R.E.C. supply unless stated otherwise by the Overseeing Organisation.

(vii) Column and bracket Data Sheets 1 and 2 completed in accordance with the instructions in Appendix 13/3 should be completed and submitted to the Overseeing Organisation before erection of the column.

(viii) Where lighting columns are to be in the vicinity of overhead power lines the Contractor shall ensure that the appropriate Electricity Company are informed and are satisfied with the clearances provided.  The Contractor will obtain this Authority at least two weeks in advance of commencement of works.

The Contractor will also notify the Overseeing Organisation before erecting the columns.

(ix) Only columns and brackets approved by the Overseeing Organisation shall be used.

2.  Column Doors

(i)   Columns are to be provided with weatherproof single doors, each with a tamper proof locking device.

(ii)  Door openings, bracket arms and columns shall be free from irregularities or burrs.

(iii)  Columns mounted on bridges or viaducts behind parapets shall have the bottom of the door opening 1.10m above the top of the parapet plinth.

(iv)  Stainless steel fixings or chains shall be used to fix and lock those doors required to be captive.

(v)  Except for doors which are required to be captive, all doors are to be interchangeable between similar types of columns without adjustments.

LIST OF STANDARD LIGHTING COLUMNS

TYPE 1

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.3 metre projection

TYPE 2

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.5 metre projection

TYPE 3

1 	Column    	5 metre mounting height galvanised (BS EN ISO 461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification.

3 	Brackets    	1.0 metre projection

TYPE 4

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	Post top.

TYPE 5

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Separate   	0.3 metre projection

TYPE 6

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.5 metre projection

TYPE 7

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Integral   	1.0 metre projection

TYPE 8

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Separate   	Post top.

TYPE 9

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Separate   	0.5 metre projection

TYPE 10

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Separate   	1.0 metre projection

TYPE 11

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets     	1.5 metre projection

TYPE 12

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets     	2.5 metre projection

TYPE 13

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets     	Post top.

TYPE 14

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets     	0.5 metre projection

TYPE 15

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth 	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 16

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 17

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 18

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top.

TYPE 19

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.5 metre projection

TYPE 20

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	1.0 metre projection

TYPE 21

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 22

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	2.5 metre projection

TYPE 23

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	Post top

TYPE 24

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.5 metre projection

TYPE 25

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	1.0 metre projection

TYPE 26

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets	Post top

TYPE 27

1 	Column   	6 metre mounting height galvanised (BS EN ISO 1461) Tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	0.5 metre projection

TYPE 28

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	1.0 metre projection

TYPE 29

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 30

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 31

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 32

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 33

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 34

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 35

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 36

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 37

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 38

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 39

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 40

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 41

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 42

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets	1.5 metre projection

TYPE 43

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 44

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 45

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets   	0.5 metre projection

TYPE 46

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 47

1 	Column    	5 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 48

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets	0.5 metre projection

TYPE 49

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 50

1 	Column    	6 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 51

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 52

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 53

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 54

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 55

1 	Column    	8 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 56

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth 	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 57

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 58

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 59

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 60

1 	Column    	10 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 61

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 62

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 63

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 64

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 65

1 	Column    	12 metre mounting height galvanised (BS EN ISO 1461) tubular steel

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 66

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.3 metre projection

TYPE 67

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.5 metre projection

TYPE 68

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification.

3 	Brackets    	1.0 metre projection

TYPE 69

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	Post top.

TYPE 70

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Separate   	0.3 metre projection

TYPE 71

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.5 metre projection

TYPE 72

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Integral   	1.0 metre projection

TYPE 73

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Separate   	Post top.

TYPE 74

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Separate   	0.5 metre projection

TYPE 75

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets Separate   	1.0 metre projection

TYPE 76

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets     	1.5 metre projection

TYPE 77

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets     	2.5 metre projection

TYPE 78

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets     	Post top.

TYPE 79

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets     	0.5 metre projection

TYPE 80

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth 	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 81

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 82

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 83

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top.

TYPE 84

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.5 metre projection

TYPE 85

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	1.0 metre projection

TYPE 86

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 87

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	2.5 metre projection

TYPE 88

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	Post top

TYPE 89

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	0.5 metre projection

TYPE 90

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets integral   	1.0 metre projection

TYPE 91

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets	Post top

TYPE 92

1 	Column   	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	0.5 metre projection

TYPE 93

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets separate   	1.0 metre projection

TYPE 94

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 95

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 96

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 97

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 98

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 99

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 100

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 101

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 102

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 103

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 104

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 105

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 106

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 107

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets	1.5 metre projection

TYPE 108

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium			base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 109

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

base-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 110

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets   	0.5 metre projection

TYPE 111

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 112

1 	Column    	5 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 113

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets	0.5 metre projection

TYPE 114

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 115

1 	Column    	6 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 116

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 117

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 118

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 119

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 120

1 	Column    	8 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 121

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth 	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 122

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 123

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 124

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 125

1 	Column    	10 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

TYPE 126

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	0.5 metre projection

TYPE 127

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.0 metre projection

TYPE 128

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	1.5 metre projection

TYPE 129

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	2.5 metre projection

TYPE 130

1 	Column    	12 metre mounting height anodized (BS EN ISO 7599) tubular aluminium

mid-hinged

2 	Planting Depth   	Refer to manufacturer’s specification

3 	Brackets    	Post top

METAL BRACKET REPLACEMENT FOR STEEL & ALUMINIUM COLUMN

Steel Brackets

TYPE 131

Supply and fit 0.3 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) to fit up to 6 mt. mounting height column.

TYPE 132

Supply and fit 0.5 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) to fit up to 6 mt. mounting height column.

TYPE 133

Supply and fit 1.0 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) to fit up to 6 mt. mounting height column.

TYPE 134

Supply and fit 0.5 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) to fit 8 mt. mounting height column or above.

TYPE 135

Supply and fit 1.0 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) to fit 8 mt. mounting height column or above.

TYPE 136

Supply and fit 1.5 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) to fit 8 mt. mounting height column or above.

TYPE 137

Supply and fit 2.5 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) to fit 8 mt. mounting height column or above.

TYPE 138

Supply and fit pole 0.5 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) up to 8 mt mounting height.

TYPE 139

Supply and fit pole 1.0 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) up to 8 mt mounting height.

TYPE 140

Wall 0.5 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) up to 8 mt mounting height.

TYPE 141

Wall corner 0.5 mt galvanised steel outreach bracket (BS EN ISO 1461) (approval of Overseeing Organisation) up to 8 mt mounting height.

TYPE 142

Supply and fit 1.0 mt extension post galvanised steel (BS EN ISO 1461) (approval of Overseeing Organisation) to fit 5 metre columns to obtain 6 metre mounting height.

Aluminium Brackets

TYPE 143

Supply and fit 0.3 mt anodized aluminium outreach bracket (BS EN ISO 7599) (approval of Overseeing Organisation) to fit up to 6 mt. mounting height column.

TYPE 144

Supply and fit 0.5 mt anodized aluminium outreach bracket (BS EN ISO 7599) (approval of Overseeing Organisation) to fit up to 6 mt. mounting height column.

TYPE 145

Supply and fit 1.0 mt anodized aluminium outreach bracket (BS EN ISO 7599) (approval of Overseeing Organisation) to fit up to 6 mt. mounting height column.

TYPE 146

Supply and fit 0.5 mt anodized aluminium outreach bracket (BS EN ISO 7599) (approval of Overseeing Organisation) to fit 8 mt. mounting height column or above.

TYPE 147

Supply and fit 1.0 mt anodized aluminium outreach bracket (BS EN ISO 7599) (approval of Overseeing Organisation) to fit 8 mt. mounting height column or above.

TYPE 148

Supply and fit 1.5 mt anodized aluminium outreach bracket (BS EN ISO 7599) (approval of Overseeing Organisation) to fit 8 mt. mounting height column or above.

TYPE 149

Supply and fit 2.5 mt anodized aluminium outreach bracket (BS EN ISO 7599) (approval of Overseeing Organisation) to fit 8 mt. mounting height column or above.

APPENDIX 13/2:

(SPECIFICATION FOR HIGHWAY WORKS)

TYPICAL COLUMN AND BRACKET DATA – SHEET 1

See Volume 3 PLANS

APPENDIX 13/3:

## INSTRUCTIONS FOR COMPLETION OF COLUMN AND BRACKET DATA (ELECTRONIC DATA FORMAT)

### General

- Where information is not required, the appropriate data field shall be populated with a dash “–” or marked as “Not Applicable (N/A)” within the electronic system.

- All data shall be completed, stored, and transmitted using an approved electronic data transfer (EDT) system in lieu of paper Data Sheets.

- Where previously submitted data is amended, a new revision shall be issued within the EDT system.

- Revision identifiers shall be consecutive alphabetical letters, commencing with “A”, and automatically tracked within the electronic record.

- The revision date shall be recorded within the EDT system and shall correspond with the date of the Contractor’s electronic approval or digital signature.

- The column or bracket material shall be recorded as steel, aluminium, or any other material approved by the Overseeing Organisation.

- Material design strengths shall comply with BS EN 40 (relevant parts) and shall not be less than the specified design values. Where multiple materials are used, all shall be recorded within the electronic data record.

- All required data fields shall be completed and validated within the EDT system prior to certification by the Contractor.

Column Data

- Column nominal height and classification shall be selected from BS EN 40 (all relevant parts) and associated UK guidance in PD 6547 and recorded within the system.

- The number of door openings shall match the manufacturer’s approved drawings and be recorded accordingly.

- The cross-sectional dimensions of the base compartment shall be provided in the form of an uploaded diagram, CAD file, or digital sketch within the EDT system.

- Acceptable positions of bracket arms relative to the door position shall be clearly indicated using a diagram or selectable options within the system. Where all positions are acceptable, the “ANY” option shall be selected.

- Where concrete surround is required for planted bases in accordance with Sub-Clauses 1305.3 and 1305.4, the minimum diameter shall be specified.

- For flange-based columns, all design forces and moments used in the design of foundations, anchorages, and fixings shall be recorded in the electronic data fields.

- The corrosion protection system applied to the column at the time of manufacture shall be specified. Where sacrificial thickness is included, the extent shall be recorded.

- Details of signs and attachments shall be provided, including:

- Surface area

- Eccentricity from the column centreline

- Height above ground to the centroid of the sign area

Bracket Data

- Luminaire and bracket design parameters recorded within the EDT system shall reflect the most adverse loading condition applicable to all compatible columns, including:

- Lever arm distances

- Weight

- Maximum windage area

Note:
Luminaire lever arms shall be defined as the horizontal distance from the centre of gravity of the lantern and, where applicable, the centroid of the windage area to the end of the bracket connection.

### Additional Requirements for Electronic Data Systems

- All submitted data shall be compatible with the Overseeing Organisation’s digital asset management or inventory systems.

- Electronic records shall be stored in a structured and searchable format, with traceability for all revisions and approvals.

- Digital drawings and supporting documents shall be submitted in approved formats (e.g., PDF, DWG, or equivalent).

- The Contractor shall ensure data accuracy, completeness, and compliance prior to electronic submission and certification.

APPENDIX 13/7:

ON SITE ROAD LIGHTING UNITS AND MANUFACTURER DETAILS

Within existing installations every effort will be made to replace like for like.  Where identical equipment is no longer available for replacements agreement must be reached with the Overseeing Organisation as to the appropriate substitution.  The Overseeing Organisation reserves the right to instruct the Contractor not to replace like with like.  Replacement of materials within existing electrical equipment unless otherwise indicated by the Overseeing Organisation.  Replacement materials shall be like existing, should similar replacement materials not be available, equivalent "like for like" materials shall be used as approved by the Overseeing Organisation.

LIST OF TYPICAL ON-SITE ROAD LIGHTING UNITS AND MANUFACTURER DETAILS:

| TYPE | MANUFACTURE |
| --- | --- |
| 5-12 mt Tubular Steel | BRITISH STEEL STAINTON FABRIKAT CONCRETE UTILITIES MALATITE BEI |
| 5-12 mt Tubular Aluminium | ALC |
| WALL POLE AND SLEEVE BRACKETS |  |
| 0.5 mt outreach | PUDSEY DIAMOND LUCY A.C. FORD |
| 1.0 mt outreach | PUDSEY DIAMOND LUCY A.C. FORD |
| 1.5 mt outreach | PUDSEY DIAMOND LUCY A.C. FORD |

APPENDIX 14/1:

SITE RECORDS

1.  As-constructed (as-built) records shall be produced by the Contractor for each instruction using drawings or digital templates provided by the Overseeing Organisation.
These shall comply with the requirements of Clause 1402 and shall include full details of the electrical installation for lighting columns, illuminated traffic signs, and associated equipment.
All records shall be submitted in approved electronic format and incorporated into the Overseeing Organisation’s asset management system.

2.  Record information shall be provided in approved electronic formats, which may include:

- CAD drawings (e.g. DWG/DXF)

- PDF drawings

- Structured electronic data files (e.g. CSV, XML, or equivalent)
Physical media (e.g. prints, transparencies, CDs) shall only be provided where specifically required by the Overseeing Organisation.

3.  The Contractor shall supply all relevant test and commissioning certificates, including electrical and structural testing where applicable.
These certificates shall:

- Be cross-referenced to the corresponding asset(s)

- Clearly identify the installed equipment

- Be submitted in electronic format and linked within the Overseeing Organisation’s approved asset records system

4.  Where applicable, the Contractor shall supply Operation and Maintenance (O&M) manuals to support the site records.
These manuals shall:

- Be provided in electronic format (PDF or equivalent)

- Be indexed and associated with the relevant asset records

- Include manufacturer’s instructions, maintenance requirements, and replacement procedures

APPENDIX 14/2:

LOCATION OF LIGHTING UNITS AND FEEDER PILLARS

The position and description of lighting units illuminated road signs, and feeder pillars shall be clearly identified on the drawings or digital records relevant to each instruction.

APPENDIX 14/3:

TEMPORARY LIGHTING

1. 	Existing road lighting units and illuminated sign units shall be maintained until the new installations have been commissioned as detailed in 4 (i), (ii) and (iii) below except where existing units are to be altered or moved as part of the CONTRACT.

2. 	Where existing plant are to be moved or altered as part of the Contract temporary lighting shall be provided where there is a delay of more than 24 hours between the removal or disconnection and re-erection and re-connection of the said plant.

3. 	Temporary lighting to Clause 1405 and to a similar standard as any existing lighting may be provided in lieu of maintenance of the existing installation.

4. 	Temporary lighting shall not be removed or switched off until:

(i)  	the permanent installation is in full operation;

(ii) 	an inspection has been carried out on the operation of the permanent installation not less than 24 hours or more than 7 days after commissioning.

(iii) 	any adjustments, remedial work or replacement found necessary have been carried out;

(iv) 	the system re-inspected as (ii) above.

APPENDIX 14/4:

ELECTRICAL EQUIPMENT FOR ROAD LIGHTING

A complete road lighting installation, excluding replaceable light sources and protective devices where applicable, shall be designed to achieve a service life of not less than 25 years under normal operating conditions.

Manufacturers and suppliers shall ensure that equipment is suitable for this design life and shall provide details of:

- Expected service life

- Inspection and maintenance requirements

- Recommended replacement intervals for key components

The Contractor shall provide full details of all equipment proposed for use within the Works, including supporting technical data, as part of the tender submission.

(i) 	Luminaires

The luminaires shall be compatible with the column and brackets offered in Appendix 13.2 and the information shall include the lamp type wattage and luminaire circuit wattage.  Generally, luminaires shall have a horizontal design attitude and be of moderate threshold increment type with an I.P. rating of at least I.P. 66.  The luminaires shall be fitted with integral control gear and nema sockets for P.E.C. units unless otherwise specified.  All luminaires mounted up to 6mt shall be provided with u.v. stabilised anti vandal bowl.  All electrical wiring subject to heat shall be protected by heat insulating sleeving internal wiring from fused cut outs to the control gear and luminaries shall be single or multi core copper conductor cables 600/1000-volt grade P.V.C. insulated and sheathed to comply with BS6004.

Each luminaire shall be separately fused and shall have double pole isolation.

Luminaires

**

| Manufacturer | Cat. No. | Glare Control Class (TI) | IP Rating | PECU Socket | (Design Table Ref No.) * | (Isoluminance Template Ref. No.) * |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | IP 65 | NEMA |  |  |

*   If required to ensure the assembly of Road Lighting units conforms to the performance requirements of BS EN 13201-2:2015

.

(ii) Ancillary Equipment

| Clause | Item | Manufacturer | Catalogue Ref | Overseeing Organisation Requirements |
| --- | --- | --- | --- | --- |
| 1409 | Photo Electric cell |  |  | 1:0.5 negative differential electronic units. 20 lux switches on |
| 1416 | Isolators Street Lighting cut outs, fuse links |  |  | 60 amp Double pole 25mm² conductor size 25amp rating double pole BS7654 2 x 25mm² conductor size BS88 HBC type BS 88-3 / BS EN 60269 |
| 1419 | Wiring |  |  | PVC or XLPE insulated and sheathed single, multicore cable to BS6004 grade 300/500 volts |

Where a lighting column is required to supply an illuminated traffic sign, the cut out in the column base shall have a separate way with a double pole isolating switch.

** Luminaire/Control gear suppliers shall provide independent test certificate stating the load consumption and UMSUG code for their equipment.

(iii) Feeder Pillars (Clause 1418)

Feeder Pillars shall be suitable for the layout shown on Drawing No. SL/SD2 and conform with the requirements shown on the drawing and as below.

Feeder Pillar shall be of the planted type with a shell constructed of 3mm minimum sheet metal galvanised to BSEN ISO 1461 and shall include a full-size back board of varnished marine plywood at least 15mm thick.

Feeder pillars with hinge doors shall have a proprietary method of ‘locking’ the doors in the open position.  It shall not be possible to either accidentally close the doors or for the wind to blow them closed once ‘locked’ in the open position.

Feeder Pillar dimensions shall be suitable for the individual installation requirements Clause 1418.2 with typical dimensions of height 800mm width 250mm and depth 150mm.

The feeder pillar is to be earthed to the main earthing point of the incoming supply and at least 1 No. set of keys (tri-head) to be provided to the Overseeing Organisation.

Feeder pillars shall be provided with screens of insulating materials to shield all live parts.

A diagram identifying the circuits shall be fixed to the inside of each pillar door and protected by a clear weatherproof cover.  The diagram shall show the circuits, columns with their respective identification numbers, signs and cable joints supplied from the pillar, all in schematic form, they should also indicate the location of ducted road crossings.

| Feeder Pillar | Manufacturer | Catalogue Ref |
| --- | --- | --- |
|  | Lucy |  |
|  | Pudsey Diamond |  |

The pillar door shall have a notice as detailed below, attached by means of stainless steel or other approved nonferrous fixing, adjacent to the warning sign indicated in Clause 1418.8.

Eiddo Cyngor Sir Ynys Môn 		Property of Isle of Anglesey County Council

Mewn argyfwng cysylltwch a: - 	In case of emergency please contact: -

(01248) 723062    			(01248) 723062

It shall be manufactured from 3mm (minimum) traffalyte or similar material.  Lettering shall be 6mm red on white background.

When feeder pillars are erected within grassed areas, to prevent vegetation growth, 600mm x 600mm concrete paving slabs or similar shall be laid in front of the door opening with minimum 150mm concrete infill to the pillar sides.  For surface water drainage the area is to be sloped away from the pillar.

After completion of feeder pillar underground cable installation, the base area of the pillar shall be filled to 25mm below the door with rounded coarse aggregates confirming with Table 4 of BS882 graded aggregate 14mm to 5mm and sealed using a cold pour resin to prevent ingress of moisture or gasses from below.

(iv) Cables and cable joints (Clause 1421 and 1422).

Cables trenches shall be excavated to the lines relating to individual works instruction and shall be in accordance with Clause 602.  Depth of excavation shall be such that cables/ducts laid under verges, footways and paved areas shall have a minimum 450mm cover and under carriageway 750mm cover.

The size and types of cables and joints are shown on the drawings relevant to each instruction.  Armoured cables shall be terminated onto a non-ferrous gland plate using compression glands and shrouds.  The gland plate shall form an integral part of a totally enclosed insulated double pole cut out unit.

For single phase 3 core cables, the phase conductor insulation shall be coloured brown, the neutral-coloured blue and the CPC coloured green/yellow and marked ‘STREET LIGHTING CABLE’ on the outer sheath.

| Cable | Manufacturer | Reference/Cat No. |
| --- | --- | --- |
| PVC or XPLE 600/1000v BS 5467 | BASEC Reg. |  |

| Joints | Manufacturer | Reference/Cat No. |
| --- | --- | --- |
| Heat Shrink |  |  |
| Cast Resin |  |  |

Cable ducts and Chambers

Cable ducts shall be impervious to water, impact resistant, capable of being laid at temperatures down to -10 degree C and be sufficiently flexible to follow undulations in the trench bottom.  All lengths shall be jointed or sleeved to give a continuous smooth bore.

| Ducts | Manufacturer | Reference/Cat No. |
| --- | --- | --- |
| 100mm ID |  |  |
| 50mm ID |  |  |

| Chambers | Manufacturer | Reference/Cat No. |
| --- | --- | --- |
| Moulded high-density polyethylene |  |  |

Erect Client lantern from store

Lantern installed at 6m height

Lantern installed at 8m height

Lantern installed at 10m height

Lantern installed at 12m height

Feeder Pillars

TYPE 78

1 	Feeder Pillar 	Galvanised (BS EN ISO 1461)

2 	Planting Depth 	Refer to Manufacturers specification

3 	Base Board 	Non-Hygroscopic

4 	Fixings 	Screws to be rust resistant

5 	Dimensions 	915mm Height

1120mm Width

300mm Depth

TYPE 79

1 	Feeder Pillar 	Galvanised (BS EN ISO 1461)

2 	Planting Depth 	Refer to Manufacturers specification

3 	Base Board 	Non-Hygroscopic

4 	Fixings 	Screws to be rust resistant

5 	Dimensions 	812mm Height

450mm Width

150mm Depth

TYPE  80

1 	Feeder Pillar 	Galvanised (BS EN ISO 1461)

2 	Planting Depth 	Refer to Manufacturers specification

3 	Base Board 	Non-Hygroscopic

4 	Fixings 	Screws to be rust resistant

5 	Dimensions 	812mm Height

250mm Width

150mm Depth

TYPE  81

1 	Feeder Pillar 	Galvanised (BS EN ISO 1461)

2 	Planting Depth 	Refer to Manufacturers specification

3 	Base Board 	Non-Hygroscopic

4 	Fixings 	Screws to be rust resistant

5 	Dimensions 	700mm Height

150mm Width

100mm Depth

APPENDIX 14/5:

ALL CONTROL GEAR FOR TRAFFIC SIGN ILLUMINATION SHALL BE HOUSED

IN THE LUMINAIRE ELECTRICAL EQUIPMENT FOR TRAFFIC SIGNS

| ITEM | MANUFACTURER | REFERENCE/CAT NO. |
| --- | --- | --- |
| Belisha Beacons Gallery and Globes | Portland, Zebrite or similar approved |  |
| Photo Electric Cells | Lucy Zodion, Westire or similar approved |  |

APPENDIX 14/6:

EARTHING

The whole of the installation shall be earthed in accordance with Clause 1420 and the requirements of BS 7671 and BS 7430 to the satisfaction of the Regional Electricity Company.  A P.V.C. covered green/yellow earth wire shall be connected to the earth block for the R.E.C. to connect at service positions.

A main protective bonding conductor shall have a cross-sectional area not less than half the cross-sectional area required for the earthing conductor of the installation and not less than 6mm² (BS 7671, Section 544.1.1).  Where PME conditions apply the earthing conductor of a street electrical fixture shall have a minimum copper equivalent cross-sectional area not less than that of the supply neutral conductor at that point or not less than 6mm², whichever is the smaller (BS 7671, Section 559.10.3.4).

A separate dedicated earth block shall be supplied and fitted adjacent to the service cut out to terminate the earth conductor and be connected to the R.E.C. earth point.  A sound mechanical and electrical connection must be made between each continuity conductor and each item of exposed metal; all earth connections shall be lugged with the appropriate size and used with the correct conductor.  Metal doors shall be bonded using flexible bonding conductors.  They shall be mechanically sound, electrically continuous and protected where necessary against corrosion.  Where substantial copper or other non-ferrous clamping terminals are not provided with the equipment it shall be the responsibility of the Contractor to supply and fit them.

Earthing system components shall comply with BS EN 62305, the rods shall be electrolytic copper molecularly bonded onto a steel core to a thickness of minimum 0.25mm and the clamps shall be cast gun metal with phosphor bronze bolt.  The terminal point shall be protected by a purpose made concrete or approved inspection pit with removable cover to gain access to connections and the conductor from the earth pit to the equipment protected from mechanical impact.

APPENDIX 14/7:

INSPECTION COMMISSIONING AND TESTING

1. General

All inspection, testing and commissioning shall be carried out in accordance with:

- BS 7671 (Requirements for Electrical Installations – IET Wiring Regulations)

- Electricity at Work Regulations

- Relevant Health and Safety legislation

Upon completion of the installation, and prior to energisation, all systems shall be inspected and tested to verify compliance with the above requirements.

Before testing:

- Circuits and equipment shall be identified

- The installation shall be safely isolated

- Sensitive equipment liable to damage (e.g. electronic devices, control gear, LED drivers) shall be disconnected

All test procedures shall ensure that:

- No danger arises to personnel or property

- No damage occurs to the installation, even in the presence of a fault

2. Alterations and Repairs

Where alterations or repairs have been undertaken, it shall be verified that:

- The safety of the installation is not impaired

- The work complies with BS 7671

- Existing protective measures remain effective

3. Periodic Inspection and Testing

Periodic inspection and testing shall be carried out at intervals not exceeding 6 years, unless otherwise directed by the Overseeing Organisation.

The frequency of inspection may be increased where:

- Environmental conditions are severe

- Equipment is subject to deterioration

- There is a higher risk to persons

4. Inspection and Testing Requirements

(a) Initial Verification (New Installations)

1. Tests with the supply disconnected

- Continuity of protective conductors, including:

  - Earthing conductor

  - Main bonding conductors

  - Supplementary bonding

- Continuity of ring final circuit conductors (where applicable)

- Insulation resistance

- Polarity

- Earth electrode resistance (TT systems)

2. Tests with the supply connected

Polarity shall be reverified before energised testing.

The following tests shall then be completed:

- Earth fault loop impedance (Zs)

- Prospective fault current (PSC / PFC)

- Verification of protective device performance

- Functional testing of equipment

- RCD testing in accordance with Section (k) below

All results shall be recorded on approved certification, such as:

- Electrical Installation Certificate (EIC)

- Associated schedules of inspection and test results

(b) Periodic Inspection and Testing (EICR)

- Visual inspection

- Continuity of protective conductors and bonding

- Polarity

- Earth fault loop impedance

- Prospective fault current

- Insulation resistance (where appropriate)

- Functional testing of switching and isolation devices

- RCD testing

- Earth electrode resistance (where applicable)

- Verification of protective devices (excluding fuses where not practicable)

(c) Visual Inspection

The installation shall be visually inspected to confirm:

- Correct selection and erection of equipment

- Adequate earthing and bonding arrangements

- Correct rating and type of protective devices

- No signs of:

  - Damage

  - Overheating

  - Corrosion

  - Overloading

  - Deterioration

  - Unsuitable environmental conditions

(d) Earthing Arrangement

The supply earthing arrangement shall be identified and recorded, including:

- TN-S

- TN-C-S (PME)

- TT

(e) Continuity of Protective Conductors

All protective conductors shall be tested to verify:

- Electrical continuity

- Adequate cross-sectional area

- Correct connection

(f) Insulation Resistance

Insulation resistance testing shall be carried out as follows:

- 500 V DC for circuits up to 500 V

- 250 V DC where equipment may be damaged (e.g. electronic circuits)

- 1000 V DC where required for higher voltage systems

Minimum acceptable values shall comply with BS 7671.

(g) Polarity

Polarity shall be verified to ensure:

- All protective devices are connected in the line conductor only

- No reversal of line and neutral

(h) Earth Fault Loop Impedance (Zs)

Measurements shall be taken:

- At the origin

- At the furthest point of each circuit

Measured values shall be equal to or less than maximum permitted values specified in BS 7671 to ensure automatic disconnection of supply (ADS).

(i) Test Records

All inspection and test results shall:

- Be recorded on appropriate certification

- Be reviewed for compliance

- Confirm satisfactory operation of protection systems

(j) Isolation and Switching Devices

All devices provided for isolation and switching shall:

- Be functionally tested

- Be correctly labelled

- Operate effectively and safely

(k) 	Operation of Residual Current Devices to BS EN 61008 and 61009

APPENDIX 14/8:

ON SITE ELECTRICAL EQUIPMENT FOR LIGHTING UNITS

Within existing installation every effort shall be made to replace like with like or with equipment approved by Overseeing Organisation.  Where identical equipment is no longer available for replacements, agreement must be reached with the Overseeing Organisation as to an appropriate substitution.  The Overseeing Organisation reserves the right to instruct a Contractor not to replace like with like.  All lanterns to be provided with NEMA socket and gear enclosed.

APPENDIX 14/9:

SPECIAL LIGHTING UNITS

| COLUMN LOCATION | DETAILS |
| --- | --- |
| Various |  |
| 5 - 6m mid hinged columns |  |
| Lamp | Various |
| Lantern | Various |
| Lowering Principle | Counter balanced mid-hinged column |

| COLUMN LOCATION | DETAILS |
| --- | --- |
| 5 - 6 mt base hinged column |  |
| Lamp | Various |
| Lantern | Various |
| Lowering Principle | Counter balanced mid-hinged column |

| COLUMN LOCATION | DETAILS |
| --- | --- |
| Amlwch Port |  |
| Various |  |
| 8 - 14 mt hinged columns |  |
| Lamp | Various |
| Lantern | Various |
| Lowering Principle | Counter balanced mid-hinged column |

APPENDIX 14/10:

LOCATION OF ILLUMINATED ROAD SIGNS

The position and description of Illuminated Road Signs are to be shown on the Drawing relevant to each instruction.

APPENDIX 17/1:

CONCRETE: CLASSIFICATION OF MIXES

Mix Reference	Mix Reference		Mix Reference

7.5/20	30/20	40/20

Ordinary or Special Concrete (0 or S)	0	0		0

*Class of Concrete (Grade/Max. Agg. Size)	7.5/20	30/20	40/20

*Minimum Cement Content (kg/m³)	325	325	325

*Maximum Free Water/Cement Ratio	0.5	0.5	0.5

Required Workability

Max. Cement Content (kg/m³)	550	550	550

[See NG 1704.3]

*Required Type and Class of Cement	OPC+PFA	OPC+PFA	OPC+PFA

Required Source/Special Type of Aggregate

Required Admixture

Air Entrainment Required (YES/NO)	NO	NO	NO

Min. or Max. Temp. of

Fresh Concrete °C

Sampling and Testing

Other Requirements

APPENDIX 19/1:

REQUIREMENTS FOR LIGHTING COLUMNS AND BRACKET ARMS

1.	Environment: - 	Inland "A"

2.	Accessibility: - 	Ready Access

3.	Required Durability: - 	No maintenance up to 8 years

Minor maintenance from 8 years

Major maintenance after 15 years

4. Ground limit:  	Shall be taken as being up to 600mm below the door opening or as per column manufacturers recommendations.

5. 	Type of columns: - 	(i) Planted root

(ii) Flange plate

6.	Surface preparation and protective systems: -

Requirements for Steel Lighting Columns and Brackets Arms Table 19/4C

Type G1, G2a and G2b

Repainting existing lighting columns/pole brackets

Surface preparation for weathered galvanised columns shall include for the removal of all zinc salts and surface soil by thoroughly washing with detergent and water followed by a clean water rinse and allowed to dry.  Previously painted surfaces should be thoroughly cleaned as above and any loose material removed to achieve a sound and firm surface for paint adhesion, patch prime areas and overlap 50mm onto sound paint.

System A 		Silicone Alkyd

1st Coat 		Item 14 - patch prime prepared surfaces

2nd Coat 		Item 74

3rd Coat 		Item 70

System B 		Moisture Cured Polyurethane

1st Coat 		Item 160 - patch prime prepared surfaces

2nd Coat 		Item 162

3rd Coat 		Item 164

System C 		Vinyl

1st Coat 		Item 110 - patch prime prepared surfaces

2nd Coat 		Item 121

3rd Coat 		Item 164

System D

1st Coat 		Item 121 - patch prime prepared surfaces

2nd Coat 		Item 112

3rd Coat 		Item 168

7. 	Paint for any one system shall be obtained from the same manufacturer.

8. 	All access doors must be removed during each painting operation, and the internal surface of the door shall receive one coat of primer undercoat and finish as specified above, all locking devices and hinges are to be lubricated.

9. 	Where equipment is situated in soft earth the soil shall be removed to a depth of 100mm the treatment shall continue for that depth, the area shall then be reinstated.

10. Identification number/s shall be included and reapplied in accordance with the Specification and shall be verified against the record for correctness.

11. The colour of the topcoats in all cases shall be selected by the Overseeing Organisation.

5.11 APPENDICIES 20/0

APPENDIX 20/1:

SPECIFICATION FOR HIGHWAY WORKS

FORM PWS PROPRIETARY WATERPROOFING SYSTEM DATA SHEET

| 1. Manufacturer or Marketing Group 2. Product 3. British Board of Agreement Roads and Bridges Certificate 4. Concrete Deck Surface Finish 5. Manufacturers maximum and minimum application temperatures at respective relative humidities. 6. Application Temp. of Bonding Agent 7. Life of Mixed Liquid Materials 8. Overseeing Department's Recommended Minimum Depth of Flexible Surfacing and Waterproofing. 9. Manufacturer's Recommended Minimum Depth of Flexible Surfacing and Waterproofing 10. Preparation and Installation 11. Form PWS with Annex A Registered with Overseeing Department | No. Date Class U4 To comply with Clause 2005 120mm See Annex A Date |
| --- | --- |
| 12. Description of Materials | Thickness Weight Nominal Nominal mm kg/m2 width length |

APPENDIX 20/1:

SPECIFICATION FOR HIGHWAY WORKS

FORM PWS PROPRIETARY WATERPROOFING SYSTEM DATA SHEET

ANNEX "A"

Manufacturer or Marketing Group:

Product:

BBA Certificate No.:

Preamble

This waterproofing system is to be installed in accordance with this Annex and the Specification.

Should there be any contradiction between the requirements of this Annex and the Specification, the Specification requirement shall take precedence unless otherwise agreed with the Overseeing Organisation.

All materials of the waterproofing system are to be as stated on the PWS (Proprietary Waterproofing System) Data Sheet.

No substitution of any of the stated materials is permitted.

General Requirements

Immediately before the application of the primer or laying of the waterproofing system or protective layer, the concrete surface or primed surface shall be clean, dry and free from ice, frost, laitence, loose aggregate, dust and other debris and also where the adhesion to the concrete would be impaired, free from curing liquids, compounds and membranes.

On any structure, providing no damage results, plant and equipment all fitted only with rubber tyres may stand or travel on the waterproofing system solely for the purposes of laying an additional protective layer or surfacing material on that structure.  All such plant and equipment shall have its tyre treads regularly inspected and any embedded stones removed.

The permission of the Overseeing Organisation shall be obtained before the plant, equipment, and traffic is permitted onto the waterproofing system.

Rollers shall not be permitted to stand or travel directly on the waterproofing system.

Where it is necessary for plant, equipment or traffic to stand or travel on the waterproofing system, suitable temporary protection is to be provided to the satisfaction of the Overseeing Organisation.

Requirements

Date of registration:

APPENDIX 24/1:

BRICKWORK, BLOCKWORK AND STONEWORK FOR RETAINING WALLS

1.0 	General

1.1	Facework mortar jointing and pointing is to be carried out with designation (iii) mortar, with the inclusion of lime.

1.2	The stone shall be obtained from a source approved by the Overseeing Organisation.  Local limestone will be acceptable.

1.3	The exposed faces of the stones shall not be greater than 600mm long by 300mm deep and not be less than 500mm by 150mm.

1.4	Pointing of joints shall be to the Overseeing Organisation’s satisfaction.

1.5	The mortar joint shall be raked out to a depth of 20mm as the work proceeds and the joints finished by either stripping the surface of the mortar with a stiff bristle brush or raked back with a comb tool after a period of two days has elapsed, or as otherwise directed the Overseeing Organisation.

1.6	The thickness of the mortar beds and vertical joints shall not exceed 20mm.

1.7	One sample panel of shall be made and approved by the Overseeing Organisation before the masonry work is commenced.

1.8	The joints shall be thoroughly wetted before the application of the mortar.

1.9	All masonry shall be kept clean and free from mortar or grout as the work proceeds.

1.10	Where pointing is to be carried out below tidal open water or affected by sea water the Contractor's proposals shall be submitted to the Overseeing Organisation for its approval.

1.11	All walling and facework shall match existing.

1.12	Thickness of stones used for walling shall be 200mm plus/minus 25mm.

1.13	Where specified weep holes shall be built into the structure for the full depth of masonry or voids at 2m centres staggered vertically and horizontally.

2.0 	Pressure pointing

2.1	The mixing equipment shall be of a type capable of producing a grout of colloidal consistency and pointing equipment shall be capable of continuous operation with a reasonably constant pressure and delivery.  The pressure pointing pump/mixer should be a proprietary machine for pressure pointing, such as a Warsop Matrix or similar.  The mixer unit should be capable of supplying 100 cubic feet/minute at 100psi, whilst the pump unit should be capable of 60 cubic feet/minute at 100psi.

2.2	Mortar for pressure pointing shall be a pre bagged propriety mix with silica sand.  The finished mortar shall be of a dark grey colour.  Type and colour to be agreed with the Overseeing Organisation prior to the start of the contract.  Details of other mortar mixes and/or admixtures shall be submitted for the Overseeing Organisation’s approval prior to use.

2.3	Where deep penetration pressure pointing is specified, the mortar shall fill the depth of the joint as indicated on the Drawings and Price List.

2.4	Large cavities shall be partly filled with stone and the interstices filled with mortar to form a homogeneous mass.

3.0 	Hand pointing

3.1	The mortar for hand pointing shall be to Clause 2404 of the Specification.  The mortar shall be Class (ii) to Table 24/1.

3.2	The colour and texture of the mortar shall be adjusted to match the adjacent work or as otherwise agreed by the Overseeing Organisation.

3.3	The joints shall be filled with well compacted mortar nearly flush with the face of the masonry unless otherwise stated on the Drawings.  The joints shall be finished by either stripping the surface of the mortar with a stiff bristle brush or raked back or raked with a combed tool after a period of two days has elapsed.

APPENDIX 26/1:

ADDITIONAL REQUIREMENTS FOR CONCRETE FOR ANCILLARY PURPOSES

All concrete for ancillary purposes shall comply with the following requirements:

1.  Ancillary concrete mixes ST4 and ST5 shall comply with Clauses 1701, 1702, 1704, 1705 and 1706.

APPENDIX 28/1:

GULLY CLEANSING SERVICE

Contractor must provide the Overseeing Organisation with information as specified in Series 2802 Section 3.0.  Such information should be reported in

the following format: -

HIGHWAYS MAINTENANCE TERM SERVICE CONTRACT

ADDITION TO APPENDIX 28/1:

Sheet .......... of ..........

JETTING RECORD FORM

| Date | Location | Travelling to site | Time on site | Time leaving site | Travelling from site | Hours | Contractor Ref | IACC Ref | Signed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| COMMENTS |  |  |  |  |  |  |  |  |  |
| Date | Location | Travelling to site | Time on site | Time leaving site | Travelling from site | Hours | Contractor Ref | IACC Ref | Signed |
| COMMENTS |  |  |  |  |  |  |  |  |  |
| Date | Location | Travelling to site | Time on site | Time leaving site | Travelling from site | Hours | Contractor Ref | IACC Ref | Signed |
| COMMENTS |  |  |  |  |  |  |  |  |  |

HIGHWAYS MAINTENANCE TERM SERVICE CONTRACT

ADDITION TO APPENDIX 28/1 (CONTINUED):

Date .......................................													Sheet .......... of ..........

RECORD OF SNAGS

| Area | Inspector/Engineer | Works Order No. | Contractor Ref | Location and Details of Snags | Date Reported | Defect | Outstanding Works | Remedial Works | Non- Conformity | Non- Compliance | Date Completed | Successful Completion |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Client | Contractor |

APPENDIX 29/1:

EMERGENCY SERVICE

The Contractor must provide the Overseeing Organisation with information as specified in Series 2905 Section 4.0.  Such information should be reported in the following format: -

5.12 APPENDICIES 30/0

APPENDIX 30/2:

WEED CONTROL

SPECIFICATION

The Contractor is required to be Amenity Assured and shall use only skilled operatives holding the appropriate NPTC certificates of competence.  The herbicide used must be approved under the Control of Pesticides Regulations 1986.

1.0.0 HERBICIDE TREATMENT BY SCHEDULED APPLICATION

1.1.0 Scope

General weed control on rural and urban footways and channels by the application of a non-residual herbicide having a "No Hazard" COSHH status rating, containing glyphosate as an active ingredient.

The above shall be applied to all emerged weeds via hydraulic nozzle using a low volume water sprayer.

1.2.1 Application frequency

The standard frequency of spraying cycle will be twice per year consisting of one application during the spring and one in the autumn.  However, the Overseeing Organisation’s Representative may make variations to the above and require an additional mid-season application depending on the weed growth in any one season. The contractor is to provide GPS type information to prove that all adopted streets have been treated at their costs.

1.2.2 Period for completion

The Contractor shall ensure that he allocates sufficient resources to complete the spraying cycle within 6 weeks of commencement.  An extension to this period will only be considered if, in the opinion of the Overseeing Organisation’s Representative, spraying operations have been delayed substantially by prolonged adverse weather conditions.

1.3.0 Notice of commencement of treatment

The Contractor shall be given a minimum of one week's notice of the date of commencement of each application cycle by the Overseeing Organisation’s Representative.

2.0.0 HERBICIDE TREATMENT BY SELECTIVE APPLICATION

2.1.0 Scope

Selective control of Japanese knotweed and injurious weeds (as covered by The Weeds Act 1959 and the Wildlife and Countryside Act 1981) in areas identified by the Supervising Officer -

2.1.1 Japanese Knotweed (Fallopia Japonica)

To be sprayed with the chemical Triclopyr via hydraulic nozzle application.  Not to be used if stands of Japanese Knotweed are growing near water.

2.1.2 Japanese Knotweed (Fallopia Japonica) when growing near water

To be sprayed with glyphosate as an active ingredient approved for use in or near water (as 1.1.0 above).

2.1.3 Injurious Weeds

To be sprayed with the chemical 2, 4-D as an active ingredient and applied at maximum recommended label rates via hydraulic nozzle application.

2.2.0 Application frequency

As and when directed by the Overseeing Organisation’s Representative.

2.3.0 Notice of commencement of treatment

The Contractor shall be given a minimum of one week's notice of the date of commencement of selective herbicide application by the Overseeing Organisation’s Representative.

3.0.0 WEATHER CONDITIONS

The Contractor shall cease all spraying operations during wet and/or windy weather conditions which could result in the spread or flow of herbicide on to areas which are not to be treated.  The Overseeing Organisation’s Representative shall be empowered to instruct the Contractor, in writing, to cease all spraying operations where weather conditions are, in his opinion, unsuitable for spraying to continue. The decision of the Overseeing Organisation’s Representative shall be final and binding in this matter.

4.0.0 TREATMENT requirements

4.0.1 Inspections

The Contractor and Overseeing Organisation’s Representative shall undertake a joint site inspection within 21 days after completion of the herbicide treatment application.

4.0.2 Determination of the effectiveness of the herbicide treatment

After the elapse of 21 days, 95% per route of all treated vegetation shall show signs of being affected by the application of herbicide.  All treated vegetation shall be destroyed after 35 days.

4.0.3 Corrective action

Should the Contractor fail to achieve the standard required by Clause 4.0.2, the Overseeing Organisation’s Representative shall issue a written instruction specifying the areas of failure and may withhold from any payment made to the Contractor, such sum as is reasonably commensurate with the extent of the failure until it is rectified.  The Contractor shall carry out, within 14 days of such instruction, any corrective action required and shall bear all costs in relation to such.

4.0.3 Definition of new seedling growth

For the purposes of this Specification any growth of seedlings which appears immediately after any application treatment will be deemed not to be included as part of the treatment for the purposes of corrective action.

5.0.0 EQUIPMENT

5.0.1 Calibration

When using equipment, the Contractor shall submit details of the calibration calculations indicating the normal operating speed and confirming that the herbicide will be uniformly applied at the correct dose rate stipulated.  All spray settings of kerb spray units fitted to equipment are to be pre-set, and no adjustments shall be made by the driver whilst the vehicle is in motion.

Handheld lances or similar equipment should not be used by the operator whilst the equipment is in motion, unless the equipment is specifically designed for one handed operation.  Under no circumstances shall the driver use such apparatus to the rear of the equipment.

5.0.2 Cleansing and repairs

No cleansing or repair of weed spraying equipment shall be carried out on the highway.  All work of this nature must be carried out strictly in accordance with the Control of Pesticides Regulations 1986, Control of Pollution Act 1974 and relevant Codes of Practice.

6.0.0 POLLUTION

The Contractor shall take all reasonable precautions to prevent pollution of any rivers, waterways, drains, watercourses, lakes and underground water resources (including percolating water) which may adversely affect the quality or appearance of the water or cause injury or death to animal and plant life.  Under no circumstances should surplus spray or washings be disposed of into the public drainage system or natural watercourse and river systems.  The Contractor’s attention is drawn to the special requirements of the Environment Agency forming part of the Conditions of Contract.

7.0.0 TRAFFIC safety and control

Refer to Appendix 1/17 Traffic safety measures and control shall be carried out in accordance with the requirements of Type C works, Chapter 8 of the Traffic Signs Manual published by HMSO or any amendment thereof for the time being in force and the Contractor shall provide, erect and remove on completion all necessary signs, barriers cones etc. as required and the rates inserted in the Schedule of Rates shall cover all the Contractor's obligations in this respect.

The use of quad bikes for undertaking this work will not be accepted, unless agreed beforehand with the Overseeing Organisation and then only in exceptional circumstances.

If quad bikes are allowed, they should only be used within HSE Guidelines, i.e. operatives must have had specific quad/ATV training, crash helmet must be worn at all times whilst work is being carried out (Health and Safety at Work Act) at no time should the operative need to take his hands from the handlebars.

METHOD STATEMENT FOR WEED CONTROL

The Tenderer must provide all the information required by completing this document.

Name of Contractor's Representative:  ..........................................................................

Position held within the Company:  ...............................................................................

Telephone Number:  .....................................................................................................

1.  operation

The Contractor shall supply brief details below of how he intends to carry out the following: -

a.  Method of application

............................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

b.  Traffic Control

............................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

c.  Control of pollution

............................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

2.  HERBICIDE TREATMENT BY SCHEDULED APPLICATION

A.  MATERIALS

A non-residual herbicide having a "No Hazard" COSHH status rating, containing glyphosate at an active ingredient.

Product Intended to be Used

| Chemical | Product name | Dosage rate | Manufacturer | MAFF ref. number |
| --- | --- | --- | --- | --- |

B.  EQUIPMENT

Hand Lances

Name of Type                                                                     Manufacturer

..............................................................           ............................................................

..............................................................           ............................................................

..............................................................           ............................................................

..............................................................           ............................................................

Plant

Name of Type                                                                      Manufacturer

..............................................................           ............................................................

..............................................................           ............................................................

..............................................................           ............................................................

..............................................................           ............................................................

3.  HERBICIDE TREATMENT BY SELECTIVE APPLICATION

A.  MATERIALS

JAPANESE KNOTWEED (FALLOPIA JAPONICA) - to be sprayed with the chemical Triclopyr via hydraulic nozzle application.  Not to be used if stands of Japanese Knotweed are growing near water.

JAPANESE KNOTWEED (FALLOPIA JAPONICA) - when growing near water, to be sprayed with Glyphosate at an active ingredient and approved for use in or near water.

INJURIOUS WEEDS - To be sprayed with the chemical 2, 4-D applied at maximum recommended label rates via hydraulic nozzle application.

Product intended to be used

| Chemical | Product name | Dosage rate | Manufacturer | MAFF ref. Number | Supplier |
| --- | --- | --- | --- | --- | --- |
| Triclopyr |  | * |  |  |  |
| Glyphosate |  | * |  |  |  |
| 2, 4-D |  | Max. label dose |  |  |  |

* As specified by manufacturer.

B.  EQUIPMENT

Hydraulic Nozzle -

Name of Type                                                                   Manufacturer

..............................................................           ............................................................

..............................................................           ............................................................

..............................................................           ............................................................

..............................................................           ............................................................

..............................................................           ............................................................

..............................................................           ............................................................

Note: The Council reserves the right to vary the way selective herbicide treatment is paid for to a time taken and material used basis.

4.  OPERATIVES DETAILS

(Copies of NPTC Certificates to be enclosed - NOT ORIGINALS)

| Name | National Insurance Number | NPTC Certificate Number |
| --- | --- | --- |

5.  ADVISORY STAFF

|  | Name | Basis Professional Reg. No |  |
| --- | --- | --- | --- |

Note: Should there be any intention to employ new operatives either before commencement or during the contract period details must be notified to the Overseeing Organisation’s Representative prior to the person starting work in this Authority.

SUMMARY OF INDICATIVE QUANTITIES

| Scheduled Application on County roads Spring Season 1. Treatment of urban channels 2. Treatment of rural channels 3. Treatment of urban footways 4. Treatment of rural footways 5. A5 – Treatment of Urban Channel 6. A5 – Treatment of Rural Channel 7. A5 – Treatment of Urban Footway 8. A5 – Treatment of Rural Footway 9. Cycle routes Route 1 Route 2 Route 3 Route 4 Route 5 Route 6 Route 7 Route 8 Route 9 Route 10 Route 11 Route 12 Route 13 Route 14 Route 15 10. Housing Footways 11. Bridge Structures | Quantity 960.40 Km 84.50 Km 517623 m2 74104 m2 11.8 km 35.9 km 31330 m2 35449 m2 58.70 m2 2901.80 m2 92.20 m2 92.20 m2 92.20 m2 92.20 m2 92.20 m2 92.20 m2 92.20 m2 92.20 m2 92.20 m2 92.20 m2 92.20 m2 m2 3121.7 m2 4894.26 m2 3154.68 m2 |
| --- | --- |

APPENDIX 31/1:

WINTER SERVICE

Contractor must provide the Overseeing Organisation with information as specified in Series 3003.  Such information should be reported in the following format: -

|  | Winter Service Report |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ref ... |  |  | Client Contact ... |  |  |  |  |  | Client Order No ... |  |  |  |
|  | Date of Instruction ... |  |  |  |  |  |  |  | Time of Instruction |  |  |  |  |
|  | Operations commence at ... |  |  |  |  |  |  |  | Rate of spread 10 grams/m² |  |  |  |  |
|  | Operation date |  |  |  |  |  |  |  | Rate of spread 20 grams/m² |  |  |  |  |
|  | Precautionary Salting ... |  |  |  | Other - |  |  |  |  |  |  |  |  |
|  | Post-Treatment Salting ... |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Post-Treatment Salting and Ploughing ... |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gritting |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gritter Operative Name |  | Route Ref No. |  |  | Start time |  | Finish time |  | Completion Period |  | Approx Stock Used |  |
| NORMAL ROUTE |  |  | 1 |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  | 2 |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  | 3 |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  | 4 |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  | 5 |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  | 6 |  |  |  |  |  |  | Hours |  | tonnes |  |
|  | Total kilometres |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SPOT GRITTING DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Approx location | Route Ref No. |  |  |  |  | Start time |  | Finish Time | Completion Period |  | Approx Stock Used |  |
| SPOT GRITTING DETAILS |  |  |  |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Hours |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Approx total stock used |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Stock in Depot |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Stock remaining |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | Stock Delivered |  | tonnes |  |
|  |  |  |  |  |  |  |  |  |  | TOTAL STOCK NOW HELD |  | tonnes |  |
|  | Details for non-compliance and reason |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Signed for Contractor ............................................................................................. |  |  |  |  |  |  |  |  | Date: ............................ |  |  |  |

ADDITION TO APPENDIX 31/1:

Refer to contract drawings (schedules) 3_2

|  | ADDITION TO APPENDIX 31/1 (CONTINUED): |
| --- | --- |

HIGHWAYS MAINTENANCE TERM SERVICE CONTRACT

WINTER SERVICE SCHEDULE AUDIT

| Inspector: .......................... | Check Action |  | Confirm Action Time |  | Actual Action Time |  |
| --- | --- | --- | --- | --- | --- | --- |

Route No: 		........................................

Vehicle Reg No.:		........................................

Driver:		........................................

Salt/m² rate:		........................................	Check gate: 	...........................

Salt/tonnes (loaded):	........................................

Start Time & Date:	........................................

Start Mileage:		........................................

Start Gritting Time (Route) ...........................................

Finish Gritting Time (Route)....................................

Finish Mileage:		........................................

Finish Time:		........................................

Salt Returned (Tonnes):	........................................

Actual Road Conditions/Air Temps

................................................................................................................................

................................................................................................................................

Comments (Seepages areas etc.)

................................................................................................................................

................................................................................................................................

................................................................................................................................

Timescales as per contract - 2½ hrs

| Default | Yes |  | No |  |  |
| --- | --- | --- | --- | --- | --- |

ADDITION TO APPENDIX 31/1 (CONTINUED):

GRITTING VEHICLES - SERVICE SCHEDULE – 6 WEEKLIES

| Customer Fleet Number Registration Odometer Inspection Date Service Type Job Number Customer Order Ref Manufacturer Model Chassis Number | Foreman – Job Signoff All safety defects listed have been corrected and to the best of my knowledge and belief the vehicle has left this workshop in a safe and roadworthy condition Name: Date: |
| --- | --- |

| IM | Defect |  |  | Action |  | Rectified By |
| --- | --- | --- | --- | --- | --- | --- |
| Safety Related Defects |  |  |  |  |  |  |
| Non-Safety Related Defects |  |  |  |  |  |  |
| Monitored Defects |  |  |  |  |  |  |
| Note: Items marked as ‘Monitored’ are deemed to be serviceable at the time of inspection and are marked as such to ensure they are highlighted on the next scheduled inspection |  |  |  |  |  |  |
| Customer Reported Defects |  |  |  |  |  |  |
| Nearside Tyres |  |  |  | Offside Tyres |  |  |
| #1 Lining 10mm | Depth: Pressure: |  | Depth: Pressure: | #1 Lining 10mm | Depth: Pressure: | Depth: Pressure: |
| #2 Lining 10mm | Depth: Pressure: |  | Depth: Pressure: | #2 Lining 10mm | Depth: Pressure: | Depth: Pressure: |
| Brake Performance |  |  |  | Additional Information |  |  |
| Test Type |  |  |  | Next 2-year tacho check |  |  |
| Laden/Un-laden |  |  |  |  |  |  |
| Service Brake (71) |  |  |  |  |  |  |
| Secondary Brake (72) |  |  |  |  |  |  |
| Parking Brake (73) |  |  |  |  |  |  |

| Customer Fleet Number Registration Odometer Inspection Date Service Type |  |
| --- | --- |
| Activity: S&M |  |
| Key: √ = Satisfactory M = Monitored R = Requires attention NA = Not applicable NC = Not complete |  |

| Ground level |  |  |  |  | Activity signoff: S&M |
| --- | --- | --- | --- | --- | --- |
| IM |  | Check |  |  | I declare that this vehicle has been inspected in accordance with the current VOSA testers manual, and all applicable items have been checked and found to be serviceable except as noted Name: Date: |
|  |  | Registration Plates |  |  |  |
|  |  | Exhaust emissions |  |  |  |
|  |  | Road wheels, hubs, wheel fixings & retorque |  |  |  |
|  |  | Tyres – size, type, load index |  |  |  |
|  |  | Tyres – condition and wear |  |  |  |
|  |  | Sideguards, rear under-run devices and bumper bars |  |  |  |
|  |  | Spare wheel and carrier |  | NA |  |
|  |  | Vehicle to trailer coupling plus air/electrical Suzie’s |  | NA |  |
|  |  | Spray suppression, wings and wheel arches |  |  |  |
|  |  | Cab security |  |  |  |
|  |  | Cab doors |  |  |  |
|  |  | Security of body, Containers & Crane Support Legs |  |  |  |
|  |  | Conspicuity Markings – Check conformity |  |  |  |
|  |  | Condition of body |  |  |  |
|  |  | Speed limiter |  |  |  |
|  |  | Electrical wiring & equipment |  |  |  |
|  |  | Fuel tanks and system |  |  |  |
|  |  | Axles, stub axles and wheel bearings |  |  |  |
|  |  | Rear markings and reflectors |  |  |  |
|  |  | Lamps |  |  |  |
|  |  | Direction indicators and hazard warning lamps |  |  |  |
|  |  | Aim of headlamps |  |  |  |
|  |  | Other dangerous defects |  |  |  |
| In cab |  |  |  |  |  |
| IM |  | Check |  |  |  |
|  |  | VTG6 plate in place |  |  |  |
|  |  | Seat belts & Restraint systems |  |  |  |
|  |  | Cab floors and steps |  |  |  |
|  |  | Seats |  |  |  |
|  |  | Mirrors & indirect vision devices |  |  |  |
|  |  | Glass and view of the road |  |  |  |
|  |  | Windscreen wipers and washers |  |  |  |
|  |  | Speedometer / tachograph |  |  |  |
|  |  | Horn |  |  |  |
|  |  | Driving controls |  |  |  |
|  |  | Steering controls |  |  |  |
|  |  | Pressure / vacuum warning and build-up |  |  |  |
|  |  | Service brake pedal |  |  |  |
|  |  | Service brake operation |  |  |  |
|  |  | Hand operated brake control valves |  |  |  |
|  |  | Electrical wiring & equipment |  |  |  |
|  |  | Other dangerous defects |  |  |  |
| Under vehicle |  |  |  |  |  |
| IM |  | Check |  |  |  |
|  |  | Condition of chassis |  |  |  |
|  |  | Engine and transmission mountings |  |  |  |
|  |  | Oil leaks |  |  |  |
|  |  | Exhaust systems |  |  |  |
|  |  | Suspension |  |  |  |
|  |  | Steering |  |  |  |
|  |  | Transmission |  |  |  |
|  |  | Brake systems and components |  |  |  |
|  |  | Other dangerous defects |  |  |  |
|  |  | Additional braking devices |  |  |  |
| Customer Fleet Number Registration Odometer Inspection Date Service Type |  |  |  |  |  |
| Activity: P3 |  |  |  |  |  |
| Key: √ = Satisfactory M = Monitored R = Requires attention NA = Not applicable NC = Not complete |  |  |  |  |  |

| Ground level |  |  |  | Activity signoff: P3 |
| --- | --- | --- | --- | --- |
| IM |  | Check |  | I declare that the vehicle has been serviced in accordance with current instructions, and all applicable items have been checked and found to be serviceable except as noted Name: Date: |
|  |  | Cab fastening, attachment, mechanism – check |  |  |
|  |  | Bogie pivot pins – lubricate |  |  |
|  |  | AdBlue level – check |  |  |
|  |  | Battery & terminals – check security & clean |  |  |
|  |  | Battery fluid level – check / top up |  |  |
|  |  | Clutch fluid level – check / top up |  |  |
|  |  | Windscreen washer fluid level – check / top up |  |  |
|  |  | Air filter indicator – check for activation |  |  |
|  |  | Coolant level – check / top up |  |  |
|  |  | Engine oil level – check / adjust |  |  |
|  |  | Accessible nuts & bolts – check tightness |  |  |
|  |  | Fuel prefilter / water separator – drain |  |  |
|  |  | V belt(s) – check condition / adjust |  |  |
|  |  | Radiator & intercooler – check / clean |  |  |
|  |  | Coolant & intercooler connections – check for leak |  |  |
|  |  | Engine idling speed – check / adjust |  |  |
|  |  | Power steering oil level – check / top up |  |  |
|  |  | King pins – jack up & lubricate |  |  |
|  |  | Trunnion shaft of rear bogie – check for leaks |  |  |
|  |  | 5th wheel attachment, operation – check & lubricate | NA |  |
|  |  | Trailer coupling – check & lubricate | NA |  |
|  |  | Autolube, where fitted – check operation & top up | NA |  |
|  |  | Check oil seals (leakage) of the leading rear axle |  |  |
|  |  | Check wheel bearing play of the leading rear axle |  |  |
|  |  | Check heater hose re TSB 40-0246 |  |  |
| In cab |  |  |  |  |
| IM |  | Check |  |  |
|  |  | Instruments and controls – check operation |  |  |
|  |  | Auxiliary heating – Test operation |  |  |
|  |  | Air conditioning – Test operation |  |  |
|  |  | Tachograph – check ‘K’ factor plate |  |  |
|  |  | Maintenance record – Stamp |  |  |
| Road test |  |  |  |  |
| IM |  | Check |  |  |
|  |  | Engine oil and pressure – check |  |  |
|  |  | Exhaust brake – Test operation |  |  |
|  |  | RSL performance / speed setting – check |  |  |
|  |  | Steering and brake behaviour – check |  |  |
|  |  | Clutch / gear / range / gate – check operation |  |  |
|  |  | Instruments & warning lights – check operation |  |  |
|  |  | Cab interior – check cleanliness |  |  |
|  |  | Road wheels removed? – retorque wheel nuts |  |  |
|  |  | ABS / EBS warning lights – check operation |  |  |
| Under vehicle |  |  |  |  |
| IM |  | Check |  |  |
|  |  | Grease as per lubricating schedule |  |  |
|  |  | Intarder, where fitted – check oil level | NA |  |
|  |  | Accessible nuts / bolts – check tightness |  |  |
|  |  | Clutch wear indicator – check / record mm as required |  |  |
|  |  | Gearbox, transfer case, PTO – check for leaks / level |  |  |
|  |  | Differential(s) – check oil leakage & level(s) |  |  |
|  |  | Brake pad / lining thickness – check |  |  |
|  |  | Auto transmission, where fitted – check oil level |  |  |
|  |  | Security of brake pipes – check |  |  |

ADDITION TO APPENDIX 31/1 (CONTINUED):

GRITTING VEHICLES - SERVICE SCHEDULE – 3 MONTHLY

| Customer Fleet Number Registration Odometer Inspection Date Service Type Job Number Customer Order Ref Manufacturer Model Chassis Number | Foreman – Job Signoff All safety defects listed have been corrected and to the best of my knowledge and belief the vehicle has left this workshop in a safe and roadworthy condition Name: Date: |
| --- | --- |

| IM | Defect |  |  | Action |  | Rectified By |
| --- | --- | --- | --- | --- | --- | --- |
| Safety Related Defects |  |  |  |  |  |  |
| Non-Safety Related Defects |  |  |  |  |  |  |
| Monitored Defects |  |  |  |  |  |  |
| Note: Items marked as ‘Monitored’ are deemed to be serviceable at the time of inspection and are marked as such to ensure they are highlighted on the next scheduled inspection |  |  |  |  |  |  |
| Customer Reported Defects |  |  |  |  |  |  |
| Nearside Tyres |  |  |  | Offside Tyres |  |  |
| #1 Lining 10mm | Depth: Pressure: |  | Depth: Pressure: | #1 Lining 10mm | Depth: Pressure: | Depth: Pressure: |
| #2 Lining 10mm | Depth: Pressure: |  | Depth: Pressure: | #2 Lining 10mm | Depth: Pressure: | Depth: Pressure: |
| Brake Performance |  |  |  | Additional Information |  |  |
| Test Type |  |  |  | Next 2-year tacho check |  |  |
| Laden/Un-laden |  |  |  |  |  |  |
| Service Brake (71) |  |  |  |  |  |  |
| Secondary Brake (72) |  |  |  |  |  |  |
| Parking Brake (73) |  |  |  |  |  |  |

| Customer Fleet Number Registration Odometer Inspection Date Service Type |  |
| --- | --- |
| Activity: S&M |  |
| Key: √ = Satisfactory M = Monitored R = Requires attention NA = Not applicable NC = Not complete |  |

| Ground level |  |  |  |  | Activity signoff: S&M |
| --- | --- | --- | --- | --- | --- |
| IM |  | Check |  |  | I declare that this vehicle has been inspected in accordance with the current VOSA testers manual, and all applicable items have been checked and found to be serviceable except as noted Name: Date: |
|  |  | Registration Plates |  |  |  |
|  |  | Exhaust emissions |  |  |  |
|  |  | Road wheels, hubs, wheel fixings & retorque |  |  |  |
|  |  | Tyres – size, type, load index |  |  |  |
|  |  | Tyres – condition and wear |  |  |  |
|  |  | Sideguards, rear under-run devices and bumper bars |  |  |  |
|  |  | Spare wheel and carrier |  | NA |  |
|  |  | Vehicle to trailer coupling plus air/electrical Suzie’s |  |  |  |
|  |  | Spray suppression, wings and wheel arches |  |  |  |
|  |  | Cab security |  |  |  |
|  |  | Cab doors |  |  |  |
|  |  | Security of body, Containers & Crane Support Legs |  |  |  |
|  |  | Conspicuity Markings – Check conformity |  |  |  |
|  |  | Condition of body |  |  |  |
|  |  | Speed limiter |  |  |  |
|  |  | Electrical wiring & equipment |  |  |  |
|  |  | Fuel tanks and system |  |  |  |
|  |  | Axles, stub axles and wheel bearings |  |  |  |
|  |  | Rear markings and reflectors |  |  |  |
|  |  | Lamps |  |  |  |
|  |  | Direction indicators and hazard warning lamps |  |  |  |
|  |  | Aim of headlamps |  |  |  |
|  |  | Other dangerous defects |  |  |  |
| In cab |  |  |  |  |  |
| IM |  | Check |  |  |  |
|  |  | VTG6 plate in place |  |  |  |
|  |  | Seat belts & Restraint systems |  |  |  |
|  |  | Cab floors and steps |  |  |  |
|  |  | Seats |  |  |  |
|  |  | Mirrors & indirect vision devices |  |  |  |
|  |  | Glass and view of the road |  |  |  |
|  |  | Windscreen wipers and washers |  |  |  |
|  |  | Speedometer / tachograph |  |  |  |
|  |  | Horn |  |  |  |
|  |  | Driving controls |  |  |  |
|  |  | Steering controls |  |  |  |
|  |  | Pressure / vacuum warning and build-up |  |  |  |
|  |  | Service brake pedal |  |  |  |
|  |  | Service brake operation |  |  |  |
|  |  | Hand operated brake control valves |  |  |  |
|  |  | Electrical wiring & equipment |  |  |  |
|  |  | Other dangerous defects |  |  |  |
| Under vehicle |  |  |  |  |  |
| IM |  | Check |  |  |  |
|  |  | Condition of chassis |  |  |  |
|  |  | Engine and transmission mountings |  |  |  |
|  |  | Oil leaks |  |  |  |
|  |  | Exhaust systems |  |  |  |
|  |  | Suspension |  |  |  |
|  |  | Steering |  |  |  |
|  |  | Transmission |  |  |  |
|  |  | Brake systems and components |  |  |  |
|  |  | Other dangerous defects |  |  |  |
|  |  | Additional braking devices |  |  |  |
| Customer Fleet Number Registration Odometer Inspection Date Service Type |  |  |  |  |  |
| Activity: P3 |  |  |  |  |  |
| Key: √ = Satisfactory M = Monitored R = Requires attention NA = Not applicable NC = Not complete |  |  |  |  |  |

| Ground level |  |  |  | Activity signoff: P3 |
| --- | --- | --- | --- | --- |
| IM |  | Check |  | I declare that the vehicle has been serviced in accordance with current instructions, and all applicable items have been checked and found to be serviceable except as noted Name: Date: |
|  |  | Cab fastening, attachment, mechanism – check |  |  |
|  |  | Bogie pivot pins – lubricate |  |  |
|  |  | Adblue level – check |  |  |
|  |  | Battery & terminals – check security & clean |  |  |
|  |  | Battery fluid level – check / top up |  |  |
|  |  | Clutch fluid level – check / top up |  |  |
|  |  | Windscreen washer fluid level – check / top up |  |  |
|  |  | Air filter indicator – check for activation |  |  |
|  |  | Coolant level – check / top up |  |  |
|  |  | Engine oil level – check / adjust |  |  |
|  |  | Accessible nuts & bolts – check tightness |  |  |
|  |  | Fuel prefilter / water separator – drain |  |  |
|  |  | V belt(s) – check condition / adjust |  |  |
|  |  | Radiator & intercooler – check / clean |  |  |
|  |  | Coolant & intercooler connections – check for leak |  |  |
|  |  | Engine idling speed – check / adjust |  |  |
|  |  | Power steering oil level – check / top up |  |  |
|  |  | King pins – jack up & lubricate |  |  |
|  |  | Trunnion shaft of rear bogie – check for leaks |  |  |
|  |  | 5th wheel attachment, operation – check & lubricate | NA |  |
|  |  | Trailer coupling – check & lubricate | NA |  |
|  |  | Autolube, where fitted – check operation & top up | NA |  |
|  |  | Check oil seals (leakage) of the leading rear axle |  |  |
|  |  | Check wheel bearing play of the leading rear axle |  |  |
|  |  | Check heater hose re TSB 40-0246 |  |  |
| In cab |  |  |  |  |
| IM |  | Check |  |  |
|  |  | Instruments and controls – check operation |  |  |
|  |  | Auxiliary heating – Test operation |  |  |
|  |  | Air conditioning – Test operation |  |  |
|  |  | Tachograph – check ‘K’ factor plate |  |  |
|  |  | Maintenance record – Stamp |  |  |
| Road test |  |  |  |  |
| IM |  | Check |  |  |
|  |  | Engine oil and pressure – check |  |  |
|  |  | Exhaust brake – Test operation |  |  |
|  |  | RSL performance / speed setting – check |  |  |
|  |  | Steering and brake behaviour – check |  |  |
|  |  | Clutch / gear / range / gate – check operation |  |  |
|  |  | Instruments & warning lights – check operation |  |  |
|  |  | Cab interior – check cleanliness |  |  |
|  |  | Road wheels removed? – retorque wheel nuts |  |  |
|  |  | ABS / EBS warning lights – check operation |  |  |
| Under vehicle |  |  |  |  |
| IM |  | Check |  |  |
|  |  | Grease as per lubricating schedule |  |  |
|  |  | Intarder, where fitted – check oil level | NA |  |
|  |  | Accessible nuts / bolts – check tightness |  |  |
|  |  | Clutch wear indicator – check / record mm as required |  |  |
|  |  | Gearbox, transfer case, PTO – check for leaks / level |  |  |
|  |  | Differential(s) – check oil leakage & level(s) |  |  |
|  |  | Brake pad / lining thickness – check |  |  |
|  |  | Auto transmission, where fitted – check oil level | NA |  |
|  |  | Security of brake pipes – check |  |  |

ADDITION TO APPENDIX 31/1 (CONTINUED):

GRITTING VEHICLES - SERVICE SCHEDULE – SUMMER

| Customer Fleet Number Registration Odometer Inspection Date Service Type Job Number Customer Order Ref Manufacturer Model Chassis Number | Foreman – Job Signoff All safety defects listed have been corrected and to the best of my knowledge and belief the vehicle has left this workshop in a safe and roadworthy condition Name: Date: |
| --- | --- |

| IM | Defect |  |  | Action |  | Rectified By |
| --- | --- | --- | --- | --- | --- | --- |
| Safety Related Defects |  |  |  |  |  |  |
| Non-Safety Related Defects |  |  |  |  |  |  |
| Monitored Defects |  |  |  |  |  |  |
| Note: Items marked as ‘Monitored’ are deemed to be serviceable at the time of inspection and are marked as such to ensure they are highlighted on the next scheduled inspection |  |  |  |  |  |  |
| Customer Reported Defects |  |  |  |  |  |  |
| Nearside Tyres |  |  |  | Offside Tyres |  |  |
| #1 Lining 10mm | Depth: Pressure: |  | Depth: Pressure: | #1 Lining 10mm | Depth: Pressure: | Depth: Pressure: |
| #2 Lining 10mm | Depth: Pressure: |  | Depth: Pressure: | #2 Lining 10mm | Depth: Pressure: | Depth: Pressure: |
| Brake Performance |  |  |  | Additional Information |  |  |
| Test Type |  |  |  | Next 2-year tacho check |  |  |
| Laden/Un-laden |  |  |  |  |  |  |
| Service Brake (71) |  |  |  |  |  |  |
| Secondary Brake (72) |  |  |  |  |  |  |
| Parking Brake (73) |  |  |  |  |  |  |

| Customer Fleet Number Registration Odometer Inspection Date Service Type |  |
| --- | --- |
| Activity: S&M |  |
| Key: √ = Satisfactory M = Monitored R = Requires attention NA = Not applicable NC = Not complete |  |

| Ground level |  |  |  |  | Activity signoff: S&M |
| --- | --- | --- | --- | --- | --- |
| IM |  | Check |  |  | I declare that this vehicle has been inspected in accordance with the current VOSA testers manual, and all applicable items have been checked and found to be serviceable except as noted Name: Date: |
|  |  | Registration Plates |  |  |  |
|  |  | Exhaust emissions |  |  |  |
|  |  | Road wheels, hubs, wheel fixings & retorque |  |  |  |
|  |  | Tyres – size, type, load index |  |  |  |
|  |  | Tyres – condition and wear |  |  |  |
|  |  | Sideguards, rear under-run devices and bumper bars |  |  |  |
|  |  | Spare wheel and carrier |  | NA |  |
|  |  | Vehicle to trailer coupling plus air/electrical Suzie’s |  | NA |  |
|  |  | Spray suppression, wings and wheel arches |  |  |  |
|  |  | Cab security |  |  |  |
|  |  | Cab doors |  |  |  |
|  |  | Security of body, Containers & Crane Support Legs |  |  |  |
|  |  | Conspicuity Markings – Check conformity |  |  |  |
|  |  | Condition of body |  |  |  |
|  |  | Speed limiter |  |  |  |
|  |  | Electrical wiring & equipment |  |  |  |
|  |  | Fuel tanks and system |  |  |  |
|  |  | Axles, stub axles and wheel bearings |  |  |  |
|  |  | Rear markings and reflectors |  |  |  |
|  |  | Lamps |  |  |  |
|  |  | Direction indicators and hazard warning lamps |  |  |  |
|  |  | Aim of headlamps |  |  |  |
|  |  | Other dangerous defects |  |  |  |
| In cab |  |  |  |  |  |
| IM |  | Check |  |  |  |
|  |  | VTG6 plate in place |  |  |  |
|  |  | Seat belts & Restraint systems |  |  |  |
|  |  | Cab floors and steps |  |  |  |
|  |  | Seats |  |  |  |
|  |  | Mirrors & indirect vision devices |  |  |  |
|  |  | Glass and view of the road |  |  |  |
|  |  | Windscreen wipers and washers |  |  |  |
|  |  | Speedometer / tachograph |  |  |  |
|  |  | Horn |  |  |  |
|  |  | Driving controls |  |  |  |
|  |  | Steering controls |  |  |  |
|  |  | Pressure / vacuum warning and build-up |  |  |  |
|  |  | Service brake pedal |  |  |  |
|  |  | Service brake operation |  |  |  |
|  |  | Hand operated brake control valves |  |  |  |
|  |  | Electrical wiring & equipment |  |  |  |
|  |  | Other dangerous defects |  |  |  |
| Under vehicle |  |  |  |  |  |
| IM |  | Check |  |  |  |
|  |  | Condition of chassis |  |  |  |
|  |  | Engine and transmission mountings |  |  |  |
|  |  | Oil leaks |  |  |  |
|  |  | Exhaust systems |  |  |  |
|  |  | Suspension |  |  |  |
|  |  | Steering |  |  |  |
|  |  | Transmission |  |  |  |
|  |  | Brake systems and components |  |  |  |
|  |  | Other dangerous defects |  |  |  |
|  |  | Additional braking devices |  |  |  |
| Customer Fleet Number Registration Odometer Inspection Date Service Type |  |  |  |  |  |
| Activity: X |  |  |  |  |  |
| Key: √ = Satisfactory M = Monitored R = Requires attention NA = Not applicable NC = Not complete |  |  |  |  |  |

| Ground level |  |  |  |  | Road test – continued |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IM |  | Check |  |  | IM |  | Check |  |  |
|  |  | Flush air circuit EAS dosing module |  |  |  |  | ABS / EBS warning lights – check operation |  |  |
|  |  | Adblue air filter / oil separator (EAS) – replace |  |  | Under vehicle |  |  |  |  |
|  |  | Cab fastening, attachment, mechanism – check |  |  | IM |  | Check |  |  |
|  |  | Bogie pivot pins – lubricate |  |  |  |  | Grease as per lubricating schedule |  |  |
|  |  | Adblue level – check |  |  |  |  | Intarder, where fitted – check oil level |  |  |
|  |  | Cab delatch mechanism – lubricate |  |  |  |  | Accessible nuts / bolts – check tightness |  |  |
|  |  | Battery & terminals – check security & clean |  |  |  |  | Clutch wear indicator – check / record mm as required |  |  |
|  |  | Battery fluid level – check / top up |  |  |  |  |  |  |  |
|  |  | Clutch fluid level – check / top up |  |  |  |  | Gearbox, transfer case, PTO – check for leaks / level |  |  |
|  |  | Windscreen washer fluid level – check / top up |  |  |  |  |  |  |  |
|  |  | Air filter indicator – check for activation |  |  |  |  | Differential(s) – check oil leakage & level(s) |  |  |
|  |  | Coolant level – check / top up |  |  |  |  | Exhaust system components – check |  |  |
|  |  | Accessible nuts & bolts – check tightness |  |  |  |  | Steering ball joints – check for play / damage |  |  |
|  |  | V belt(s) – check condition / adjust |  |  |  |  | Steering hydraulic lines / hoses / connections |  |  |
|  |  | Radiator & intercooler – check / clean |  |  |  |  | Brake components / system – check for leaks |  |  |
|  |  | Coolant & intercooler connections – check for leak |  |  |  |  | Brake pad / lining thickness – check |  |  |
|  |  | Engine idling speed – check / adjust |  |  |  |  | Component breathers – check for blockages |  |  |
|  |  | Power steering oil level – check / top up |  |  |  |  | Road springs – check condition / attachment |  |  |
|  |  | King pins – jack up & lubricate |  |  |  |  | Shock absorbers – check condition / attachment |  |  |
|  |  | Trunnion shaft of rear bogie – check for leaks |  |  |  |  | Axle / air suspension attachments – check |  |  |
|  |  | 5th wheel attachment, operation – check & lubricate |  |  |  |  | Air bellows of air suspension – check |  |  |
|  |  | Trailer coupling – check & lubricate |  |  |  |  | Air suspension X member (Primaax), tighten bolts – 1st X |  |  |
|  |  | Ancillaries equip – check operation, condition, secure |  |  |  |  |  |  |  |
|  |  | Earth connections & leakage - check |  |  |  |  | Wheel hubs – check for leaks |  |  |
|  |  | Valve clearances, 1st X service only – check / adjust |  |  |  |  | Auto transmission, where fitted – check oil level |  |  |
|  |  | Engine oil & filter(s) – replace |  |  |  |  | U bolt nuts tightening torque – check with torque wrench |  |  |
|  |  | Centrifugal oil filter, where fitted – replace |  |  |  |  |  |  |  |
|  |  | Fuel filter(s) – replace |  |  |  |  |  |  |  |
|  |  | Fuel prefilter / water Sep. filter – replace |  |  | Activity signoff: X |  |  |  |  |
|  |  | Fuel system – check for leaks |  |  | I declare that the vehicle has been serviced in accordance with current instructions, and all applicable items have been checked and found to be serviceable except as noted Name: Date: |  |  |  |  |
|  |  | Engine ECU mountings – check |  |  |  |  |  |  |  |
|  |  | Exhaust system – check for accumulated debris |  |  |  |  |  |  |  |
|  |  | Gear linkage mechanism – lubricate |  |  |  |  |  |  |  |
|  |  | Autolube, where fitted – check operation & top up |  |  |  |  |  |  |  |
|  |  | Check oil seals (leakage) of the leading rear axle |  |  |  |  |  |  |  |
|  |  | Check wheel bearing play of the leading rear axle |  |  |  |  |  |  |  |
| In cab |  |  |  |  |  |  |  |  |  |
| IM |  | Check |  |  |  |  |  |  |  |
|  |  | Instruments and controls – check operation |  |  |  |  |  |  |  |
|  |  | Auxiliary heating – Test operation |  |  |  |  |  |  |  |
|  |  | Air conditioning – Test operation |  |  |  |  |  |  |  |
|  |  | Tachograph – check ‘K’ factor plate |  |  |  |  |  |  |  |
|  |  | Maintenance record – Stamp |  |  |  |  |  |  |  |
|  |  | Next service date / tacho distance – enter in VIC |  |  |  |  |  |  |  |
| Road test |  |  |  |  |  |  |  |  |  |
| IM |  | Check |  |  |  |  |  |  |  |
|  |  | Engine oil and pressure – check |  |  |  |  |  |  |  |
|  |  | Exhaust brake – Test operation |  |  |  |  |  |  |  |
|  |  | RSL performance / speed setting – check |  |  |  |  |  |  |  |
|  |  | Steering and brake behaviour - check |  |  |  |  |  |  |  |
|  |  | Clutch / gear / range / gate – check operation |  |  |  |  |  |  |  |
|  |  | Instruments & warning lights – check operation |  |  |  |  |  |  |  |
|  |  | Cab interior – check cleanliness |  |  |  |  |  |  |  |
|  |  | Road wheels removed? – retorque wheel nuts |  |  |  |  |  |  |  |
| Customer Fleet Number Registration Odometer Inspection Date Service Type |  |  |  |  |  |  |  |  |
| Activity: Y5 |  |  |  |  |  |  |  |  |
| Key: √ = Satisfactory M = Monitored R = Requires attention NA = Not applicable NC = Not complete |  |  |  |  |  |  |  |  |

| Ground level |  |  |  |  |  | Under vehicle – continued |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IM |  |  | Check |  |  | IM |  | Check |  |
|  |  |  | Cab front pivot grease units – check / replace |  |  |  |  | Load sensing valve – check / adjust |  |
|  |  |  | EAS Prefilter – replace |  |  |  |  | Drive shafts, U/Js, C/bearings – check |  |
|  |  |  | Adblue air filter / oil separator (EAS) – replace |  |  |  |  | Crown wheel guide bolt 5.12 rear axle – adjust |  |
| 19 |  |  | Security of body, containers & crane support legs |  |  |  |  | Differential gear lock – check operation |  |
|  |  |  | Cab fastening, attachment, mechanism – check |  |  |  |  | Trailing axle lifting device – check fluid level |  |
|  |  |  | Adblue level – check |  |  |  |  | L – bracket lift axle crossmember – check |  |
|  |  |  | Cab filter cartridge – replace |  |  |  |  | ASTronic Lite fluid reservoir level – check |  |
|  |  |  | Accessible nuts & bolts – check tightness |  |  |  |  | Trailing axle lifting device pivots – check |  |
|  |  |  | Radiator & intercooler – check / clean |  |  |  |  | Intarder, specification mineral oil – replace oil |  |
|  |  |  | King pins – jack up & lubricate |  |  |  |  | Rear axle / hub, mineral oil – replace oil |  |
|  |  |  | 5th wheel attachment, operation – check & lubricate |  |  |  |  | Rear axle oil change, hub reduction 1355 axle only |  |
|  |  |  | Trailer coupling – check & lubricate |  |  |  |  | Check vibration damper |  |
|  |  |  | Earth connections & leakage – check |  |  |  |  |  |  |
|  |  |  | Cab heater filter element – replace |  |  | Activity signoff: Y5 |  |  |  |
|  |  |  | Water drains sleeve of heater – check |  |  | I declare that the vehicle has been serviced in accordance with current instructions, and all applicable items have been checked and found to be serviceable except as noted Name: Date: |  |  |  |
|  |  |  | Cab tilt pump – check fluid level |  |  |  |  |  |  |
|  |  |  | Air filter element – replace |  |  |  |  |  |  |
|  |  |  | Air filter dust trapping valve – check |  |  |  |  |  |  |
|  |  |  | Coolant – check antifreeze concentration % |  |  |  |  |  |  |
|  |  |  | Coolant filter, where fitted – replace |  |  |  |  |  |  |
|  |  |  | Steering box input spline U/J – check for play / a |  |  |  |  |  |  |
|  |  |  | Compressor line – check for carbon build up |  |  |  |  |  |  |
|  |  |  | Dryer element – replace |  |  |  |  |  |  |
|  |  |  | ECAS, where fitted – check operation & ride height |  |  |  |  |  |  |
|  |  |  | Valve clearances – check / adjust |  |  |  |  |  |  |
|  |  |  | Coolant – replace |  |  |  |  |  |  |
|  |  |  | Autolube, where fitted – check operation & top up |  |  |  |  |  |  |
|  |  |  | Check oil seals (leakage) of the leading rear axle |  |  |  |  |  |  |
|  |  |  | Check wheel bearing play of the leading rear axle |  |  |  |  |  |  |
|  |  |  | DPF filter replacement / clean |  |  |  |  |  |  |
| In cab |  |  |  |  |  |  |  |  |  |
| IM |  | Check |  |  |  |  |  |  |  |
|  |  | Maintenance record – stamp |  |  |  |  |  |  |  |
|  |  | Next service date / tacho distance – enter in VIC |  |  |  |  |  |  |  |
|  |  | Inspect brake pedal plate/bracket for corrosion |  |  |  |  |  |  |  |
|  |  | Pedal rubbers – check condition |  |  |  |  |  |  |  |
|  |  | Fuses in the central box – check genuine & correct value |  |  |  |  |  |  |  |
|  |  | Spare fuses – check present in the central box |  |  |  |  |  |  |  |
|  |  | Programme the axle overloading monitoring system |  |  |  |  |  |  |  |
| Under vehicle |  |  |  |  |  |  |  |  |  |
| IM |  |  | Check |  |  |  |  |  |  |
|  |  |  | Grease as per lubricating schedule |  |  |  |  |  |  |
|  |  |  | Accessible nuts / bolts – check tightness |  |  |  |  |  |  |
|  |  |  | Steering ball joints – check for play / damage |  |  |  |  |  |  |
|  |  |  | Steering hydraulic lines / hoses / connections |  |  |  |  |  |  |
|  |  |  | Component breathers – check for blockages |  |  |  |  |  |  |
|  |  |  | Vulcanised vibration damper – check |  |  |  |  |  |  |
|  |  |  | Change gearbox oil – NOT AS Tronic (Mineral) |  |  |  |  |  |  |
|  |  |  | Change gearbox oil (Allison Automatic) – see plan |  |  |  |  |  |  |
|  |  |  | Change gearbox oil (Mineral) |  |  |  |  |  |  |
|  |  |  | Brake cylinder fastening – check |  |  |  |  |  |  |
|  |  |  | Automatic slack adjusters – check operation |  |  |  |  |  |  |

SPREADER INSPECTION SHEET BULK SPREADER

| OPERATOR |  |  |  |
| --- | --- | --- | --- |
| LOCATION |  |  |  |
| REGISTRATION NUMBER |  |  |  |
| BODY SERIAL NUMBER |  |  |  |
| DATE |  |  |  |
| MILEAGE |  |  |  |
| ITEM | CHECK | COMMENTS | OK |
| IN CAB INSPECTIONS |  |  |  |
| Spreader Control Box | Security, Functions, Condition |  |  |
| Lighting Control Box | Security, Functions |  |  |
| Pm Weigher Box | Functions, Calibration |  |  |
| Condition Of Comms Cable | Presence, Condition, Position, Security |  |  |
| Other |  |  |  |
| Exterior Inspections |  |  |  |
| Hopper | Security, Condition |  |  |
| Salt Screen | Security, Condition, Access |  |  |
| Platform Ladders | Security, Condition |  |  |
| Hydraulic Tank | Security, Content, Leakage |  |  |
| All Hoses | Security, Leakage |  |  |
| Drive Motors & Gearbox | Security, Leakage |  |  |
| Roller Bearings | Condition |  |  |
| Conveyor Belt | Tension, Tracking, Condition |  |  |
| Belt Scrapers | Function, Condition |  |  |
| Distributor Assembly | Security, Condition, Position |  |  |
| Discharge Gate | Operation, Calibration |  |  |
| Sideguards | Security, Condition |  |  |
| Mudguards & Flap | Security, Condition |  |  |
| Chassis Screen | Security, Condition |  |  |
| Lighting, Reflectors | Presence, Condition, Position, Security |  |  |
| Marker Boards | Presence, Condition, Position, Security |  |  |
|  | INSPECTOR |  |  |

APPENDIX 31/1 CONTINUED:

WINTER SERVICES VEHICLES AND SNOW PLOUGHS

The Contractor will then maintain and service the fleet to the same standard for the duration of the contract at the overseeing organisations cost. The contractor is liable for damages incurred and picked up in the scheduled gritter checks. The contractor will also fully comprehensively insure and licence the whole vehicle gritting fleet (7 vehicles) plus the snowploughs at his cost from the starting date.

Maintain: includes all parts as required at the overseeing organisations cost.  The snow equipment to be serviced every 6 weeks during the winter service period also once annually during June to September as part of the summer overhaul.

The snow plough Equipment Schedule is shown in Table 1.

The Contractor to maintain, service and repair as necessary all aspects of the “Econ” low throw bulk spreader at the overseeing organisations cost from the stating date.

The inspection sheet to be submitted to the Overseeing Organisation every 6 weeks during the Winter Service Period and once annually as part of the summer overhaul.

5. Lease Agreement Menai Bridge and Gaerwen Depot

See appendix

7. Special Requirements

7.1 UTILITIES AND OTHER BODIES

SPECIAL REQUIREMENTS IN RELATION TO UTILITIES AND OTHER BODIES

The special requirements can be found on the Welsh HAUC website Working Group - Communications (Wales) | HAUC(UK) (hauc-uk.org.uk) for the relevant information and latest updates in relation to the contract for:

a)  Wales and West Utilities

b)  Openreach BT

c)  Electricity Generating and Distribution Companies

d)  Natural Resources Wales

e)  Network Rail

f)   Water and Sewerage Companies

7.2 WORK ON EXISTING HIGHWAYS - IACC

SPECIAL REQUIREMENTS IN RELATION TO WORK ON EXISTING HIGHWAYS - IACC

1.	The Contractor shall take into account when carrying out the work on trafficked highways the recommendations of the Joint Report of the County Surveyors Society and the Department of Transport which provides Notes for Guidance in relation to the implementation of the Health and Safety at Work Act 1974.

2.	All works carried out on or adjacent to trafficked highways shall be signed in accordance with the version of Chapter 8 of the Traffic Signs Manual that is current at the time.

3.	The Contractor shall give the Overseeing Organisation written notice of the actual dates and duration for which he requires possession of part of any trafficked highway as required in the Contract as shown in Appendix 1/17.

4.	The Contractor shall obtain the Overseeing Organisation's approval and shall inform the Police of any operations that involve the restriction of the free flow of traffic on any highway.

i)	Overseeing Organisation: -			ii)	North Wales Police Headquarters

Isle of Anglesey County Council 			Glan y Don

Maintenance Section					Colwyn Bay

Highways, Waste and Property			Clwyd

Council Offices

Llangefni

Ynys Môn

LL77 7TW

Tel:		01248 750057		Tel:  101

Please refer to Appendix 1/17 and additions regarding the requirements of the Traffic Management Act 2010.

7.4 UK HIGHWAYS A55

SPECIAL REQUIREMENTS IN RELATION TO WORK IN THE VICINITY OF THE A55 SLIP ROADS, UK HIGHWAYS A55 LIMITED

1.	'UK Highways' means UK Highways A55 Limited, the company appointed to manage the routine operation and management of the A55 Llandygai to Holyhead highway and the Menai Loop.

2.	'UK Highways' Representative' means a competent person nominated by UK Highways in writing for specific responsibilities in respect of the Contract.

3.	 UK Highways' Representative may require plans, types of work, duration and timings, so that such works can be programmed with actual maintenance closures of the A55 and the Menai Loop.

4.	UK Highways' Representative's address: -

A55 Manager

Plot No. 7

Mona Industrial Park

Mona

Anglesey.	LL65 4RJ

Tel. No. 01407 721296	Out of Hours	01286 677731 (24 hours)

5.	Liaison contact addresses:

| Organisation | Web Address |
| --- | --- |
| Isle of Anglesey County Council Gethin Gilford, Chief Engineer Highways Alan Jones, Streetworks Manager | Cyngor Sir Ynys Môn - Isle of Anglesey County Council. 01248 752149 01248 752389 |
| Gwynedd Council Highways & Engineering Dept. | Residents (llyw.cymru) |
| Traffic Wales | Traffic Wales ¦ Traffic Wales 0300 123 1213 |
| North Wales Police HQ | Home ¦ North Wales Police |
| North Wales Fire Service | North Wales Fire and Rescue Service (gov.wales) |
| Welsh Ambulance Service Locality | Home - Welsh Ambulance Services University NHS Trust. 01286 676088 |
| Holyhead Port Authority | Holyhead Port Authority |
| Natural Resources Wales | Natural Resources Wales |
| Transport for Wales | Book Train Tickets ¦ Buy Cheap Rail Tickets - No Booking Fee ¦ TfW |
| North and Mid Wales Trunk Road Agent | North and Mid Wales Trunk Road Agent (NMWTRA) ¦ Traffic Wales |

END