CCTV Maintenance for Conwy ARC

Tender Specification

Final 1.4

July 2026

Prepared by

Roger Hardiman

Technical Consultant

(updated by Conwy County Borough Council)

RJH Technical Consultancy Ltd

26 The Paddocks

Coleford

Gloucestershire

GL16 7PX

Email info@rjh.org.uk

Tel 07968 245759

1	Introduction	4

2	Overview of Requirements	5

3	Summary of Existing System	7

3.1	Public Space Camera Locations	7

3.2	Public Space Camera Types	7

3.3	Data Transmission	8

3.4	Recording System	8

3.5	Meyertech System and Video Wall	8

3.6	Police HQ Link	9

3.7	Re-Deployable (4G/5G) cameras	9

3.8	Remote Site Systems	9

3.9	NTP Time Server	9

3.10	Experience and Vendor Approval	9

3.11	Other items not in scope	10

3.12	Recent Fault History for the last 6 months	10

4	System Maintenance	11

4.1	Contract Duration	11

4.2	Response Times	13

4.3	Fault Reporting	13

4.4	Performance Monitoring	13

4.5	Planned Preventative Maintenance and Cleaning	14

4.6	Back to Back Maintenance Contracts	14

4.7	Engineer Training	14

4.8	Software and Firmware updates	14

4.9	Spares Holding	14

5	Ad-Hoc Prices	15

6	Dahua VMS	15

7	Documentation	15

8	Price Tables	16

8.1	Maintenance – Labour and Repairs or Replacements with caps in Year 3 and beyond	16

8.2	Maintenance – Labour and Repairs or Replacements for items under £250 ex VAT	16

8.3	Option Price – Dahua VMS – these will not be included in the evaluation of prices	17

8.4	Ad-Hoc Prices – these will not be included in the evaluation of prices	18

| Issue 1 | March 2026 - First draft |
| --- | --- |
| Issue 1.1 | May 2026 – Updated with comments from Council |
| Issue 1.2 | Not used |
| Issue 1.3 | June 2026 – updated with maintenance details |
| Issue 1.4 | July 2026 – amended by ConwyCBC |

# Introduction

Conwy County Borough Council has an existing NSI Gold Category 1 Certified CCTV Control Room and Alarm Receiving Centre (ARC) located in Colwyn Bay.

| Figure 1 - Conwy ARC | Figure 2 - Conwy ARC |
| --- | --- |

This tender is to procure a new Maintenance Service for the Public Space CCTV and the Control Room part of the Remote Site CCTV system.

The tender also requests an Option Price to replace the current Public Space CCTV Control equipment with a system from another manufacturer.

The tender does not include the Telecare System, Patriot Alarm System or the Synectics system from another Local Authority.

# Overview of Requirements

The Maintenance Service is to cover the following systems in the Control Room

- Public Space CCTV system

There are 123 Public Space CCTV system is based around Dahua cameras, Dahua video recorders and a Meyertech Video Management System (VMS).

The Service must cover the whole system and includes (but is not limited to)

CCTV Cameras

Wireless links or hard wired communications between Cameras and Node Points

Dahua Video Recorders

Meyetech Stream Casters

Meyertech Server

Meyertech Workstations, Monitors and Joysticks

Meyertech Video Wall Servers and Monitors

Cabling and Power supplies

The cameras make extensive use of Node Points on the Council IT network.

The only parts excluded from the whole system is the Council IT network that form the “long distance links” from Node Points to the ARC, the Core Switch in the Server Room and UPS equipment in the ARC and the Server Room.

- Remote Site CCTV systems

There are 54 remote site CCTV systems linked to the ARC via an IT network which are linked to computers running the Dahua Control Room software (Smart PSS) and the HikVision Control Room software (iVMS-4200).

The IT network links are maintained and configured by the Council IT Department

Each remote site consists of one or more cameras and a remote site Video Recorder.

These are not included in the scope of this tender.

The Maintenance Service must cover the Control Room Viewing system for the remote sites including computers, screens, software and the link to the IT network.

It must also include integrating remote sites to the Control Room viewing software or re-configuring existing sites on the Control Room viewing software for up to 10 remote sites per annum.

- Ad-Hoc work

The winning contractor may be invited to price for additional works, for example installing some new public space CCTV cameras or installing equipment in a Remote Site.

- Items not in scope

The Maintenance Service does not cover

- the Synectics system from another Local Authority

- the Telecare equipment

- the Cameras connected via 4G or 5G Data SIMs (approx. 20 cameras)

- Patriot alarm receiving software

# Summary of Existing System

## Public Space Camera Locations

Camera location maps are online at

https://www.conwy.gov.uk/en/Resident/Crime-and-emergencies/Crime/CCTV/CCTV-Camera-Location-Map.aspx

- Abergele/Pensarn/Kinmel Bay (12)

- Betys-y-Coed (3)

- Colwyn Bay (30)

- Conwy (11)

- Glan Conwy (1)

- Llandudno (31)

- Llandudno Junction and Deganwy (11)

- Llanfairfechan (2)

- Llanwrst (1)

- Mochdre (4)

- Old Colwyn and Llysfaen (6)

- Penmaenmawr (3)

- Rhos on Sea (8)

Total 123 cameras

## Public Space Camera Types

All cameras are made by Dahua.

Camera Types

There are 114 PTZ Cameras

There are 9 Fixed Cameras

Ages of equipment

| Number of PTZ cameras | Number of Fixed Cameras | Age in Years (Approx) |
| --- | --- | --- |
| 2 | 0 | Under warranty |
| 1 | 1 | 0.5 |
| 5 | 1 | 1 |
| 3 | 3 | 2 |
| 7 | 0 | 3 |
| 2 | 0 | 4 |
| 1 | 2 | 5 |
| 1 | 0 | 6 |
| 92 | 2 | 8 |
| TOTAL 114 | TOTAL 9 |  |

## Data Transmission

The CCTV Cameras are linked together by Council owned wireless networks and hard wired cables to a number of Node Points on the Council’s IT network.

As not all camera or Node Point locations have line of site to each other there some Relay Points using in use, often on street lighting columns.

Currently there are approximately 220 Wireless devices manufactured by Ligowave in use.

Cameras and Relay Points use network switches and there are approximately 64 network switches to be included in this maintenance tender, mostly from Ubiquiti.

Most of the wireless links and network switches will be 8 years old.

## Recording System

All cameras are recorded on 3 Dahua Video Recorders (model DH-NVR5864-I

These are to be covered in the maintenance service.

They were installed in June 2022.

## Meyertech System and Video Wall

The ARC VMS (Video Management System) is manufactured by Meyertech.

The system comprises of

- Two Meyertech workstations, each has with multiple monitors and Joysticks

- One Meyetech VMS server

- Meyertech display wall drivers

- Meyertech Streamcasters (RTSP stream distributors)

- Old Meyertech video recorder re-purposed as a NAS drive

- 12 Large video wall of monitors

- Spot Monitor recording of a HDMI screen on each workstation (C19 and C20)

- The PCs also include iVMS 4200 and SmartPSS software for accessing the Remote Sites

## Police HQ Link

There is a Meyertech video decoder server located at Colwyn Bay Police HQ which allows video images to be sent to North Wales Police with a wireless link from C103.

## Re-Deployable (4G/5G) cameras

There are 20 4G/5G re-deployable cameras manufactured by IVID.
These use data SIMs with enough monthly data for the cameras to connect to the Meyertech system 24x7 for central viewing and central recording.

All the re-deployable cameras use Dahua cameras.

This system will remain unchanged. The camera maintenance will remain with their current provider.

The ARC maintenance provider will be the first point of contact if one of these cameras goes offline and will then pass on relevant faults.

## Remote Site Systems

There are 54 remote sites linked to a PC in the ARC using remote site recorders.

In the ARC is a PC running the Hik iVMS4200 software and the Dahua SmartPSS software.

The PC and the upkeep of the Hik and Dahua software is to be included in the maintenance service.

The remote sites are not part of the maintenance service, but the service provider must work with each 3rd party company who does maintain the equipment at remote sites to co-ordinate on fault reports, IP addresses, usernames and passwords.

8 of the remote sites use HikVision Recorders.

46 of the remote sites use Dahua Recorders.

## NTP Time Server

This is provided by the IT department.
The service supplier must ensure all equipment capable of using NTP time servers must be set to use the Time Server to allow for consistency of Timestamps across Log Files.

## Experience and Vendor Approval

In order to maintain a high level of service for a 24x7 ARC without delays caused by inexperienced staff, the bidders must be able to show that the engineers deployed to the ARC and to the Cameras have sufficient experience, tools and equipment.

Meyertech

Bidders must be able to demonstrate a close working relationship with Meyertech and either be an existing Meyertech Certified Partner or complete join the Partner Program and complete the necessary training within 4 weeks of the start of the contract. This is to ensure that camera additions or configuration changes to the VMS and the resolution of VMS faults can take place as quickly as possible.

Bidders must ensure they have sufficient Meyertech Support Credits to cover all faults for the duration of the maintenance service.

Bidders must ensure they have a suitable back to back Service Level Agreement (SLA) with Meyertech to meet the Council’s Service Level Agreement.

Dahua

Bidders must be able to demonstrate a close working relationship with Dahua and have experience of the supply and install of Dahua cameras, Dahua Smart PSS and Dahua video recorders.

Remote Access

Bidders must be able to demonstrate experience of secure Remote Access methods of working and be available to resolve issues remotely via a secure Remote Access method.

This is to minimise downtime and also to minimise unnecessary travel.

Fluent with IT

Must work closely with the Council IT Network manager who is responsible for the long distance communications from camera Node Points, IT edge switches, core switches and for the Firewall.
The engineer assigned to the contract must be fluent in IT terminology including VLANs, Subnets and IP ranges.

## Other items not in scope

The ARC door entry system is not part of the maintenance service.

The Telecare system is not in scope

The Patriot alarm system is not in scope

The Synectics system used to monitor another Local Authority is not in scope

## Recent Fault History for the last 6 months

In the last 6 months there have been

144 fault tickets raised

26 on site visits required (with Cherry Picker)

3 cameras replaced

3 switches replaced
4 wireless links replaced
some PSUs and PoE Injectors replaced

# System Maintenance

Although specific items have been listed in this document, the maintenance service must include all equipment that forms the Public Space CCTV System and the ARC element of the Remote Site unless specifically excluded.

This includes (but is not limited to)

CCTV Cameras

Wireless Links

Network switches (unless owned by Council IT)

The power supplies

Brackets

Cable routes and containment

Cabling (data and power)

Control Room Workstations and Joysticks, Dahua Video Recorders, Meyertech VMS, Server and any Streamcasters, Monitors (desk and video wall), Monotor wall driver PCs, Spot Monitor Encoders.

Control Room data cabling (power and switches are by the IT department)

It does not include the upkeep of columns but any damage or corrosion must be reported to the Council.

## Contract Duration

Contract is for 3 years with an option for the Council to extend on year by year to a total of 5 years.

Maintenance Prices

Prices for two types of maintenance contract are being sought which have different responsibilities for irreparable items.

OPTION A) Labour, Repairs and Like-For-Like Replacements with caps in Year 3 and beyond
The Council is also interested in a price for a Fully Comprehensive Maintenance where all faulty items are either repaired or replaced as part of the bidder’s price.

This saves the Council and the bidder from having to approve purchase orders for every repair or replacement.

However the Council acknowledge that large parts of the system are 8 years old and additional rules are given below for Year 3 and beyond.

The cost must include all labour, equipment (including Cherry Pickers and access equipment), barriers, pedestrian and vehicle management, tools, wiper blades (if applicable), hard disks and repair costs or replacement item costs.

When an item is repaired, it must restore the original functionality of the item.

When an item is replaced, the replacement item must meet the same performance specification as the faulty item and must be fully integrated into the existing system.

For example, a HD PTZ camera would be replaced with another HD PTZ camera, not by a fixed camera.
A monitor would be replaced by another monitor of similar size.
A recorder must be replaced by another recorder of at least the same capacity.

For compatibility with the AI features of the Dahua recorders, replacement cameras must also be manufactured by Dahua.

It is acceptable to use pre-used items for the like-for-like replace. This is not a new-for-old requirement.

Cameras and Wireless Devices that are 10 years or more of age in Year 3 and beyond

For equipment that is 10 years of age or older, there is a cap of 15 cameras and 10 wireless devices per annum.
At the end of each contract year of these quantities of cameras or wireless devices have not been replaced, the service provider must pro-actively replace items to reach these quantieis.

For example in Year 3 there may be 11 camera faults which the bidder repairs or replaces. At the end of Year 3 they must then pro-actively replace 4 other cameras (putting the removed items into stores as spare parts).

Where there are more faults that these quantities, the service provider will be asked to take working equipment from elsewhere on the system, with labour costs included in the tender price.

OPTION B) Labour and Small Value replacements

The Council is also interested in a price for a Maintenance covering all labour costs and the repair or replacement of items below a threshold price in the bidder’s price.

The cost must include all labour, equipment (inc Cherry Pickers and access equipment), barriers, pedestrian and vehicle management, tools, wiper blades (if applicable) and repair or replacement of all items under £250 ex VAT (for example, power supplies or one end of a wireless link).

For items where the repair or replacement is more than £250 ex VAT or more, that cannot be repaired, the service provider must supply a quote for a new item.
The £250 ex VAT limit will adjust with inflation each year.

The cost of installing the new item or relocate existing equipment (for example, to move a working camera into the position of a failed camera) must be included in the Bidder’s tender price.

## Response Times

The Bidder must accept the response times set out below.

| Major Fault | On site attendance within 8 hours of fault being reported, 24x7x365 (inc public holidays) Problem repaired or a loan item installed within 48 hours of the fault being reported (24x7x365). |
| --- | --- |
| Other faults | On site attendance next working day. Problem repaired or a loan item installed within 72 hours of the fault being reported. |
| Online diagnostics response | Bidder to offer Secure Remote Access as a first level diagnostics to reduce downtime. |

A Major Fault is: Loss of more than 25% of the back wall, loss of 50% or more of operator positions, loss of recording of more than one camera, loss of replay of video, loss of PTZ control of more than one camera, loss of view of more than one camera, loss of any of 10 key cameras (to be defined at award) or any other fault which the CCTV Supervisor has reasonable grounds to class as a major fault.

## Fault Reporting

The Bidder must supply either a 24 hour telephone line for reporting faults or an on-line web based fault reporting system.

## Performance Monitoring

The Bidder must supply the Councils with monthly reports describing the fault and the fix, the fault reporting time/date, the time the engineer attendance time/date, the time/date of the fix or loan item.

The Council will review the Bidder’s actual performance and may impose penalties if the Bidder’s performance does not meet these requirements.

Penalties are

- Early termination of the contract in the event of multiple failures to meet the timescales, in which case the Bidder will not be able to claim for any loss of income from early termination of the Contract and the Bidder must refund pro-rata any maintenance money paid in advance.

- For each calendar month where 2 times or more the response times are not met, the Council will be intitled to a 25% reduction of that month’s payment. Where money is paid in advance, this must be refunded by the Bidder or deducted from the next invoice.

## Planned Preventative Maintenance and Cleaning

The Bidder must allow for 2 (two) visits to site each year to carry out Preventative Maintenance, System checking and Camera Cleaning.

Checks must include the following (as a minimum):

Control Room

Check Error Logs, live image quality, recorded image quality, check time synchronisation, check recorder hard disks and temperature, check screens, check PTZ control and camera functions

Transmission

Check Transmission Links, transmission electronics and cabling

Check data loss / packet loss

Check for low signal levels on wireless

Check for high bandwidth usage on links

Camera Sites

Check Camera, Seals, Brackets, Cabling, PSU voltage
Clean the camera

(PTZ functions tested in the Control Room)

Check for any water or damp ingress that could effect equipment

Check locks

Actions

Correct any issues that have been identified

## Back to Back Maintenance Contracts

The Bidder must include in their price any necessary Back to Back maintenance contracts for equipment manufacturers to ensure that during the maintenance period, that the Bidder is able to receive fast resolution of faults, technical issues and fast turnaround of repairs in like with the Service Level Agreement set out in this specification.

## Engineer Training

Installation and Maintenance engineers must be fully trained on the equipment to be maintained.

## Software and Firmware updates

During the maintenance period, bidders must supply and install all critical software updates and Security Updates. This includes the VMS, Operating Systems, Switches, Wireless Devices and Cameras.

## Spares Holding

The bidder must hold sufficient spares in order to meet the repair times set out in this specification.

# Ad-Hoc Prices

The Bidder must provide item prices should any ad-hoc work be required during the tender period. If extra ad-hoc work is awarded, prices can be adjusted in line with inflation.

There is no guarantee that the Council will award any additional work, and the Council may award additional work to another contractor or service provider instead.

# Dahua VMS

The Meyertech VMS is unable to support some features of the newer Dahua Cameras and Dahua Video Recorders, especially those for Analytics and AI Appearance Searching.

The Council are interested in an option price to replace parts of the Meyertech system with the Dahua DSS Video Management System (VMS), subject to funding and internal business plans.

Dahua’s Incident Logging feature is currently lacking some features and so Meyertech would remain as an Incident Management logging tool.

Two (2) Operator Workstations are to be replaced with Dahua DSS workstations, both with PTZ joysticks and the ability to select cameras via Camera Number. Monitors can be re-used.

Existing DSS trial equipment and computers running Meyertech software can be re-used where suitable.

For the video wall, equipment must be provided to drive 12 existing back wall monitors, each monitor showing up to 9 cameras at a time.
It is expected that only 10 monitors will be connected as some have been reserved for the Synectics system, but the hardware must support 12 monitors.

The new Dahua VMS must integrate both the Public Space CCTV Cameras and all the Remote Site Systems. It is understood that Dahua can connect to remote HikVision recorders and bidders can assume this. If there are issues connecting to HikVision, then alternative arrangements will be put in place at the time of the upgrade (eg replace recorders, or retain iVMS-4200 for those sites)

# Documentation

The Bidder must maintain up to date documentation that is shared with the Council on the system including

- Schematic of the new system

- List of Makes, Models and Serial numbers and software versions of equipment

- IP addresses for all systems (including Cameras and Wireless links)

- Usernames and Passwords (both user and administrator/engineer)

- Copy of all configuration files on an external USB drive or CD/DVD disk to allow for rapid restoring of servers, recorders, cameras, switches and firewalls.

# Price Tables

All prices are to exclude VAT

Payment Plan

Payments for maintenance will be made quarterly in arrears.

## Maintenance – Labour and Repairs or Replacements with caps in Year 3 and beyond

|  | Year 1 | Year 2 | Year 3 | Year 4 | Year 5 |
| --- | --- | --- | --- | --- | --- |
| Maintenance option A | £ | £ | £ | £ | £ |
| TOTAL for 5 Years (used in the evaluation) | £ Please include this in the Form of Tender (Invitation to Tender Appendix 1) |  |  |  |  |

## Maintenance – Labour and Repairs or Replacements for items under £250 ex VAT

|  | Year 1 | Year 2 | Year 3 | Year 4 | Year 5 |
| --- | --- | --- | --- | --- | --- |
| Maintenance option B | £ | £ | £ | £ | £ |
| TOTAL for 5 Years (used in the evaluation) | £ Please include this in the Form of Tender (Invitation to Tender Appendix 1) |  |  |  |  |

## Option Price – Dahua VMS – these will not be included in the evaluation of prices

| Item | Unit Cost | Quantity | Total Cost |
| --- | --- | --- | --- |
| Workstation PC (re-using monitors) | £ |  | £ |
| PTZ Joysticks | £ |  | £ |
| Display Wall Drivers (re-using monitors) | £ |  | £ |
| Dahua Server Hardware | £ |  | £ |
| Dahua DSS VMS software and licence costs | £ |  | £ |
| Installation, Configuration and System Documentation | £ |  | £ |
| Training to all CCTV Operators over two sessions | £ |  | £ |
| Any other items …………………….. …………………….. …………………….. …………………….. |  |  | £ £ £ £ |
| TOTAL CAPITAL |  |  | £ |
| Revenue Costs |  |  |  |
| Dahua annual support package costs for DSS |  |  | £ |
| Any other items …………………….. …………………….. |  |  |  |
| TOTAL REVENUE |  |  |  |

Changes in Maintenance Costs if switching from the Meyertech VMS to the Dahua DSS VMS – these will not be included in the evaluation of prices

Note that Fusion Incident will remain.

| Change in maintenance price against the Option A (Labour and Like-For-Like) costs | £ (increase/decrease per year) |
| --- | --- |
| Change in maintenance price against the Option B (Labour and Parts under £250) costs | £ (increase/decrease per year) |

Other Costs  – these will not be included in the evaluation of prices

| Capital Cost for a new camera licence | £ |
| --- | --- |
| Annual costs for DSS for a new camera | £ |

## Ad-Hoc Prices – these will not be included in the evaluation of prices

| Item | Unit Cost |
| --- | --- |
| Dahua external HD PTZ Camera | £ |
| Dahua internal static HD IP camera (mini dome) | £ |
| Dahua external static HD IP camera | £ |
| Dahua external 180 degree static HD IP camera | £ |
| Dahua internal 360 degree fish-eye camera | £ |
| Dahua external 360 degree fish-eye camera | £ |
| Dahua external multi-sensor panoramic camera | £ |
| Power supply for PTZ cameras | £ |
| Power supply for static/360/multi-sensor cameras | £ |
| Wall bracket for camera | £ |
| Swan Neck bracket for cameras fixed to top of columns | £ |
| Bracket for mounting to side of Column (e.g. to street lighting) | £ |
| 8 channel Dahua Video Recorder to use at Remote Sites | £ |
| Point to Point Wireless Link (short range < 100m) inc bracket and power supply | £ |
| Point to Point Wireless Link (medium range < 1km) inc bracket and power supply | £ |
| Managed Network Switch (for installation on street) with power supply (8 port) | £ |
| Electrical Equipment Box (external box, pole mounted) with securing strap | £ |
| Day rate for an engineer with a Cherry Picker included | £ |
| Day rate for an engineer (no Cherry Picker) | £ |
| Hourly rate for a configuration engineer in the control room for system configurations (e.g. Setting Privacy Zones or Analytic/AI features) | £ |

- End of Document