SPECIFICATION FOR HANDRAILS AND SAFETY RAILING

External Handrails

- Key clamp railing

- Galvanised uprights and fittings, yellow powder coated handrails

- No overhanging edges, rail to terminate at post with 90 degree angled fitting

- All uprights to be evenly spaced

- The clearance between the handrail and any adjacent wall surface must be between 60mm and 75mm

- Diameter of handrail must be 42.4mm

- For flat or ramped areas, overall height from ground level must be between 900-1000mm. For steps and stairs, overall height from the pitch line (diagonal line connecting the step nosings) must be between 900mm–1000mm.

- For steps, a handrail must be fitted to one side of a flight of steps where the flight comprises three or more risers

- For steps wider than 1m, a handrail must be fitted on both sides.

- The loading criteria must comply with current BS regulations

- To be fitted as per manufacturers instructions

Edge Protection - Drops between 300mm-599mm

- Key clamp railing with crossbar

- Overall height from ground level must be 1100mm

- Spacing between ground level to crossbar and crossbar to handrail must not exceed 550mm

- Galvanised uprights and fittings with yellow powder coated handrails

- No overhanging edges, rail to terminate at post with 90 degree angled fitting

- All uprights to be evenly spaced

- Diameter of handrail must be a minimum of 42.4mm

- The loading criteria must comply with current BS regulations

- To be fitted as per manufacturers instructions

Edge protection - Drops of 600mm or more

Front of properties or communal areas

- Hoop top safety fencing (style as pictured)

- Overall height from ground level must be a minimum of 1100mm

- Spacing between vertical railings must be no larger than 100mm (so a max gap of 99mm)

- Galvanised finish (or yellow powder coated for visibility requirements)

- Safety fencing to terminate with fitted post

- The loading criteria must comply with current BS regulations

- To be fitted as per manufacturers instructions

Rear of properties

- Flat top type safety fencing (style as pictured)

- Overall height from ground level must be a minimum of 1100mm

- Spacing between vertical railings must be no larger than 100mm (so a max gap of 99mm)

- Galvanised finish (or yellow powder coated for visibility requirements)

- Safety fencing to terminate with fitted post

- The loading criteria must comply with current BS regulations

- To be fitted as per manufacturers instructions