BARCUD

# Pre-Construction
Information

CDM Regulations 2015 | Project-specific information

REFURBISHMENT OF 3 TAN Y CASTELL

3 Tan Y Castell, Llwynygroes, Ceredigion, SY25 6QB

Issue status: DRAFT FOR TENDER ¦ Version: 01 ¦ Date: 21 September 2026

Purpose: This document provides the project-specific health and safety information needed to design, plan and carry out the refurbishment of 3 Tan Y Castell. It must be reviewed and updated as the outstanding information and design decisions are closed.

# Document control

| Version | Date | Prepared by | Reviewed by | Approved by | Status / change |
| --- | --- | --- | --- | --- | --- |
| 01 | 22/09/2026 | Barcud Ltd | To be confirmed | To be confirmed | Draft transferred into the current Barcud PCI format and reduced to project-specific information. |

# Contents

| Section | Content |
| --- | --- |
| 1 | Project overview |
| 2 | Dutyholders and contacts |
| 3 | Existing information and outstanding actions |
| 4 | Site, neighbours and operational constraints |
| 5 | Existing structure, services and hazardous materials |
| 6 | Significant design and construction risks |
| 7 | Project management and site rules |
| 8 | Construction Phase Plan |
| 9 | Welfare |
| 10 | Emergency arrangements |
| 11 | Health and Safety File |
| 12 | Appendices and final issue checklist |

# 1. Project overview

| Project name | Refurbishment of a two-bedroom bungalow at 3 Tan Y Castell |
| --- | --- |
| Site address | 3 Tan Y Castell, Llwynygroes, Ceredigion, SY25 6QB |
| Project reference | To be confirmed |
| Client | Barcud Ltd |
| Project description | Refurbishment of an existing two-bedroom bungalow, including internal, roof, building-services and external works. |
| Main work activities | Replacement plasterboard ceilings; heating upgrade; roof works; solar photovoltaic installation; decoration; flooring; landscaping; paths; fencing and railings. |
| Exclusions | No exclusions are confirmed in the information supplied. The final specification and drawings must define the work boundary. |
| Occupancy | The property will be vacant for the first 42 calendar days of the contract, from this point a tenant will move in and the remaining works will be completed with the contract holder in the property. Any additional needs or requirements for the contract holder is not yet known. |
| Planned start | To be confirmed |
| Planned duration | 42 calendar days for the property to be completed to a habitable standard as agreed with Barcud. All works to be complete by 29/01/2026. |
| Working hours | To be confirmed, taking account of neighbouring homes and any resident arrangements. |
| Estimated peak workforce | To be confirmed |
| Estimated person-days | To be confirmed |
| Expected number of contractors | More than one contractor is expected; confirm the planned contracting arrangement and subcontract packages. |

## 1.1 CDM notification (F10)

Threshold: A project is notifiable when it is expected to last longer than 30 working days and have more than 20 workers on site at the same time, or exceed 500 person-days.

| Notifiable project? | To be confirmed |
| --- | --- |
| Assessment / calculation | The stated 42-day duration is not enough on its own. Record the expected peak workforce and total person-days. |
| F10 reference and date | To be confirmed if the notification threshold is met |
| F10 displayed / updated by | Client to nominate the responsible person if notification is required |

## 1.2 Key outcomes and client priorities

| Health and safety priorities | Confirm occupancy; protect residents, neighbours and the public; control roof work and intrusive work; verify asbestos and services information before disturbance. |
| --- | --- |
| Quality / operational priorities | Coordinate the roof, PV, heating, ceilings and external works so that temporary weathering, fire separation, isolations and reinstatement remain effective. |
| Environmental priorities | Comply with the bat survey; control dust, noise, waste and pollution; protect drainage and neighbouring land. |

# 2. Dutyholders and contacts

| Role | Organisation | Named contact | Contact details | Appointment / notes |
| --- | --- | --- | --- | --- |
| Client | Barcud Ltd | Lindsay James-Preece | Lindsay.james-preece@barcud.cymru | Client authority and project lead to be recorded |
| Principal Designer | Barcud Ltd | Lindsay James-Preece | Lindsay.james-preece@barcud.cymru | Written appointment required before construction |
| Principal Contractor | To be confirmed | To be confirmed | To be confirmed | Written appointment required before construction |
| Designer contact | Barcud Ltd | Lindsay James-Preece | Lindsay.james-preece@barcud.cymru | Discipline and design authority to be confirmed |
| Client site / housing contact | Barcud Ltd | To be confirmed | To be confirmed | Access, occupancy and resident liaison |

## 2.1 Practical responsibilities

| Dutyholder | What this means for this project |
| --- | --- |
| Client | Confirm suitable arrangements, time and resources; appoint the Principal Designer and Principal Contractor in writing; provide available project information; ensure a suitable Construction Phase Plan and welfare are in place before construction starts; retain the Health and Safety File. |
| Principal Designer | Coordinate the pre-construction phase, designers and this PCI; close or control outstanding design information; communicate significant residual risks; prepare and update the Health and Safety File. |
| Designers | Eliminate or reduce foreseeable risks, coordinate interfaces and record the significant residual risks and assumptions that remain. |
| Principal Contractor | Plan, manage and monitor the construction phase; prepare and maintain the Construction Phase Plan; coordinate contractors and subcontractors; provide welfare, security and site controls; supply handover information. |
| Contractors | Plan and supervise their work, cooperate with the Principal Contractor, follow site arrangements and provide risk, design and handover information promptly. |

# 3. Existing information and outstanding actions

| Information | Reference / date | Status | Key limitation or finding | Action owner | Due date |
| --- | --- | --- | --- | --- | --- |
| Asbestos refurbishment survey | Reference/date to be confirmed | Available as a separate document | Confirm that the survey covers every area and material affected by the final scope; close all exclusions before disturbance. | Client / Principal Designer | Before tender issue and before intrusive work |
| Existing Health and Safety File | Reference/date to be confirmed | Availability to be confirmed | Search Barcud asset records and issue relevant information. | Client | Before tender issue |
| Structural / roof condition information | Not identified | Outstanding | Roof build-up, condition, fragility, load limits and temporary-works needs are not confirmed. | Lead designer / Principal Designer | Before roof or PV work |
| Utility records / service drawings | Reference/date to be confirmed | Partial information only | Water, electricity, telecoms, foul and surface-water drainage are identified, but routes and status are not verified. | Client / Principal Contractor | Before intrusive work |
| Fire and occupancy information | Reference/date to be confirmed | Outstanding | Confirm occupancy, escape route, alarms, fire-service access and any temporary impairment controls. | Client / Principal Contractor | Before mobilisation |
| Bat scoping survey | Reference/date to be confirmed | Available as a separate document | Confirm survey scope, validity, seasonal constraints and any licence or stop-work requirements. | Principal Designer | Before roof opening |
| Site / floor plans | Reference/revision to be confirmed | Available as separate documents | Confirm that the red-line boundary and external work areas match the final scope. | Client / Principal Designer | Before tender issue |
| Specification and drawing schedule | Reference/revision to be confirmed | Available in part | Issue the current coordinated scope, details and drawing register. | Lead designer / Principal Designer | Before tender issue |

## 3.1 Outstanding information tracker

| Ref | Information / decision needed | Why it is needed | Owner | Due | Status / date closed |
| --- | --- | --- | --- | --- | --- |
| OI-03 | Record peak workforce and person-days and decide whether an F10 is required. | CDM notification assessment | Client | Before construction | Open |
| OI-04 | Check the asbestos survey scope against the final intrusive works and resolve exclusions. | Safe roof, ceiling and services work | Principal Designer | Before affected work | Closed 28/09/2026 |
| OI-05 | Provide available service information and agree verification, isolation and reinstatement arrangements. | Avoid service strikes and unplanned outages | Client / Principal Contractor | Before intrusive work | Open - hold point |
| OI-06 | Confirm roof condition, structural capacity, PV loads, scaffold interfaces and temporary works. | Safe access, loading and installation sequence | Lead designer / Principal Designer | Before roof or PV work | Open - hold point |
| OI-07 | Confirm fire, emergency, access, delivery, compound and welfare arrangements. | Safe mobilisation and public protection | Principal Contractor | Before mobilisation | Open |
| OI-08 | Review the bat survey and close any seasonal, licensing or ecological constraints. | Avoid harm to protected species and delay | Principal Designer | Before roof opening | Open - hold point |

# 4. Site, neighbours and operational constraints

| Site boundaries | The work is at the red-lined property numbered 3. Confirm the final boundary, external work areas and any shared access before mobilisation. |
| --- | --- |
| Access and egress | Access is from local B roads and the adjoining public road and footway. Keep safe pedestrian access and emergency access available. |
| Deliveries | Plan delivery size, timing, unloading and reversing. Use a competent banksman where vehicle movements could affect pedestrians or neighbouring properties. |
| Parking | On-site parking is limited. Agree contractor parking and prevent obstruction of roads, footways, drives and emergency access. |
| Storage / compound | Agree a secure compound within the authorised work area. Do not load roofs, floors, paths or ground beyond verified capacity. |
| Neighbours / shared access | The property is within a small residential village with adjoining homes, public roads and footways. Minimise obstruction, noise, dust and disturbance. |
| Residents / occupants | Occupancy is not confirmed. Before work starts, agree access, notice, safeguarding of possessions, escape arrangements and any decant or support needs. |
| Public protection and security | Define and secure work, scaffold, excavation, storage and unloading areas. Control keys, visitors and out-of-hours access. |
| Existing fire arrangements | Identify and maintain escape routes and fire-service access. Coordinate any alarm, power or compartmentation impairment before it occurs. |
| Noise / dust / vibration | Use methods and sequencing suited to a residential setting. Give notice of disruptive work and prevent dust migration into occupied or neighbouring areas. |
| Waste | Keep routes clear; store and remove waste safely; segregate hazardous waste; retain transfer and consignment records. |
| Environmental constraints | Comply with the bat scoping survey and stop work if unexpected evidence of protected species is found. Protect drains and neighbouring land from pollution. |

## 4.1 Site logistics plan

| Drawing / plan reference | A marked-up site logistics plan is required before mobilisation; reference and revision to be confirmed. |
| --- | --- |
| Key controls shown | Work boundary, access, pedestrian segregation, delivery/unloading point, compound, scaffold/exclusion zones, welfare, waste, emergency route and assembly point. |
| Who will update it | Principal Contractor, in coordination with the Client and Principal Designer |

# 5. Existing structure, services and hazardous materials

## 5.1 Existing structure and condition

| Construction / structural form | Existing two-bedroom bungalow. Detailed construction, roof build-up, alterations and load paths are not confirmed in the supplied information. |
| --- | --- |
| Known defects | No verified defect or condition schedule has been provided. Inspect and escalate evidence of fragility, rot, corrosion, water damage or instability. |
| Load limits | Roof, scaffold-tie, temporary-storage and external-ground load limits are not confirmed. |
| Retained elements | Existing structure, finishes and services outside the agreed removal areas are to remain and must be protected. |
| Temporary works | Scaffolding, edge protection, roof access, ceiling, temporary weathering and any excavation support require competent planning; engineered design may be needed. |
| Unverified assumptions | Do not assume the roof is non-fragile or suitable for PV, access or storage. Confirm hidden construction and stability before alteration. |

## 5.2 Existing services

Safe isolation: Record drawings are indicative unless verified. Services must be located, identified and safely isolated using a competent procedure before intrusive work starts.

| Service | Known route / location | Live / isolated | Control or permit requirement | Information reference |
| --- | --- | --- | --- | --- |
| Electricity | Supply present; routes not verified | Treat as live | Identify, prove, isolate and lock off before affected work; control temporary supplies and re-energisation. | Records / survey TBC |
| Water | Supply present; routes not verified | Treat as live | Locate isolation points and plan flooding control and reinstatement. | Records / survey TBC |
| Drainage | Foul and surface-water drainage identified; routes not verified. | Operational status TBC | Locate and protect drains; prevent contamination and uncontrolled discharge. | Site plan / survey TBC |
| Telecoms / data | Service identified; route not verified | Treat as live | Locate and protect; agree any interruption with the Client and occupant. | Records TBC |
| Other / hidden services | Concealed services may be present in ceilings, walls, roof and external work areas | Unknown | Scan, trace, inspect and use safe digging/opening-up methods before disturbance. | Verification record required |

## 5.3 Asbestos and other hazardous materials

Asbestos: The asbestos survey must be appropriate to the planned level of disturbance and cover every affected area. A competent assessment must determine whether any work is licensed, notifiable non-licensed or non-licensed under the Control of Asbestos Regulations 2012. If suspect material is found, stop work, prevent disturbance, secure the area and follow the agreed escalation procedure. Record other relevant hazardous materials, including lead paint, silica, contaminated soil, refrigerants or biological hazards, in the risk register and supporting information.

| Survey type / reference / date | ASBESTOS MANAGEMENT SURVEY WITH A REFURBISHMENT TO SPECIFIED AREAS OF 3 TAN Y CASTELL, LLWYNYGROES, TREGARON, CEREDIGION, SY25 6QB, June 2026. Ceilings, floor tiles and storage heaters with ACMs have been removed prior to contract start date. |
| --- | --- |
| Survey scope matches the work? | Not yet confirmed. Check every roof, ceiling, heating/service penetration and external area that will be disturbed. |
| Known ACMs / excluded areas | ACMs are stated to be present. Locations, condition, actions and excluded or inaccessible areas must be taken from the current survey. |
| Work category / emergency procedure | A competent assessment must classify each task. If suspect material is found: stop, prevent disturbance, secure the area and report through the agreed escalation route. |

# 6. Significant design and construction risks

| Ref | Element / activity | Significant hazard and who may be affected | Design response / risk reduction | Residual risk or information the PC needs | Owner / status |
| --- | --- | --- | --- | --- | --- |
| DR-01 | Roof refurbishment | Unverified fragility, condition, edges, loading and temporary weathering could affect workers and people below. | Confirm roof build-up and condition; coordinate access, scaffold, edge protection, sequencing and temporary weathering. | No roof access or work until capacity, fragility, access and temporary-works requirements are confirmed. | Principal Designer / lead designer - hold point |
| DR-02 | Solar photovoltaic installation | Roof loading, fixings, weathering and live DC generation create structural, electrical and maintenance risks. | Coordinate structural check, fixing/weathering detail, installation sequence, isolation, commissioning and safe future access. | PV design and roof capacity must be approved before installation; provide labels, isolations and handover information. | PV designer / Principal Designer - hold point |
| DR-03 | Ceiling replacement and asbestos | Intrusive work may encounter ACMs, hidden services, falling material or loss of fire separation. | Match survey coverage to the final scope; coordinate service isolation, controlled removal, exclusion and reinstatement. | Close survey exclusions and confirm the sequence before disturbance. | Principal Designer / Principal Contractor - hold point |
| DR-04 | Heating and electrical work | Live or concealed services, temporary supplies and poorly sequenced penetrations could cause injury, outage or loss of fire separation. | Verify routes and isolations; coordinate penetrations, temporary services, testing, commissioning and fire stopping. | Record isolations and safe re-energisation; update service information. | Services designer / Principal Contractor - open |
| DR-05 | External paths, fencing, railings and landscaping | Buried services, excavations, uncertain ground, level changes and public access affect workers and neighbours. | Confirm levels, drainage and service routes; design any required support; phase and segregate the work. | No excavation until services and support needs are verified. | Designer / Principal Contractor - hold point |
| DR-06 | Occupied property and public interface | Uncontrolled access, dust, noise and disruption of escape or services could affect residents, visitors and neighbours. | Confirm occupancy; define segregation, communication, access, escape and any decant arrangements. | Occupation arrangements must be agreed before mobilisation and updated when phases change. | Client / Principal Contractor - hold point |
| DR-07 | Bat and ecological constraints | Roof opening could disturb protected species and stop the project. | Review the bat survey and incorporate timing, licensing and stop-work requirements into the design and programme. | No roof opening until ecological constraints are closed. | Principal Designer - hold point |
| DR-08 | Fire and hot work | Roof, ceiling and service work may impair alarms, escape or compartmentation; ignition could affect the property and neighbours. | Prefer cold methods and maintain escape and compartmentation. Coordinate temporary impairments. | Hot work is prohibited unless Barcud formally authorises a permit, controls, fire watch and close-out arrangement. | Principal Contractor / Client - hold point if hot work proposed |

## 6.1 Design assumptions and hold points

| Ref | Assumption / hold point | How it will be verified | Owner | Required by | Outcome / close-out |
| --- | --- | --- | --- | --- | --- |
| HP-01 | Asbestos survey scope covers every planned disturbance and exclusions are closed. | Principal Designer records survey-to-scope review and any further inspection or sampling. | Principal Designer | Before intrusive work | Open |
| HP-02 | Roof condition, fragility, structural capacity, PV loads and temporary works are confirmed. | Approved survey, calculation and coordinated design information. | Lead designer / Principal Designer | Before roof access or loading | Open |
| HP-03 | Occupancy, fire, access and resident arrangements are agreed. | Written mobilisation plan and Construction Phase Plan review. | Client / Principal Contractor | Before mobilisation | Open |
| HP-04 | Buried and concealed services are located and isolations are planned. | Records, survey/scan, site verification and isolation schedule. | Principal Contractor | Before excavation or opening-up | Open |
| HP-05 | Bat survey constraints are reviewed and satisfied. | Principal Designer/ecology confirmation and any licence or timing record. | Principal Designer | Before roof opening | Open |

## 6.2 Coordination record

| Latest design risk review | To be confirmed; this draft identifies the minimum review topics. |
| --- | --- |
| Principal Contractor input | Obtain before construction, particularly for roof access, temporary works, deliveries, sequencing and occupied-property controls. |
| Residual risks shown on drawings/specification | References to be added when the coordinated drawing and specification register is issued. |
| Next review | Before tender issue; again after contractor appointment and whenever scope, survey or design information changes. |

# 7. Project management and site rules

| Topic | Project requirement |
| --- | --- |
| Competence and supervision | Use competent, appropriately supervised trades and licensed specialists where required. The Principal Contractor must verify subcontractor capability for the work assigned. |
| Induction and briefing | Brief the project risks, boundaries, residents/public controls, asbestos information, fire arrangements, services and hold points before work. |
| Access control | Secure the work area and control keys, visitors, deliveries and out-of-hours access. Visitors must be accompanied in work areas. |
| Permits and hold points | Use written controls where required for roof access, excavation, service isolation, temporary works and any authorised hot work. |
| Hot work | Not permitted unless Barcud formally approves a project-specific permit, fire watch and close-out arrangement. |
| Work at height | The Construction Phase Plan must define roof access, scaffold/edge protection, inspection, weather limits, falling-object controls and rescue. |
| Temporary works and lifting | Identify design needs, appointments, checks and release points. Coordinate lifting and unloading with the restricted residential access. |
| Service isolation | Authorised competent persons must identify, isolate, lock off, prove and control re-energisation or reinstatement. |
| Fire safety | Maintain escape and fire-service access; control combustible storage and temporary impairments; complete daily checks where required. |
| Housekeeping and waste | Keep paths and public interfaces clear; secure materials and waste; remove debris regularly; control hazardous waste records. |
| Noise, dust and vibration | Select lower-impact methods, contain dust, coordinate disruptive work and communicate with occupants and neighbours. |
| Incident and change reporting | Report incidents, unexpected asbestos/services, design changes and scope changes immediately so controls and this PCI can be reviewed. |
| Handover / occupation | Do not hand over or reoccupy affected areas until services, fire separation, access, testing, cleaning and resident information are complete. |

## 7.1 Communication and consultation

| Design / coordination meetings | Principal Designer to set a proportionate review frequency and include designers and the Principal Contractor when their input is needed. |
| --- | --- |
| Progress / site meetings | Principal Contractor to report significant risks, inspections, changes, incidents and hold-point status to Barcud. |
| Resident / occupant communication | Barcud housing contact to coordinate access and notices; Principal Contractor to provide accurate programme and disruption information. |
| Worker consultation | Principal Contractor to use induction, task briefings, toolbox talks and a clear route for feedback and stop-work concerns. |
| Change control | No material design, scope, sequence or site change is to proceed until its health and safety effect is reviewed and communicated. |

# 8. Construction Phase Plan

CDM requirement: The Principal Contractor must prepare a suitable Construction Phase Plan before the construction phase begins. The Client must ensure that it has been drawn up. Any advance submission period set below is a Barcud contractual or project-management requirement, not a statutory CDM timescale.

| Principal Contractor | To be appointed |
| --- | --- |
| Contractual submission period | To be confirmed by Barcud. In every case, a suitable plan must be in place before construction starts. |
| Submitted to | Client and Principal Designer; copy to the Contract Administrator / project manager if appointed |
| Client assurance completed by | Named Barcud client lead, with advice from the Principal Designer - name and date to be recorded |
| Review triggers | Design or scope change; new survey information; change in occupancy, programme or workforce; new contractor; significant incident; phase change. |

The plan should be proportionate to the project and explain how the construction phase will be planned, managed, monitored and coordinated.

It should address the significant project risks in this PCI, site rules, welfare, emergency arrangements, fire safety, public protection, temporary works and cooperation between contractors.

Client or Principal Designer review does not transfer the Principal Contractor’s responsibility for managing the construction phase.

## 8.1 Pre-start readiness record

| Check | Responsible person | Evidence / reference | Complete? | Date |
| --- | --- | --- | --- | --- |
| Principal Designer and Principal Contractor appointed in writing | Client | Appointment letters and scope | No | Before construction |
| PCI updated, approved and issued to those who need it | Principal Designer / Client | Controlled PCI issue record | No - draft | Before tender / construction |
| Suitable Construction Phase Plan drawn up and reviewed | Principal Contractor / Client | Current plan and review record | No | Before construction |
| Suitable welfare arrangements confirmed from day one | Principal Contractor / Client | CPP, logistics plan and welfare schedule | No | Before site set-up |
| Asbestos survey scope and affected ACM controls confirmed | Principal Designer | Survey-to-scope review and work assessment | No | Before intrusive work |
| Services located and isolation / reinstatement planned | Principal Contractor | Records, survey and isolation schedule | No | Before affected work |
| Fire, emergency, rescue and public-protection arrangements coordinated | Principal Contractor | CPP, fire plan, rescue plan and logistics plan | No | Before mobilisation |
| Outstanding information resolved or controlled through hold points | Client / Principal Designer | Section 3 tracker and section 6 hold-point record | No | Before relevant work |

# 9. Welfare

| Facility | Proposed arrangement | Location / capacity | Available from | Responsible person |
| --- | --- | --- | --- | --- |
| Toilets | Principal Contractor to propose suitable facilities and servicing. | Location and capacity TBC in logistics plan | Before site set-up | Principal Contractor |
| Washing | Hot/warm and cold water, soap and drying; add specialist decontamination arrangements if the risk assessment requires them. | Location and capacity TBC | Before site set-up | Principal Contractor |
| Drinking water | Wholesome drinking water with suitable cups or bottles. | Location TBC | Before site set-up | Principal Contractor |
| Rest / eating | Heated, lit rest area with seating, tables and facilities for preparing hot drinks and food as required. | Location and capacity TBC | Before site set-up | Principal Contractor |
| Changing / storage | Secure storage, drying and separation of workwear/PPE from personal clothing where required. | Location and capacity TBC | Before site set-up | Principal Contractor |
| Special provision | Confirm accessibility, gender, occupancy and contamination needs from the workforce and risk assessments. | TBC | Before site set-up | Principal Contractor |

# 10. Emergency arrangements

| Site address for emergency services | 3 Tan Y Castell, Llwynygroes, Ceredigion, SY25 6QB. Add an access point or what3words reference to the Construction Phase Plan if useful. |
| --- | --- |
| Site emergency contact | Principal Contractor to nominate and display the name and mobile number before mobilisation. |
| Nearest hospital / urgent care | Principal Contractor to confirm the appropriate facility, route and contact details before mobilisation. |
| First aid | Principal Contractor to assess needs and provide trained personnel, equipment and a clearly briefed location. |
| Fire / evacuation | Confirm alarm/raising the alarm, escape routes, assembly point, roll call and fire-service access before work. |
| Rescue from height | A work-at-height rescue arrangement is required for roof and scaffold work; reliance on the fire and rescue service alone is not sufficient. |
| Utility incident | Stop work, make the area safe, isolate where competent to do so and use the agreed owner/emergency contact route. |
| Suspected asbestos disturbance | Stop work, prevent access and further disturbance, report immediately and follow the competent assessment and decontamination procedure. |
| Structural instability | Stop work, evacuate and establish an exclusion zone; contact the Principal Contractor, Principal Designer and competent engineer. |
| Spill / environmental incident | Protect drains, use suitable spill controls and follow the agreed reporting route. |
| Resident / public incident | Make the area safe, provide first aid if required and notify the Principal Contractor and Barcud contact immediately. |

# 11. Health and Safety File

Responsibility: The Principal Designer prepares, reviews and updates the Health and Safety File. Designers, the Principal Contractor and contractors must provide relevant information promptly. If the Principal Designer’s appointment ends before the project finishes, the remaining duty passes in accordance with CDM 2015.

| File format / platform | Indexed electronic file in Barcud’s agreed system; searchable PDF plus editable originals where available. |
| --- | --- |
| Information submission dates | Principal Contractor to agree interim dates with the Principal Designer and submit the complete draft before practical completion. |
| Review / acceptance route | Principal Designer coordinates completeness; designers and specialists confirm technical accuracy; Client records acceptance. |
| Final handover to Client | Named Barcud recipient, date and receipt record to be completed at handover. |

## 11.1 Information required

| Item | Required? | Responsible source | Format / reference | Due | Received / checked |
| --- | --- | --- | --- | --- | --- |
| As-built drawings and specifications | Yes | Designers / Principal Contractor | PDF and editable originals where available | Before handover | Open |
| Residual risks and safe access information | Yes | Designers / Principal Contractor | Drawings, schedules and concise instructions | Before handover | Open |
| Services, isolations and controls | Yes | Principal Contractor / specialists | Marked-up drawings and schedules | Before handover | Open |
| Updated asbestos information | Yes | Principal Contractor / analyst | Survey/register updates and clearance records | Before handover | Open |
| Testing and commissioning certificates | Yes | Principal Contractor / specialists | Certificates and settings | Before handover | Open |
| Operation and maintenance information | Yes | Suppliers / Principal Contractor | Relevant manuals only | Before handover | Open |
| Fire safety / compartmentation records | Yes | Designers / Principal Contractor | Details, products, photos and certificates | Before handover | Open |
| Warranties and specialist contacts | Where applicable | Principal Contractor / suppliers | Indexed certificates and contacts | Before handover | Open |
| Inspection / maintenance access needs | Yes | Designers / suppliers | Safe access, isolation and maintenance information | Before handover | Open |

# 12. Appendices and final issue checklist

## 12.1 Appendices register

| Appendix | Document | Reference / revision | Date | Attached / location |
| --- | --- | --- | --- | --- |
| A | Current project drawings, specification and schedule of works | References / revisions TBC | TBC | Separate controlled documents |
| B | Asbestos refurbishment survey / register | Reference / revision TBC | TBC | Separate attachment - scope check required |
| C | Bat scoping survey | Reference / revision TBC | TBC | Separate attachment - review required |
| D | Site / location plan and external floor plan | References / revisions TBC | TBC | Separate attachments |
| E | Structural / roof condition and PV design information | Not yet confirmed | TBC | Outstanding - hold point |
| F | Services and isolation information | Not yet confirmed | TBC | Outstanding - hold point |
| G | F10 notification | Assessment / reference TBC | TBC | Attach if the project is notifiable |